JOB HAZARD ANALYSIS

Aerial Lift / Boom Lift Operations

Regulatory References: 29 CFR 1926.453, ANSI A92.22

PROJECT INFORMATION

| Project Name: |  | Date: |  |
| --- | --- | --- | --- |
| Location: |  | JHA Number: | JHA-031 |
| Supervisor: |  | Reviewed By: |  |
| Trade/Activity: | Aerial Lift / Boom Lift Operations | Permit Required: | ☐ Yes ☐ No |

JOB HAZARD ANALYSIS

| Task Step | Hazard | Risk Level | Control Measures |
| --- | --- | --- | --- |
| 1. Pre-Use Inspection | Mechanical failure Hydraulic leaks Defective controls Tire/outrigger damage | H M H M | Complete manufacturer's pre-use checklist before each shift Inspect hydraulic hoses, fittings, and cylinders for leaks or damage Test all controls (lift, lower, swing, drive, emergency stop) at ground level Check tires for proper inflation; inspect outriggers for cracks Verify fall protection anchor points and SRL operation Tag out and remove from service any defective equipment |
| 2. Work Area Assessment | Overhead power lines Unstable ground Overhead obstructions Pedestrian traffic | H H M M | Survey for overhead power lines — maintain min. 10 ft clearance (20 ft for >350kV) Assess ground conditions: slope, soft soil, holes, debris Identify overhead obstructions (beams, pipes, trusses) Establish barricades and exclusion zone around work area Coordinate with other trades to prevent simultaneous overhead work Check weather — wind speed, lightning risk |
| 3. Lift Positioning & Setup | Tip-over on uneven ground Struck-by during positioning Outrigger failure | H M M | Position on firm, level surface; use cribbing or pads under outriggers Set outriggers fully before elevating platform Use spotter when driving/positioning lift near edges or obstacles Engage wheel locks / parking brake before elevating Verify rated capacity includes all occupants, tools, and materials Do NOT exceed manufacturer's maximum platform load |
| 4. Elevated Work Operations | Falls from basket Electrocution Struck by boom Entanglement Dropped objects | H H H M M | Wear full-body harness with lanyard attached to basket anchor point at all times Keep both feet on platform floor — do NOT stand on railings or use ladders in basket Maintain minimum approach distances to energized power lines Do NOT wrap limbs or lanyards around structural members Use tool lanyards; secure all materials to prevent dropped objects Do NOT exceed rated platform capacity Keep platform gate/chain closed during operation |
| 5. Repositioning | Tip-over during travel Struck-by pedestrians Overhead contact | H M H | Lower platform to stowed (travel) position before driving Use a ground spotter when traveling near personnel or obstructions Travel at slow speed; watch for floor openings, slopes, and debris Re-assess work area conditions at each new position Never drive with platform elevated unless machine is designed for it |
| 6. Shutdown & Securing | Unauthorized use Hydraulic energy release Equipment damage | M L L | Lower platform fully; retract outriggers Turn off engine/power; remove and secure key Set parking brake and wheel chocks Clean platform of debris, tools, and materials Complete post-use inspection log Report any deficiencies discovered during operation |

Additional Notes: All operators must possess valid aerial lift operator certification. Daily pre-use inspections are mandatory per ANSI A92.22. Wind speed must be monitored — cease operations if gusts exceed manufacturer limits (typically 28 mph).

PERSONAL PROTECTIVE EQUIPMENT (PPE)

| Required PPE |  |
| --- | --- |
| ☑ Hard Hat (ANSI Z89.1) | ☑ Safety Glasses (ANSI Z87.1) |
| ☑ Full-Body Harness w/ Lanyard | ☑ High-Visibility Vest (Class 2/3) |
| ☑ Steel/Composite Toe Boots | ☑ Dielectric Gloves (near power lines) |
| ☑ Hearing Protection (if applicable) | ☑ Fall Restraint / SRL |

EMERGENCY PROCEDURES

• In case of tip-over: Brace yourself inside the basket; do NOT jump. Activate emergency stop.

• Contact with power lines: Do NOT touch the machine or ground simultaneously. Jump clear with feet together if evacuation is necessary. Call 911 immediately.

• Fall from basket: Initiate rescue plan immediately. Ensure trained personnel perform aerial rescue within minutes to prevent suspension trauma.

• Medical emergency at height: Lower the platform to ground level if possible. Administer first aid. Call 911.

• Emergency lowering: Know location and operation of ground-level emergency controls on every machine used.

• Report all incidents, near-misses, and equipment malfunctions to the supervisor immediately.

WORKER SIGN-OFF

By signing below, I acknowledge that I have reviewed and understand this Job Hazard Analysis, the associated hazards, and the required control measures for the work to be performed.

| # | Print Name | Signature | Company | Date | Time |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |
| 6 |  |  |  |  |  |
| 7 |  |  |  |  |  |
| 8 |  |  |  |  |  |
| 9 |  |  |  |  |  |
| 10 |  |  |  |  |  |

REVIEW & APPROVAL

| Role | Name / Signature | Title | Date |
| --- | --- | --- | --- |
| Prepared By |  |  |  |
| Reviewed By |  |  |  |
| Approved By |  |  |  |
| Site Supervisor |  |  |  |

REVISION HISTORY

| Rev. | Date | Description | Author |
| --- | --- | --- | --- |
| 0 |  | Initial Release |  |
| 1 |  |  |  |