JOB HAZARD ANALYSIS

Asphalt Paving

Regulatory References: 29 CFR 1926.200-203 (Signaling), NIOSH Asphalt Fumes Criteria Document, 29 CFR 1926.601

PROJECT INFORMATION

| Project Name: |  | Date: |  |
| --- | --- | --- | --- |
| Location: |  | JHA Number: | JHA-034 |
| Supervisor: |  | Reviewed By: |  |
| Trade/Activity: | Asphalt Paving Operations | Permit Required: | ☐ Yes ☐ No |

JOB HAZARD ANALYSIS

| Task Step | Hazard | Risk Level | Control Measures |
| --- | --- | --- | --- |
| 1. Traffic Control Setup | Struck by passing vehicles Pedestrian intrusion Worker visibility | H M H | Implement Traffic Control Plan (TCP) per MUTCD standards Deploy signs, cones, arrow boards, and barricades before work begins All workers in Class 3 high-visibility garments Certified flaggers at all ingress/egress points Establish buffer zone between live traffic and work zone Conduct traffic control briefing with all crew before starting |
| 2. Surface Preparation / Milling | Silica dust from milling Noise exposure (100+ dB) Struck by milling machine Pinch points | H H M M | Wet milling or dust suppression to control silica exposure Wear hearing protection — double protection recommended Maintain safe distance from milling drum during operation Spotter required when machine is backing or turning Sweep/vacuum milled surface before tack coat application Lock out milling machine before any maintenance or drum inspection |
| 3. Asphalt Delivery & Transfer | Burns from hot mix (300-350°F) Struck by haul truck Backing vehicle incidents | H H H | Truck drivers follow paver operator signals only — one designated signal person Spotter guides all backing trucks into the paver hopper No personnel between truck and paver during coupling Wear heat-resistant gloves and long sleeves during material transfer Inspect truck bed for hang-ups before raising — stand clear of dump bed Do NOT ride on truck bed or tailgate |
| 4. Paving Operations | Burns from hot asphalt Asphalt fume inhalation Pinch points on paver Heat stress | H H M H | Position workers upwind of paver when possible to reduce fume exposure Screed fume extraction system operational at all times if equipped Monitor asphalt fume exposure — provide respirators if PEL exceeded Keep hands/fingers clear of auger, conveyor, and screed pinch points Never reach into paver hopper — use shovels/lutes for material management Implement heat illness prevention: mandatory water, rest breaks, shade |
| 5. Compaction / Rolling | Struck by roller Pinch/crush between roller and paver Burns from hot mat Vibration exposure | H H M M | Roller operators maintain 360° awareness — use mirrors and spotter No ground personnel within roller operating zone during compaction Sound horn before reversing direction Keep workers off freshly paved mat until cooled below 160°F for foot traffic Limit vibrating roller exposure time to reduce whole-body vibration Roller operators must have clear line of sight at all times |
| 6. Traffic Control Removal | Struck by traffic during removal Premature traffic release Trip hazards | H M L | Remove traffic control devices in reverse order of installation (farthest from work zone first) Maintain traffic control until pavement is safe for public traffic Workers face oncoming traffic during device removal Verify all equipment, tools, and materials are removed from roadway Flaggers remain until all devices are collected Conduct final walk-through of work zone before opening to traffic |

Additional Notes: Monitor ambient temperature and enforce heat illness prevention plan. Ensure water, rest, and shade per OSHA heat guidance. All flaggers must have valid traffic control certification.

PERSONAL PROTECTIVE EQUIPMENT (PPE)

| Required PPE |  |
| --- | --- |
| ☑ Hard Hat (ANSI Z89.1) | ☑ Safety Glasses (ANSI Z87.1) |
| ☑ High-Visibility Vest / Shirt (Class 3) | ☑ Steel/Composite Toe Boots (heat-resistant soles) |
| ☑ Long Sleeves & Long Pants (burn protection) | ☑ Heat-Resistant Gloves |
| ☑ Hearing Protection (NRR 25+) | ☑ Respirator (if fume levels exceed PEL) |
| ☑ Sunscreen / Shade (heat stress) | ☑ Knee Pads (hand work) |

EMERGENCY PROCEDURES

• Burn from hot asphalt (300°F+): Cool burn with water immediately for at least 20 minutes. Do NOT attempt to remove adhered asphalt. Cover with sterile dressing and seek medical attention.

• Asphalt fume overexposure: Move worker upwind to fresh air. Seek medical evaluation for respiratory distress or nausea.

• Struck by vehicle/equipment: Call 911 immediately. Administer first aid. Secure the work zone to prevent additional incidents.

• Heat illness (heat stroke/exhaustion): Move to shade, cool with water/ice, call 911 if altered consciousness, confusion, or high body temperature.

• Equipment fire: Evacuate area. Use dry chemical or foam extinguisher — NOT water on hot asphalt fire. Call 911.

• Report all incidents, near-misses, and safety concerns to supervisor immediately.

WORKER SIGN-OFF

By signing below, I acknowledge that I have reviewed and understand this Job Hazard Analysis, the associated hazards, and the required control measures for the work to be performed.

| # | Print Name | Signature | Company | Date | Time |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |
| 6 |  |  |  |  |  |
| 7 |  |  |  |  |  |
| 8 |  |  |  |  |  |
| 9 |  |  |  |  |  |
| 10 |  |  |  |  |  |

REVIEW & APPROVAL

| Role | Name / Signature | Title | Date |
| --- | --- | --- | --- |
| Prepared By |  |  |  |
| Reviewed By |  |  |  |
| Approved By |  |  |  |
| Site Supervisor |  |  |  |

REVISION HISTORY

| Rev. | Date | Description | Author |
| --- | --- | --- | --- |
| 0 |  | Initial Release |  |
| 1 |  |  |  |