JOB HAZARD ANALYSIS

Concrete Cutting & Sawing

Regulatory References: 29 CFR 1926.1153 (Silica), 29 CFR 1926.300, Table 1 Silica Rule

PROJECT INFORMATION

| Project Name: |  | Date: |  |
| --- | --- | --- | --- |
| Location: |  | JHA Number: | JHA-033 |
| Supervisor: |  | Reviewed By: |  |
| Trade/Activity: | Concrete Cutting & Sawing | Permit Required: | ☐ Yes ☐ No |

JOB HAZARD ANALYSIS

| Task Step | Hazard | Risk Level | Control Measures |
| --- | --- | --- | --- |
| 1. Layout & Marking | Striking embedded utilities/rebar Slips/trips Silica dust (pre-existing) | H L M | Scan concrete with GPR or X-ray to identify rebar, conduit, and utilities Mark cut lines clearly with chalk or marking paint Clear work area of debris, tools, and obstructions Verify depth of cut will not contact embedded utilities Review structural drawings to confirm cut will not compromise integrity Obtain engineering approval for structural cuts |
| 2. Saw / Equipment Setup | Blade failure/breakage Electrical hazards Improper guarding | H H M | Inspect blade for cracks, missing segments, warping, or damage — replace if defective Verify blade RPM rating matches saw specifications Check all guards are in place and functioning properly Ensure GFCI protection on all electrical connections Inspect power cords for damage; route away from water and cutting path Test emergency shut-off before starting work |
| 3. Wet Cutting Operations | Silica dust (reduced but present) Electrical shock from water Slurry slip hazard Blade kickback | M H M H | Provide continuous water flow to blade per Table 1 requirements Use GFCI protection on ALL electrical equipment in wet environment Contain slurry with berms/vacuum — prevent runoff to drains or waterways Maintain firm footing; use anti-slip footwear Allow blade to reach full speed before contacting concrete Never force blade; let it cut at its own pace to prevent kickback Operator must use both hands on saw at all times |
| 4. Dry Cutting with Vacuum | Silica dust exposure Noise (95-110 dB) Flying debris Fire from sparks | H H M M | Use saw with integrated dust collection (vacuum shroud) per Table 1 Ensure HEPA vacuum is operating at manufacturer-specified flow rate Operator wears half-face APR with P100 filters as backup Wear hearing protection — double protection if >100 dB Wear face shield over safety glasses Clear all combustible materials within 35 ft of cutting area Have fire extinguisher within 25 ft |
| 5. Material Removal | Heavy lifting (concrete sections) Pinch points Sharp edges Collapse of cut sections | M M M H | Plan removal sequence to prevent uncontrolled collapse of cut sections Use mechanical lifting (forklift, crane) for sections over 50 lbs Wear cut-resistant gloves when handling cut concrete Support cut sections before making final cuts to prevent falling Keep hands and feet clear of pinch points during removal Stack removed material in designated area away from work zone |
| 6. Cleanup | Silica in settled dust Slurry disposal Tripping hazards | M M L | Use HEPA vacuum for dry dust cleanup — NEVER dry sweep or use compressed air Collect and properly dispose of slurry per local environmental regulations Decontaminate equipment before leaving the work area Remove all PPE carefully to avoid secondary silica exposure Inspect area for remaining trip hazards; patch or barricade open cuts Complete silica exposure log for the shift |

Additional Notes: Silica exposure control plan required per 29 CFR 1926.1153. Follow Table 1 controls for each saw type. Maintain exposure monitoring records. All operators must complete silica awareness training.

PERSONAL PROTECTIVE EQUIPMENT (PPE)

| Required PPE |  |
| --- | --- |
| ☑ Hard Hat (ANSI Z89.1) | ☑ Safety Glasses / Goggles (ANSI Z87.1) |
| ☑ Face Shield (during cutting) | ☑ Hearing Protection (NRR 25+) |
| ☑ Respirator (N95 min / Half-Face APR w/ P100 for silica) | ☑ High-Visibility Vest |
| ☑ Steel/Composite Toe Boots | ☑ Cut-Resistant Gloves |
| ☑ Knee Pads (for slab cutting) | ☑ Waterproof Clothing (wet cutting) |

EMERGENCY PROCEDURES

• Blade contact injury: Apply direct pressure to wound. Call 911 for deep lacerations or amputations. Do NOT remove embedded blade fragments.

• Silica overexposure: Remove worker from exposure area. Seek medical evaluation for respiratory symptoms.

• Electrical shock (wet cutting): De-energize equipment. Do NOT touch the victim if still in contact with power. Call 911. Use GFCI on all circuits.

• Fire from dry cutting sparks: Use ABC fire extinguisher on-site. Clear combustibles from cutting area before work begins.

• Eye injury from debris: Flush eyes with clean water for 15 minutes. Seek medical attention.

• Report all incidents, near-misses, and equipment malfunctions to supervisor immediately.

WORKER SIGN-OFF

By signing below, I acknowledge that I have reviewed and understand this Job Hazard Analysis, the associated hazards, and the required control measures for the work to be performed.

| # | Print Name | Signature | Company | Date | Time |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |
| 6 |  |  |  |  |  |
| 7 |  |  |  |  |  |
| 8 |  |  |  |  |  |
| 9 |  |  |  |  |  |
| 10 |  |  |  |  |  |

REVIEW & APPROVAL

| Role | Name / Signature | Title | Date |
| --- | --- | --- | --- |
| Prepared By |  |  |  |
| Reviewed By |  |  |  |
| Approved By |  |  |  |
| Site Supervisor |  |  |  |

REVISION HISTORY

| Rev. | Date | Description | Author |
| --- | --- | --- | --- |
| 0 |  | Initial Release |  |
| 1 |  |  |  |