FORBES SAFETY SOLUTIONS

JOB HAZARD ANALYSIS

CONCRETE FORMWORK & SHORING

29 CFR 1926 Subpart Q — Concrete and Masonry Construction

PROJECT INFORMATION

| Company Name: |  | Date: |  |
| --- | --- | --- | --- |
| Project Name: |  | Supervisor: |  |
| JHA Number: |  | Revision: |  |
| Work Activity: | Concrete Formwork & Shoring Operations |  |  |
| References: | 29 CFR 1926 Subpart Q; 29 CFR 1926.703; ACI 347; OSHA STD 03-11-002 |  |  |

HAZARD ANALYSIS

| TASK STEP | HAZARD | RISK LEVEL | CONTROL MEASURES |
| --- | --- | --- | --- |
| 1. Form Layout & Preparation | • Manual handling strain (heavy form panels 50-100+ lbs) • Struck-by falling panels during unloading • Pinch points during assembly • Uneven ground / trip hazards | HIGH | • Use mechanical lifting devices (crane/forklift) for panels >50 lbs • Tag lines on suspended loads; never stand under • Leather gloves required during handling • Pre-task housekeeping; level staging area • Two-person minimum for panel handling • Stretch & flex program before shift start |
| 2. Shoring / Reshoring Installation | • Formwork collapse / structural failure • Falls from elevated work surfaces • Struck-by falling shoring components • Crushing during jack/post adjustment | HIGH | • Engineered shoring design by qualified person per 29 CFR 1926.703(b) • Inspect all shoring components before installation • Fall protection required above 6 ft (guardrails or PFAS) • Hard hats mandatory in shoring zone • Do not exceed rated load capacity—verify manufacturer specs • Exclusion zone below active shoring installation |
| 3. Rebar Placement & Tying | • Impalement on protruding rebar • Cuts / lacerations from rebar ends and tie wire • Repetitive motion injury (tying) • Falls from elevated rebar mats | HIGH | • Rebar caps (mushroom type) on all protruding vertical rebar per OSHA • Cut-resistant gloves (ANSI A4 minimum) required • Automatic rebar tying tools to reduce repetitive strain • Fall protection on rebar mats above 6 ft; use rebar runner planks • Pre-shift inspection of rebar caps and covers |
| 4. Form Inspection Before Pour | • Formwork blowout during pour (catastrophic failure) • Undetected damage to ties, braces, or strongbacks • Unauthorized modifications to engineered system | MEDIUM | • Qualified person inspection before each concrete pour per 1926.703(e) • Document inspection on pre-pour checklist • Verify form tie spacing matches engineering drawings • Check all braces, walers, and strongbacks for secure connection • Stop-work authority for any deficiency found |
| 5. Concrete Placement (Pump / Bucket / Chute) | • Concrete burns (alkaline pH 12–13 causes chemical burns) • Silica dust exposure (cutting / grinding cured concrete) • Struck-by from concrete pump hose whip • Engulfment in wet concrete (wall / deep forms) • Noise exposure from pumps and vibrators | HIGH | • Full concrete PPE: rubber boots, long sleeves, alkali-resistant gloves, splash goggles • Immediate skin wash stations within 25 ft of pour area • Pump operator certification; safety meeting before pump setup • Hose restraint cables on all pump line couplings • Workers shall not enter forms during placement • Hearing protection required near pumps/vibrators (>85 dBA) |
| 6. Form Stripping & Shoring Removal | • Premature stripping (structural collapse) • Falls during elevated stripping work • Struck-by falling form panels and hardware • Concrete edge sharpness (lacerations) | HIGH | • Concrete cylinder break tests must meet minimum PSI before stripping • Qualified person authorization required—document on permit • Controlled demolition sequence per engineering plan • Exclusion zone below stripping operations; barricade and signage • Fall protection (PFAS or guardrails) on all elevated stripping • Cut-resistant gloves during handling of stripped forms |

PERSONAL PROTECTIVE EQUIPMENT (PPE) REQUIREMENTS

| PPE ITEM | STANDARD | NOTES |
| --- | --- | --- |
| Hard Hat (Type I, Class E) | ANSI/ISEA Z89.1-2014 | Required at all times in work zone |
| Safety Glasses / Goggles | ANSI/ISEA Z87.1-2020 | Splash goggles during concrete pour |
| Alkali-Resistant Gloves | ANSI/ISEA 105-2016 | Rubber/nitrile for wet concrete; leather for form handling |
| Steel-Toe Rubber Boots | ASTM F2413-18 | Waterproof rubber boots during concrete placement |
| High-Vis Safety Vest (Class 2) | ANSI/ISEA 107-2020 | Required on all active construction sites |
| Fall Protection Harness | ANSI Z359.11-2021 | Full-body harness for work above 6 ft |
| Hearing Protection | ANSI S3.19 / S12.6 | Required near pumps, vibrators, saws (>85 dBA) |
| Respiratory Protection | NIOSH 42 CFR Part 84 | N95 minimum when cutting/grinding concrete (silica) |
| Long Sleeves / Coveralls | — | Prevent concrete contact burns; alkali-resistant material preferred |

EMERGENCY PROCEDURES

Emergency Contact: 911 for all life-threatening emergencies

First Aid Station: Located at _________________ (identify before each shift)

Concrete Burns: Immediately flush affected skin with clean water for 20+ minutes. Remove contaminated clothing. Seek medical attention for persistent irritation or chemical burns.

Eye Contact: Flush eyes immediately with clean water for at least 15 minutes at nearest eyewash station. Seek medical attention.

Formwork Collapse: Evacuate area immediately. Account for all personnel. Do not re-enter until qualified engineer inspects and clears the area. Notify Project Manager and Safety Department.

Fall Injury: Do not move injured person unless in immediate danger. Call 911. Provide first aid as trained. Document incident per company procedures.

Fire / Pump Fuel Spill: Use nearest fire extinguisher if safe to do so. Evacuate to designated muster point. Call fire department.

Nearest Hospital: _________________________________ (identify before each shift)

CREW ACKNOWLEDGMENT & SIGN-OFF

By signing below, I acknowledge that I have reviewed this JHA, understand the identified hazards and required control measures, and agree to follow all safety procedures.

| # | PRINTED NAME | SIGNATURE | COMPANY / TRADE | DATE | UNDERSTOOD (INITIALS) |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |
| 6 |  |  |  |  |  |
| 7 |  |  |  |  |  |
| 8 |  |  |  |  |  |
| 9 |  |  |  |  |  |
| 10 |  |  |  |  |  |

REVIEW & APPROVAL

| ROLE | SIGNATURE | DATE | PRINTED NAME |
| --- | --- | --- | --- |
| Prepared By: |  |  |  |
| Reviewed By (Supervisor): |  |  |  |
| Approved By (Safety Mgr): |  |  |  |
| Client/GC Representative: |  |  |  |

DISCLAIMER

This JHA is a template intended to be customized for site-specific conditions. It does not replace a qualified safety professional's assessment. Employers remain responsible for compliance with all applicable OSHA standards and local regulations. This document should be reviewed and updated whenever work conditions change.