| FORBES SAFETY SOLUTIONS | JOB HAZARD ANALYSIS |
| --- | --- |

DEMOLITION — JOB HAZARD ANALYSIS

Reference: 29 CFR 1926 Subpart T — Demolition

PROJECT INFORMATION

| Company Name: | [Enter Company Name] |
| --- | --- |
| Project Name: | [Enter Project Name] |
| Date: | [MM/DD/YYYY] |
| Supervisor: | [Enter Supervisor Name] |
| JHA Number: | [JHA-DM-001] |
| Work Activity: | Demolition — 29 CFR 1926 Subpart T |

JOB HAZARD ANALYSIS — TASK STEPS & CONTROLS

The following analysis identifies task steps, associated hazards, risk levels, and control measures for demolition operations. All workers must review this JHA before commencing work.

| Task Step | Hazard(s) | Risk Level (H/M/L) | Control Measures |
| --- | --- | --- | --- |
| 1. Engineering Survey & Pre-Demolition Planning | Unidentified structural weaknesses, unknown hazardous materials, incomplete utility mapping, adjacent structure damage | H | • Engineering survey conducted by competent person per 1926.850(a) • Written demolition plan developed and reviewed • Structural drawings obtained and reviewed • Adjacent properties assessed for exposure risk • Survey documented and kept on-site at all times |
| 2. Utility Disconnection & Verification | Electrical contact/electrocution, gas line rupture/explosion, water main flooding, telecom/fiber damage | H | • All utilities identified, disconnected, and capped by utility provider • Written confirmation obtained from each utility company • Lockout/tagout (LOTO) procedures applied per 1926.417 • Test all conductors as de-energized before work begins • Utility map posted at site entrance |
| 3. Hazardous Material Abatement (Asbestos, Lead, PCBs) | Asbestos fiber inhalation (mesothelioma), lead dust exposure, PCB contamination, silica dust, chemical burns | H | • Comprehensive hazmat survey by licensed inspector before demolition • Asbestos abatement by licensed contractor per 1926.1101 • Lead abatement per 1926.62; blood lead level monitoring for workers • Negative air pressure enclosures for abatement areas • Air monitoring throughout abatement; clearance testing before proceeding • Waste manifested and disposed at licensed facility |
| 4. Structural Weakening Sequence | Premature/uncontrolled collapse, falls from height, struck-by falling debris, floor/wall failure | H | • Sequential demolition plan followed top-down per 1926.850(e) • No workers permitted on structures being weakened unless protected • Continuous structural monitoring by competent person • Temporary shoring/bracing installed as required • Exclusion zones established and enforced around weakening areas • Floor openings covered or barricaded per 1926.502 |
| 5. Mechanical Demolition (Excavator, Wrecking Ball, Shears) | Struck-by debris/projectiles, equipment rollover, dust inhalation (silica), noise exposure, flying glass/concrete | H | • Minimum exclusion zone of 1.5x building height maintained • Dust suppression: continuous water spray on demolition face • Silica exposure monitoring per 1926.1153 (Table 1 or exposure assessment) • ROPS/FOPS on all demolition equipment • Operator protected in enclosed cab with HEPA filtration • Spotters posted at exclusion zone perimeter |
| 6. Debris Removal & Material Sorting | Struck-by falling debris, crushing by heavy equipment, rebar puncture wounds, hidden hazmat in rubble, dust exposure | M | • Debris chutes or crane/bucket for upper floor removal • No materials dropped from height outside designated chutes • Workers clear of equipment swing radius at all times • Material sorted: concrete, metals, wood, hazmat waste • Dust suppression maintained during loading/hauling • All loads secured before transport per DOT requirements |
| 7. Site Stabilization & Final Clearance | Unstable foundations/basements, open excavations, standing water, residual contamination | M | • Below-grade structures filled or properly shored • Environmental clearance sampling completed • Open excavations barricaded or backfilled • Final site inspection documented by competent person • Erosion and sediment controls installed per SWPPP |

REQUIRED PERSONAL PROTECTIVE EQUIPMENT (PPE)

☐  Hard hat (ANSI Z89.1 Type I, Class E) — required at all times on site

☐  Safety glasses with side shields (ANSI Z87.1) — required at all times; goggles for dust exposure

☐  High-visibility vest (ANSI/ISEA 107 Class 2 minimum) — required at all times

☐  Steel-toed boots (ASTM F2413) with puncture-resistant soles and metatarsal guards

☐  Cut-resistant gloves (ANSI A4 minimum) for debris handling; chemical-resistant gloves for hazmat

☐  Hearing protection (NRR 25+) required during mechanical demolition operations

☐  Respiratory protection: N95 minimum for dust; half-face P100 for lead; full-face with HEPA for asbestos

☐  Personal Fall Arrest System (PFAS) for work above 6 feet

☐  Disposable Tyvek coveralls during hazardous material abatement operations

☐  Face shield for cutting/sawing operations with projectile hazards

EMERGENCY PROCEDURES

| Emergency Contact Number: | [Enter Phone Number] |
| --- | --- |
| Nearest Hospital: | [Enter Hospital Name & Address] |
| Site Assembly Point: | [Enter Location] |
| First Aid Kit Location: | [Enter Location] |
| AED Location: | [Enter Location] |
| Fire Extinguisher Locations: | [Enter Locations] |
| Hazmat Spill Kit Location: | [Enter Location] |
| Decontamination Station: | [Enter Location — Required for Hazmat Operations] |

CREW SIGN-OFF — I HAVE REVIEWED AND UNDERSTAND THIS JHA

By signing below, each crew member acknowledges they have reviewed this Job Hazard Analysis, understand the identified hazards and control measures, and agree to follow all safety procedures outlined herein.

| Printed Name | Signature | Date |
| --- | --- | --- |

ADDITIONAL HAZARDS / SITE-SPECIFIC NOTES

Use this section to document any additional hazards, site-specific conditions, or supplemental control measures not covered above.

JHA REVIEW & APPROVAL

| Role | Name / Signature | Date |
| --- | --- | --- |
| Prepared By (Competent Person): |  |  |
| Reviewed By (Site Superintendent): |  |  |
| Approved By (Project Manager): |  |  |

DISCLAIMER: This JHA template is provided as a starting point for site-specific hazard analysis. It must be reviewed and customized by a competent person familiar with the specific project conditions. Forbes Safety Solutions is not liable for misuse or failure to adapt this template to actual site conditions. Always consult current OSHA regulations and site-specific safety plans.