FORBES SAFETY SOLUTIONS

JOB HAZARD ANALYSIS

Drywall Hanging & Finishing

Document No: FSS-JHA-049

Regulatory References: 29 CFR 1926 Subpart M (Fall Protection), 29 CFR 1926.1153 (Respirable Crystalline Silica)

PROJECT INFORMATION

| Project Name: |  | Date: |  |
| --- | --- | --- | --- |
| Location: |  | JHA Number: |  |
| Supervisor: |  | Permit Required: | ☐ Yes ☐ No |
| Work Description: | Drywall Hanging & Finishing | Revision: | 0 |

JOB HAZARD ANALYSIS

| Task Step | Hazard | Risk Level (H/M/L) | Control Measures |
| --- | --- | --- | --- |
| 1. Material Staging and Delivery | Heavy lifting (4×12 sheet = 90+ lbs) Struck by falling material Slips/trips over staged materials | H M M | Use mechanical aids (drywall cart, boom lift) for transport; team lift for sheets; limit manual carries to 2 people per sheet minimum Stack sheets vertically against structural walls; secure to prevent tipping; establish exclusion zones during overhead lifts Maintain clear walkways; designate staging areas; housekeeping throughout shift |
| 2. Sheet Cutting and Fitting | Knife cuts Dust from scoring/cutting Eye irritation from gypsum particles | M L L | Use retractable utility knives; cut away from body; replace blades frequently (dull blades require more force) Score and snap rather than saw when possible; use dust collection on power tools Safety glasses required at all times; dust mask for extended cutting operations |
| 3. Hanging (Walls and Ceilings) | Ergonomic strain — overhead work, heavy sheets Falls from stilts or scaffolding Screw gun injuries Struck by falling sheet | H H L M | Use drywall lift (panel hoist) for all ceiling work; rotate overhead tasks every 20 minutes; provide anti-fatigue mats Drywall stilts: max 24” height on level surfaces only, no near openings/edges; scaffolding must be fully decked with guardrails per 1926 Subpart M Wear appropriate gloves; keep fingers clear of bit; use depth-setting clutch Two-person minimum for ceiling sheets; secure sheets immediately after positioning |
| 4. Taping and Mudding | Ergonomic strain (repetitive arm/shoulder motion) Skin irritation from joint compound Eye irritation from compound dust | M L L | Use automatic taping tools (bazooka/flat box) to reduce repetitive motion; take breaks every 30 minutes Wear nitrile gloves; wash hands frequently; moisturizing skin care program Safety glasses required; ventilate workspace |
| 5. Sanding | Silica dust exposure (respirable crystalline silica) Eye irritation from joint compound dust Falls from stilts/scaffolding Noise from power sanders | H M H M | MANDATORY: Use vacuum-attached sanding system per 29 CFR 1926.1153 Table 1; if not feasible — full exposure assessment and medical surveillance required; never dry sand without dust controls Wear safety goggles (sealed); face shield for overhead sanding Same stilt/scaffold requirements as hanging; additional caution — reduced visibility from dust Hearing protection required when using power sanders; maintain below 85 dB TWA with engineering controls |
| 6. Cleanup | Silica dust (re-suspended during cleanup) Slips on compound residue Cuts from scrap materials | H M L | Use HEPA-filtered vacuum for all dust cleanup — NEVER dry sweep silica-containing dust; wet methods as secondary control Damp-mop compound residue from floors; post wet floor signs Wear cut-resistant gloves when handling scrap; use covered waste containers |

REQUIRED PERSONAL PROTECTIVE EQUIPMENT (PPE)

• Safety glasses (ANSI Z87.1) — required at all times; sealed goggles for sanding operations

• N95 respirator minimum for sanding operations (half-face APR with P100 for extended sanding)

• Hard hat (ANSI Z89.1) when overhead hazards present

• Cut-resistant gloves (ANSI A3) for handling sheets and cutting

• Nitrile gloves for taping/mudding

• Steel-toe/composite-toe boots (ASTM F2413)

• Knee pads for lower wall work

• Hearing protection for power sanding operations (NRR 25+)

• Back support belt (optional, for material handling — does not replace proper lifting technique)

• High-visibility vest if working near other trades

ADDITIONAL SAFETY REQUIREMENTS

• Silica exposure control plan must be established per 29 CFR 1926.1153 before any sanding operations

• Competent person required for all scaffold erection and use

• Pre-task safety briefing required before each shift

• All stilts must be inspected daily; damaged stilts removed from service immediately

• Material Safety Data Sheets (SDS) for joint compounds must be available on site

• Housekeeping is continuous — maintain clear egress paths at all times

EMERGENCY PROCEDURES

• Call 911 for all serious injuries; notify site supervisor immediately

• For eye exposure to dust/compound: Flush with clean water for 15 minutes; seek medical attention if irritation persists

• For falls from stilts/scaffolding: Do not move victim; stabilize and call 911; report incident to supervisor

• For deep lacerations: Apply direct pressure; elevate limb; call 911 if bleeding cannot be controlled

• For suspected silica overexposure: Move to fresh air; seek medical evaluation; document exposure incident

• First aid kit located at: ____________________

• Nearest hospital/emergency room: ____________________

• Emergency assembly point: ____________________

• Site emergency contact: ____________________

WORKER SIGN-OFF

By signing below, I acknowledge that I have reviewed and understand this Job Hazard Analysis, the associated hazards, and the required control measures for this work activity.

| # | Print Name | Signature | Date | Company/Trade |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |
| 9 |  |  |  |  |
| 10 |  |  |  |  |

REVIEW & APPROVAL

| Role | Name / Signature | Date | Next Review Date |
| --- | --- | --- | --- |
| Prepared By |  |  |  |
| Reviewed By |  |  |  |
| Approved By |  |  |  |
| Site Safety Manager |  |  |  |

DISCLAIMER: This JHA is a template provided by Forbes Safety Solutions for general guidance. It must be reviewed, customized, and approved by a qualified safety professional before use on any specific project. Site-specific conditions may require additional hazard controls not listed herein.