FORBES SAFETY SOLUTIONS

JOB HAZARD ANALYSIS

HVAC Installation & Service

Document No: FSS-JHA-050

Regulatory References: 29 CFR 1926 Subpart K (Electrical), 29 CFR 1910.147 (LOTO), EPA Section 608 (Refrigerants)

PROJECT INFORMATION

| Project Name: |  | Date: |  |
| --- | --- | --- | --- |
| Location: |  | JHA Number: |  |
| Supervisor: |  | Permit Required: | ☐ Yes ☐ No |
| Work Description: | HVAC Installation & Service | Revision: | 0 |

JOB HAZARD ANALYSIS

| Task Step | Hazard | Risk Level (H/M/L) | Control Measures |
| --- | --- | --- | --- |
| 1. Equipment Placement/Rigging | Heavy lifting (condensing units 150-400+ lbs) Falls from rooftops/ladders Struck by suspended loads Crane/rigging failure | H H H H | Use crane or mechanical lifting equipment; never exceed equipment load ratings; rigging by qualified riggers only 100% tie-off for rooftop work; ladder safety (3-point contact, 4:1 ratio, secured at top); guardrails at roof edges Establish exclusion zone under suspended loads; use tag lines; never stand under crane boom Inspect all rigging equipment before each lift; qualified signal person required; pre-lift meeting for all critical lifts |
| 2. Ductwork Fabrication and Installation | Sharp sheet metal cuts Falls from ladders/lifts Noise from fabrication tools Ergonomic strain (overhead installation) | H M M M | Wear cut-resistant gloves (ANSI A4+); deburr all cut edges; handle sheet metal with proper technique — never slide hands along edges Use appropriate access equipment (scissor lift preferred over ladders); fall protection per 1926 Subpart M Hearing protection required during fabrication; maintain below 85 dB TWA where possible Use mechanical lifts for duct sections; limit overhead work duration; rotate tasks |
| 3. Refrigerant Line Brazing | Burns from torch/molten brazing material Fire — combustible materials ignition Toxic fumes from brazing flux Oxygen displacement in confined spaces | H H M H | Wear heat-resistant gloves and long sleeves; keep fire extinguisher within 10 feet; fire watch for 30 min after brazing Clear combustibles within 35 feet; use fire blankets on adjacent surfaces; hot work permit required per NFPA 51B Ensure adequate ventilation; use low-fume brazing alloys; respiratory protection if ventilation inadequate Never braze in confined spaces without ventilation and atmospheric monitoring; nitrogen purge lines during brazing to prevent oxidation and copper contamination |
| 4. Electrical Connections | Electrical shock/electrocution Arc flash Incorrect wiring (fire risk) | H H M | LOTO all circuits per 29 CFR 1910.147; verify de-energized with rated meter; only qualified electricians perform connections Arc flash PPE per NFPA 70E hazard assessment; maintain approach boundaries Verify against wiring diagrams; use appropriately rated wire/breakers; torque all connections to specification |
| 5. System Startup and Commissioning | Refrigerant exposure (asphyxiation risk in enclosed spaces) Electrical contact during testing High-pressure line failure | H M M | EPA Section 608 certified technicians only; check for leaks with electronic detector before startup; ensure ventilation in mechanical rooms; refrigerant monitors in enclosed spaces Use insulated test equipment; maintain clearances from energized components; only qualified personnel test Verify all connections and fittings before pressurizing; use nitrogen for initial pressure test; stand clear of fittings during test |
| 6. Maintenance/Service Work | Electrical contact with live components Refrigerant burns (rapid expansion) and inhalation Confined spaces (ductwork, mechanical rooms) Biological hazards (mold, bacteria in drain pans/coils) | H H H M | LOTO before any service; verify de-energized; assume all wiring is live until tested Recover refrigerant before opening system; wear safety goggles and gloves for potential releases; EPA 608 compliance Evaluate for confined space entry per 29 CFR 1910.146; atmospheric testing, attendant, rescue plan if applicable Wear N95 respirator and gloves when cleaning coils/drain pans; use EPA-registered biocide; avoid pressure washing mold (aerosolization) |

REQUIRED PERSONAL PROTECTIVE EQUIPMENT (PPE)

• Safety glasses (ANSI Z87.1) — required at all times

• Hard hat (ANSI Z89.1 Type I, Class E) when overhead hazards present

• Cut-resistant gloves (ANSI A4+) for sheet metal handling

• Heat-resistant gloves for brazing operations

• Voltage-rated gloves for electrical work (class per voltage level)

• Steel-toe/composite-toe EH-rated boots (ASTM F2413)

• Hearing protection (NRR 25+) for fabrication and power tool use

• N95 or half-face respirator for confined space and coil cleaning

• Full-body harness with shock-absorbing lanyard for rooftop/elevated work (ANSI Z359)

• Arc-rated clothing per NFPA 70E for electrical work

• High-visibility vest for exterior work areas

ADDITIONAL SAFETY REQUIREMENTS

• EPA Section 608 certification required for all technicians handling refrigerants

• Hot work permit required for all brazing operations per NFPA 51B

• Confined space entry permit required for ductwork and mechanical room entry where applicable

• Pre-task safety briefing required before each shift or when conditions change

• All LOTO procedures must follow site-specific energy control program per 29 CFR 1910.147

• Refrigerant leak detection and recovery equipment must be on site during all commissioning and service

• Coordinate with other trades to prevent conflicting operations (e.g., welding near refrigerant lines)

EMERGENCY PROCEDURES

• Call 911 for all serious injuries; notify site supervisor immediately

• For refrigerant inhalation: Move to fresh air immediately; administer oxygen if available; call 911; do NOT induce vomiting if ingested

• For refrigerant skin/eye contact (frostbite): Do NOT rub affected area; flush with lukewarm water; seek medical attention

• For electrical shock: De-energize if safe; do NOT touch victim in contact with source; call 911; begin CPR/AED if trained

• For burns from brazing: Cool with water for 20 minutes; do not apply ice; cover loosely; seek medical attention for burns larger than palm-sized

• For falls: Do not move victim; stabilize and call 911; activate rescue plan for suspended workers

• Fire response: Use ABC extinguisher; activate fire alarm; evacuate per site plan

• First aid kit and AED located at: ____________________

• Nearest hospital/emergency room: ____________________

• Emergency assembly point: ____________________

• Site emergency contact: ____________________

WORKER SIGN-OFF

By signing below, I acknowledge that I have reviewed and understand this Job Hazard Analysis, the associated hazards, and the required control measures for this work activity.

| # | Print Name | Signature | Date | Company/Trade |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |
| 9 |  |  |  |  |
| 10 |  |  |  |  |

REVIEW & APPROVAL

| Role | Name / Signature | Date | Next Review Date |
| --- | --- | --- | --- |
| Prepared By |  |  |  |
| Reviewed By |  |  |  |
| Approved By |  |  |  |
| Site Safety Manager |  |  |  |

DISCLAIMER: This JHA is a template provided by Forbes Safety Solutions for general guidance. It must be reviewed, customized, and approved by a qualified safety professional before use on any specific project. Site-specific conditions may require additional hazard controls not listed herein.