FORBES SAFETY SOLUTIONS

JOB HAZARD ANALYSIS

Mechanical Insulation / Pipe Wrapping

REGULATORY REFERENCES

• 29 CFR 1926.1101 — Asbestos (removal of existing insulation)

• 29 CFR 1910.132 — General Requirements for PPE

• 29 CFR 1926.451 — Scaffolding (General Requirements)

• 29 CFR 1926.1053 — Ladders

• 29 CFR 1910.1200 — Hazard Communication

• EPA NESHAP 40 CFR 61, Subpart M — Asbestos

SCOPE & DESCRIPTION

This JHA covers mechanical insulation and pipe wrapping operations including existing insulation assessment and removal (with asbestos identification), surface preparation, insulation cutting and fitting, metal jacketing and banding, sealing and finishing, and waste disposal. Legacy insulation may contain asbestos, requiring specialized abatement procedures. Workers face hazards from thermal burns on hot pipes, sharp tool injuries, ergonomic strain from overhead work, fiber inhalation, and falls from scaffolding or ladders.

PROJECT INFORMATION

| Project Name: |  | Date: |  |
| --- | --- | --- | --- |
| Project Number: |  | Location: |  |
| Contractor: |  | Supervisor: |  |
| JHA Prepared By: |  | Reviewed By: |  |

JOB HAZARD ANALYSIS

| Task Step | Hazard | Risk Level (H/M/L) | Control Measures |
| --- | --- | --- | --- |
| 1. Existing Insulation Assessment / Removal | Asbestos exposure (friable ACM) Other hazardous fibers (refractory ceramic) Falls from scaffolding/ladders Cuts from removal tools | H | AHERA-accredited inspector must assess before any disturbance Assume ACM until lab analysis confirms otherwise (NESHAP) Licensed asbestos abatement contractor for confirmed ACM Full negative-pressure enclosure for friable ACM removal Workers in powered air-purifying respirators (PAPR) Disposable Tyvek suits, gloves, boot covers Wet removal methods to minimize fiber release Air monitoring during and after removal |
| 2. Surface Preparation | Thermal burns from hot pipes/equipment Sharp edges on pipe supports/hangers Chemical exposure from cleaning solvents Residual asbestos fibers | M | Verify pipe/equipment temperature — lockout/tagout if feasible Heat-resistant gloves rated for pipe temperature Cut-resistant gloves for handling sharp metal Confirm asbestos clearance before surface prep begins Clean surfaces per insulation manufacturer specifications |
| 3. Insulation Cutting & Fitting | Knife/tool cuts (box cutters, insulation knives) Dust/fiber inhalation (fiberglass, mineral wool) Eye irritation from fibers Ergonomic strain — overhead/awkward positions | M | Use retractable safety knives; cut away from body Cut-resistant gloves mandatory N95 respirator minimum for fiberglass/mineral wool cutting Safety glasses or goggles — no contact lenses Long sleeves and pants to prevent skin irritation Rotate overhead work; use adjustable scaffolding to optimize height Pre-cut insulation at ground level when possible |
| 4. Jacketing / Banding | Lacerations from sheet metal edges Pinch points during banding tool operation Falls from scaffolding Dropped tools/materials from height | H | Cut-resistant gloves (ANSI A4 minimum) for metal handling Deburr all cut metal edges before installation Banding tool training required — keep hands clear of tensioner 100% tie-off on scaffolding above 6 ft Scaffold daily inspection by competent person Tool lanyards for all hand tools at height Toe boards and debris nets to protect workers below |
| 5. Sealing & Finishing | Chemical exposure (sealants, mastics, adhesives) Slippery surfaces from sealant overspray Ergonomic strain from repetitive motion | M | Review SDS for all sealants and adhesives Chemical-resistant gloves; avoid skin contact Ventilation in enclosed areas — organic vapor respirator if needed Clean overspray immediately to prevent slip hazard Ergonomic breaks; alternate between tasks |
| 6. Waste Disposal | Asbestos waste handling (regulated) Fiberglass/mineral wool skin irritation Sharp metal scraps — puncture/laceration Manual handling of heavy waste bags | M | Asbestos waste double-bagged in labeled 6-mil poly bags Non-ACM insulation waste in covered containers Metal scraps in designated sharp-waste bins All waste characterized and disposed per EPA/state regulations Proper lifting techniques; limit bag weight to 50 lbs Maintain waste manifests for all hazardous materials |

PERSONAL PROTECTIVE EQUIPMENT (PPE)

| PPE Required |  |
| --- | --- |
| ☐ Hard Hat (ANSI Z89.1) | ☐ Safety Glasses / Goggles (fiber protection) |
| ☐ N95 or P100 Respirator (fiber cutting/removal) | ☐ PAPR (asbestos removal operations) |
| ☐ Cut-Resistant Gloves (ANSI A4 for jacketing) | ☐ Heat-Resistant Gloves (hot pipe work) |
| ☐ Long-Sleeve Shirt / Tyvek Coveralls | ☐ Steel-Toe Safety Boots |
| ☐ Fall Protection Harness (scaffolding above 6 ft) | ☐ Tool Lanyards (work at height) |

EMERGENCY PROCEDURES

1. ASBESTOS FIBER RELEASE: Stop work immediately. Evacuate area and seal off. Notify site safety officer and client. Do NOT attempt cleanup — licensed abatement contractor required. Air monitoring must be conducted before re-entry.

2. THERMAL BURN FROM HOT PIPE: Cool burn with clean, cool water for 10+ minutes. Remove clothing near burn unless adhered. Cover with sterile dressing. Seek medical attention for second-degree burns or larger.

3. SEVERE LACERATION (METAL/KNIFE): Apply direct pressure with clean cloth. Elevate injured limb. Call 911 if bleeding is uncontrolled. Do not remove embedded objects.

4. FALL FROM SCAFFOLDING: Call 911. Do not move victim if spinal injury suspected. If in fall arrest harness, initiate suspension trauma rescue within 15 minutes. Monitor vitals.

5. FIBER INHALATION / RESPIRATORY DISTRESS: Move to fresh air. If difficulty breathing, administer oxygen if available. Call 911. Provide SDS information on fiber type to responders.

6. EMERGENCY CONTACTS: 911 for all emergencies. Site Safety Officer: ____________. Poison Control: 1-800-222-1222. Asbestos Abatement Coordinator: ____________.

ADDITIONAL NOTES / SITE-SPECIFIC REQUIREMENTS

• All existing insulation must be surveyed for asbestos content before any disturbance per NESHAP requirements.

• Insulation workers must complete asbestos awareness training per 29 CFR 1926.1101(k)(9).

• Scaffolding must be erected and inspected by a competent person per Subpart L.

• Hot pipe insulation requires temperature verification — lockout/tagout of heat source preferred.

• Material Safety Data Sheets for all insulation products, adhesives, and sealants must be on-site.

WORKER SIGN-OFF

By signing below, I confirm that I have reviewed and understand this JHA, including all identified hazards and required control measures.

| # | Print Name | Signature | Date | Company/Trade |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |
| 9 |  |  |  |  |
| 10 |  |  |  |  |

REVIEW & APPROVAL

This JHA has been reviewed and approved by the following authorized personnel:

| Role | Signature | Date |
| --- | --- | --- |
| Prepared By (Safety Professional) |  |  |
| Reviewed By (Site Supervisor) |  |  |
| Approved By (Project Manager) |  |  |
| Client Representative (if required) |  |  |

REVISION HISTORY

| Revision | Date | Description | Approved By |
| --- | --- | --- | --- |
| 0 |  | Initial Release |  |
| 1 |  |  |  |

DISCLAIMER: This Job Hazard Analysis is a guideline for identifying hazards and implementing controls. It does not replace site-specific safety plans, competent person assessments, or regulatory requirements. All workers must comply with applicable OSHA standards and company safety policies. This JHA must be reviewed and updated whenever work conditions change.