FORBES SAFETY SOLUTIONS

JOB HAZARD ANALYSIS

Pole Setting / Utility Pole Installation

Regulatory References: 29 CFR 1910.269, 29 CFR 1926 Subpart V, NESC

Document No: FSS-JHA-038  |  Revision: 0  |  Date: _______________

| Project Name: |  | Date: |  |
| --- | --- | --- | --- |
| Location: |  | JHA Number: |  |
| Supervisor: |  | Permit Required: | ☐ Yes ☐ No |
| Prepared By: |  | Reviewed By: |  |

JOB HAZARD ANALYSIS TABLE

| Task Step | Hazard | Risk Level (H/M/L) | Control Measures |
| --- | --- | --- | --- |
| 1. Hole Augering / Excavation | Underground utility strike Excavation cave-in Auger caught on obstruction Noise/vibration exposure | H H M M | Complete 811 utility locate request minimum 48 hours prior Hand dig within 18 inches of marked utilities (tolerance zone) Verify auger guard and emergency shutoff are operational Maintain safe distance from rotating auger (10-ft minimum) Barricade open holes immediately; cover if left unattended Wear hearing protection during augering operations |
| 2. Pole Inspection & Rigging | Defective pole (rot, splits, cracks) Improper rigging failure Struck by during handling | M H H | Inspect pole for damage, splits, rot, and hardware defects per ANSI O5.1 Reject any pole failing visual or sounding inspection Attach rigging at designated lift point (manufacturer recommendation) Use tag lines to control pole swing (minimum 2 tag lines) All personnel clear of swing radius during rigging Verify sling/choker capacity exceeds pole weight by 5:1 safety factor |
| 3. Pole Setting with Crane/Boom Truck | Struck by pole during setting Overhead line contact Crane tip-over Pinch points | H H H M | Conduct pre-lift meeting covering load weight, radius, ground conditions Maintain minimum approach distance (MAD) from energized lines per 29 CFR 1926.1408 Use a qualified signal person; establish clear hand signals Set outriggers on firm ground with cribbing; verify level Do NOT exceed crane load chart capacity at any boom angle Keep all personnel out of the swing/fall zone Use tag lines to guide pole into hole; never use hands directly |
| 4. Pole Tamping & Backfill | Pole shifting/falling before secured Hand/foot injury during tamping Ergonomic strain | H M M | Do NOT release crane/boom until pole is plumb and tamped Tamp in 6-inch lifts with approved tamping equipment Keep hands and feet clear of tamping zone Verify pole plumb with level before final backfill Compact backfill to prevent future pole settlement Use mechanical tamper for large-diameter holes when available |
| 5. Hardware & Crossarm Installation | Falls during climbing Dropped tools/materials Overhead line proximity | H M H | 100% fall protection required when climbing or working above 4 feet Use approved climbing equipment (belt, gaffs, positioning lanyard) Haul hardware and tools up with hand line; never carry while climbing Install tool lanyards on all hand tools Maintain MAD from all energized conductors Use insulated tools if working near energized lines |
| 6. Conductor Stringing | Overhead line contact Conductor snap-back Falls from elevation Traffic near work zone | H H H M | Establish proper grounding/bonding before stringing near energized lines Use calibrated tension equipment; do not exceed conductor rated tension All workers on poles tied off 100% during stringing Establish traffic control per MUTCD for road crossings Use pulling ropes and guards at road crossings Clear area below stringing operations of all personnel |

REQUIRED PERSONAL PROTECTIVE EQUIPMENT (PPE)

| ☐ Hard Hat (ANSI Z89.1 Type I, Class E) | ☐ Safety Glasses (ANSI Z87.1) |
| --- | --- |
| ☐ High-Visibility Vest (ANSI/ISEA 107 Class 2) | ☐ Steel-Toe Boots (ASTM F2413) |
| ☐ Leather Work Gloves | ☐ FR-Rated Clothing (near energized lines) |
| ☐ Climbing Belt and Positioning Lanyard | ☐ Fall Protection Harness (100% tie-off above 4 ft) |
| ☐ Hearing Protection (during augering) | ☐ Rubber Insulating Gloves (if near energized lines) |

EMERGENCY PROCEDURES

1. If a pole begins to fall uncontrolled, all personnel move perpendicular to the fall path. Sound air horn or yell 'POLE DOWN' immediately.

2. For overhead line contact with equipment, assume all metal is energized. Stay in the cab or hop clear with feet together. Do NOT touch the ground and equipment simultaneously.

3. In case of excavation cave-in, do NOT enter the excavation. Call 911 and initiate rescue per the site-specific rescue plan.

4. For crane tip-over, evacuate the radius of the boom length plus pole length. The operator should remain in the cab if safer than exiting.

5. For falls from poles, call 911 immediately. Do NOT move the victim unless in immediate life-threatening danger. Administer first aid for suspension trauma.

6. All struck-by incidents require immediate medical evaluation even if no visible injury. Report to site supervisor and complete incident documentation.

| Emergency Contact: | 911 / Site-specific: _______________ |
| --- | --- |
| Assembly Point: |  |
| Nearest Hospital: |  |
| First Aid Kit Location: |  |

WORKER SIGN-OFF

By signing below, I acknowledge that I have reviewed and understand this Job Hazard Analysis, the associated hazards, and the required control measures.

| # | Print Name | Signature | Company | Date |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |
| 9 |  |  |  |  |
| 10 |  |  |  |  |

REVIEW & APPROVAL

This JHA has been reviewed and approved by the following authorized personnel:

| Role | Name / Signature | Date |
| --- | --- | --- |
| Prepared By (Safety Professional) |  |  |
| Reviewed By (Site Supervisor) |  |  |
| Approved By (Project Manager) |  |  |
| Client Representative (if required) |  |  |

| Next Review Date: |  |
| --- | --- |
| Revision Notes: |  |

FORBES SAFETY SOLUTIONS  |  Committed to Zero Incidents

This document is valid only when all sections are completed and signatures obtained.