FORBES SAFETY SOLUTIONS

JOB HAZARD ANALYSIS

Solar Panel Installation

Document No: FSS-JHA-046

Regulatory References: 29 CFR 1926 Subpart M (Fall Protection), NFPA 70, 29 CFR 1926 Subpart K (Electrical)

PROJECT INFORMATION

| Project Name: |  | Date: |  |
| --- | --- | --- | --- |
| Location: |  | JHA Number: |  |
| Supervisor: |  | Permit Required: | ☐ Yes ☐ No |
| Work Description: | Solar Panel Installation | Revision: | 0 |

JOB HAZARD ANALYSIS

| Task Step | Hazard | Risk Level (H/M/L) | Control Measures |
| --- | --- | --- | --- |
| 1. Racking/Mounting Structure Installation | Falls from roof/elevated structure Struck by falling materials Heat stress (rooftop work) Pinch points during assembly | H M H M | 100% tie-off; install guardrails/warning lines per 1926 Subpart M Establish controlled access zone below; use tool lanyards Implement heat illness prevention plan (water, rest, shade); monitor wet-bulb globe temperature Wear cut-resistant gloves; use proper torque wrenches for fasteners |
| 2. Panel Placement and Securing | Falls from height Struck by panels in wind Ergonomic strain (repetitive bending/kneeling) Electrical shock — panels energized by sunlight | H H M H | Personal fall arrest system (PFAS) with roof anchors rated to 5,000 lbs Stop work when winds exceed 25 mph; two-person lifts; secure panels immediately Use knee pads; rotate tasks every 30 min; provide ergonomic training Cover panels with opaque material during handling; wear voltage-rated gloves; assume all panels are energized in daylight |
| 3. DC Wiring and String Connections | Electrical shock from DC voltage (cannot fully de-energize during daylight) Arc flash at connectors Cuts from wire stripping | H H L | Work with voltage-rated gloves (Class 00 min); use insulated tools; cover exposed conductors immediately Verify polarity before making connections; use MC4-rated connectors only; never disconnect under load Use proper wire strippers; wear cut-resistant gloves |
| 4. Inverter/Combiner Box Installation | Electrical shock (AC and DC) Arc flash at combiner/inverter Heavy lifting | H H M | LOTO all AC sources; verify zero energy state; DC side — open all string disconnects and verify with rated meter Perform arc flash risk assessment per NFPA 70E; wear appropriate PPE category Use mechanical lifting aids; two-person team for units over 50 lbs |
| 5. System Testing and Commissioning | Electrical shock during testing Arc flash at high-voltage points Unexpected energization | H H M | Only qualified electrical workers perform testing; use CAT III/IV rated meters Wear arc-rated PPE per NFPA 70E; maintain approach boundaries Establish clear communication protocol; use lockout tags on all disconnects until final commissioning |
| 6. Site Cleanup | Falls from ladders Cuts from debris/packaging Slip/trip hazards | M L M | Three-point contact on ladders; inspect before use Wear cut-resistant gloves; dispose of packaging properly Clear walkways continuously; remove tools and debris from roof surface |

REQUIRED PERSONAL PROTECTIVE EQUIPMENT (PPE)

• Hard hat (ANSI Z89.1 Type I, Class E)

• Safety glasses (ANSI Z87.1) with side shields

• Voltage-rated gloves (Class 00 minimum) with leather protectors for electrical work

• Arc-rated clothing per NFPA 70E hazard assessment

• Cut-resistant gloves (ANSI A4 minimum) for material handling

• Steel-toe/composite-toe boots (ASTM F2413)

• Full-body harness with shock-absorbing lanyard and roof anchor (ANSI Z359)

• High-visibility vest (ANSI/ISEA 107 Class 2)

• Knee pads for rooftop work

• Sunscreen (SPF 30+) and wide-brim hard hat attachment

ADDITIONAL SAFETY REQUIREMENTS

• Solar panels cannot be fully de-energized during daylight hours — treat all DC conductors as energized

• Pre-task safety briefing (tailgate talk) required before each shift

• Weather monitoring required — stop all rooftop work during lightning, high winds (>25 mph), or wet conditions

• All fall protection equipment must be inspected daily before use

• Only qualified electricians shall perform wiring and commissioning activities per NFPA 70

• Rooftop fire safety: maintain clear access to fire department connections and roof hatches at all times

EMERGENCY PROCEDURES

• Call 911 for all life-threatening injuries; notify site supervisor immediately

• For electrical shock: Do NOT touch the victim if still in contact with source; de-energize if safe to do so; call 911; begin CPR/AED if trained

• For falls: Do not move victim; stabilize and call 911; activate rescue plan for suspended workers within 6 minutes

• For heat-related illness: Move to shade immediately; apply cool water; call 911 if altered consciousness

• First aid kit and AED located at: ____________________

• Nearest hospital/emergency room: ____________________

• Emergency assembly point: ____________________

• Site emergency contact: ____________________

WORKER SIGN-OFF

By signing below, I acknowledge that I have reviewed and understand this Job Hazard Analysis, the associated hazards, and the required control measures for this work activity.

| # | Print Name | Signature | Date | Company/Trade |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |
| 9 |  |  |  |  |
| 10 |  |  |  |  |

REVIEW & APPROVAL

| Role | Name / Signature | Date | Next Review Date |
| --- | --- | --- | --- |
| Prepared By |  |  |  |
| Reviewed By |  |  |  |
| Approved By |  |  |  |
| Site Safety Manager |  |  |  |

DISCLAIMER: This JHA is a template provided by Forbes Safety Solutions for general guidance. It must be reviewed, customized, and approved by a qualified safety professional before use on any specific project. Site-specific conditions may require additional hazard controls not listed herein.