JOB HAZARD ANALYSIS

Structural Steel Welding

Regulatory References: 29 CFR 1926.350-354, AWS D1.1 Structural Welding Code, 29 CFR 1926 Subpart R (Steel Erection)

PROJECT INFORMATION

| Project Name: |  | Date: |  |
| --- | --- | --- | --- |
| Location: |  | JHA Number: | JHA-035 |
| Supervisor: |  | Reviewed By: |  |
| Trade/Activity: | Structural Steel Welding | Permit Required: | ☐ Yes ☐ No |

JOB HAZARD ANALYSIS

| Task Step | Hazard | Risk Level | Control Measures |
| --- | --- | --- | --- |
| 1. Weld Prep & Joint Fit-Up | Falls from steel (elevated work) Cuts from sharp edges Pinch points during fit-up Grinding sparks/debris | H M M M | 100% tie-off when working at height — fall protection per Subpart R Inspect walking/working surfaces for ice, grease, debris Use cut-resistant gloves when handling steel members Verify bolt torque and fit-up alignment before welding Grind joint prep with guarded grinder; wear face shield Secure members with erection bolts/clamps before releasing crane |
| 2. Equipment Setup & Cable Routing | Electrical shock Trip hazards from cables Damaged equipment | H M M | Inspect welding machine, cables, electrode holder, and ground clamp daily Replace damaged cables, connectors, or insulation immediately Route cables to minimize trip hazards; cover or elevate at walkways Ensure proper grounding — ground clamp directly on workpiece or structure Keep welder dry and away from standing water Verify correct voltage/polarity settings for weld procedure |
| 3. Tack Welding | UV radiation (arc flash) Burns from spatter Fume exposure Fire | H M H H | Wear welding helmet (Shade 10-14) and FR clothing for ALL welds including tacks Erect welding screens/curtains to protect adjacent workers from UV Clear combustibles within 35 ft or cover with fire-resistant blankets Position fire extinguisher within 25 ft of welding location Assign fire watch for duration of work plus 30 minutes post-weld Verify adequate ventilation or use local exhaust / fume extraction |
| 4. Multi-Pass Welding | Prolonged fume exposure (Mn, Cr⁶⁺) UV radiation Burns Fire Fatigue at height | H H M H M | Use local exhaust ventilation (LEV) positioned within 12-18 inches of arc Wear P100 respirator with OV cartridge — required for galvanized or stainless steel Monitor exposure to manganese and hexavalent chromium per 29 CFR 1926.1152/1126 Take breaks to prevent fatigue — especially critical at elevated positions Maintain fire watch — continuous observation during multi-pass welding Inspect interpass temperature with contact thermometer per WPS requirements |
| 5. Post-Weld Inspection | Burns from hot steel Falls during inspection Grinding hazards | M H M | Allow welds to cool before handling — test temperature before touching Maintain fall protection during inspection — do NOT remove harness Use magnetic particle or ultrasonic testing per AWS D1.1 inspection requirements Grind repairs with guarded grinder; wear face shield and hearing protection Mark rejected welds clearly for repair; document all inspection results Verify weld meets WPS parameters (size, profile, discontinuities) |
| 6. Cleanup | Hot slag and spatter Welding rod stub disposal Fire from smoldering materials | M L M | Allow all slag and spatter to cool before sweeping up Collect used welding rod stubs in metal container — do NOT leave on steel members Fire watch inspects area 30 minutes after last weld — confirm no smoldering Disconnect and properly store welding cables and equipment Clean work area of all debris, tools, and consumables Complete weld log and daily inspection documentation |

Additional Notes: Hot work permit required for all welding operations. Fire watch must remain 30 minutes after welding ceases. All welders must have current AWS certification for the applicable process (SMAW, FCAW, GMAW). Welding on galvanized steel requires enhanced respiratory protection for hexavalent chromium.

PERSONAL PROTECTIVE EQUIPMENT (PPE)

| Required PPE |  |
| --- | --- |
| ☑ Hard Hat (ANSI Z89.1) | ☑ Welding Helmet (Shade 10-14) |
| ☑ Safety Glasses under Helmet (ANSI Z87.1) | ☑ Welding Jacket / FR Clothing |
| ☑ Welding Gloves (leather gauntlet) | ☑ Steel/Composite Toe Boots |
| ☑ Hearing Protection | ☑ Respirator (P100 w/ OV cartridge for fumes) |
| ☑ Fall Protection Harness & Lanyard | ☑ Fire-Resistant Boot Covers / Spats |

EMERGENCY PROCEDURES

• Fall from steel: Activate rescue plan immediately. Trained rescuers deploy to retrieve fallen worker within minutes to prevent suspension trauma. Call 911.

• Welding fume overexposure (manganese, hexavalent chromium): Move worker to fresh air immediately. Seek medical evaluation. Review ventilation adequacy.

• Burn injury: Cool burn with water. Remove smoldering clothing (not if adhered). Cover with sterile dressing. Seek medical attention.

• Fire from welding sparks: Use ABC fire extinguisher. Activate fire watch procedure. If fire cannot be controlled immediately, evacuate and call 911.

• Electrical shock from welder: De-energize equipment. Do NOT touch victim if still in contact with energized source. Administer CPR if needed. Call 911.

• Arc flash eye injury (flash burn): Irrigate eyes with clean water. Apply cool compress. Seek medical evaluation — symptoms may be delayed 6-12 hours.

• Report all incidents, near-misses, and equipment malfunctions to supervisor immediately.

WORKER SIGN-OFF

By signing below, I acknowledge that I have reviewed and understand this Job Hazard Analysis, the associated hazards, and the required control measures for the work to be performed.

| # | Print Name | Signature | Company | Date | Time |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |
| 6 |  |  |  |  |  |
| 7 |  |  |  |  |  |
| 8 |  |  |  |  |  |
| 9 |  |  |  |  |  |
| 10 |  |  |  |  |  |

REVIEW & APPROVAL

| Role | Name / Signature | Title | Date |
| --- | --- | --- | --- |
| Prepared By |  |  |  |
| Reviewed By |  |  |  |
| Approved By |  |  |  |
| Site Supervisor |  |  |  |

REVISION HISTORY

| Rev. | Date | Description | Author |
| --- | --- | --- | --- |
| 0 |  | Initial Release |  |
| 1 |  |  |  |