FORBES SAFETY SOLUTIONS

JOB HAZARD ANALYSIS

TREE TRIMMING / VEGETATION MANAGEMENT NEAR POWER LINES

29 CFR 1910.269 | ANSI Z133 — Arboricultural Operations Safety

PROJECT INFORMATION

| Company Name: |  | Date: |  |
| --- | --- | --- | --- |
| Project Name: |  | Supervisor: |  |
| JHA Number: |  | Revision: |  |
| Work Activity: | Tree Trimming / Vegetation Management Near Energized Power Lines |  |  |
| References: | 29 CFR 1910.269; 29 CFR 1910.331-335; ANSI Z133-2017; OSHA 1910.333(c)(3); MUTCD |  |  |

MINIMUM APPROACH DISTANCE (MAD) REFERENCE TABLE

Per 29 CFR 1910.269 Table R-6 / Table R-7 — Verify voltage with utility before work begins.

| VOLTAGE RANGE | MAD — QUALIFIED LINE-CLEARANCE | MAD — UNQUALIFIED PERSONNEL |
| --- | --- | --- |
| 0 – 50 V | No specific distance | No specific distance |
| 51 V – 750 V | Avoid contact | Avoid contact |
| 751 V – 15 kV | 2 ft 1 in (0.64 m) | 10 ft (3.05 m) |
| 15.1 kV – 36 kV | 2 ft 4 in (0.72 m) | 10 ft (3.05 m) |
| 36.1 kV – 46 kV | 2 ft 7 in (0.77 m) | 10 ft (3.05 m) |
| 46.1 kV – 72.5 kV | 3 ft 3 in (0.98 m) | 10 ft (3.05 m) |
| 72.6 kV – 121 kV | 3 ft 4 in (1.02 m) | 10 ft (3.05 m) |
| 138 kV – 145 kV | 3 ft 10 in (1.17 m) | 10 ft (3.05 m) |
| 161 kV – 169 kV | 4 ft 3 in (1.29 m) | 16 ft (4.88 m) |
| 230 kV – 242 kV | 5 ft 3 in (1.59 m) | 16 ft (4.88 m) |

HAZARD ANALYSIS

| TASK STEP | HAZARD | RISK LEVEL | CONTROL MEASURES |
| --- | --- | --- | --- |
| 1. Pre-Work Site Assessment & Utility Coordination | • Unidentified energized power lines (electrocution) • Overhead widow-makers (dead/hanging limbs) • Uneven terrain / hidden ground hazards • Inadequate communication with utility provider | HIGH | • Contact utility company 72 hrs minimum before work; confirm line status • Walk-through hazard assessment by qualified person before work begins • Identify all power lines within work zone; determine voltage level • Reference MAD table to establish approach distances • Mark exclusion zones with flagging tape / barricades • Document assessment on pre-work checklist |
| 2. Establishing Work Zone & Traffic Control | • Vehicle strikes on roadway work zones • Unauthorized pedestrian entry into drop zone • Inadequate sight distance for traffic | HIGH | • Traffic control plan per MUTCD (Manual on Uniform Traffic Control Devices) • Cones, signs, and flaggers as required for roadway work • High-visibility apparel Class 2 minimum; Class 3 for flaggers • Establish drop zone perimeter — no unauthorized entry • Spotter positioned for traffic exposure areas • Work vehicle emergency lighting activated |
| 3. Aerial Lift Positioning & Setup | • Electrocution from boom contact with power lines • Aerial lift tip-over on uneven ground • Falls from aerial lift bucket • Entanglement with overhead conductors during positioning | HIGH | • Only line-clearance qualified workers within 10 ft of energized lines per 29 CFR 1910.269 • Insulated aerial lift (Category A or B per ANSI A92.2) for line-clearance work • Set outriggers on firm, level ground; use cribbing/pads as needed • Operator must wear full-body harness attached to boom/bucket anchor point • Pre-use inspection of lift per manufacturer checklist • Maintain MAD at all times — use spotter for boom movements |
| 4. Limb Cutting & Removal Sequence | • Electrocution from limb contact with power line • Chain saw kickback • Struck-by falling limbs (gravity hazard) • Overhead widow-makers dropping unexpectedly • Hand/arm laceration from saw | HIGH | • Cut sequence planned to direct limb fall away from power lines • Three-cut method for large limbs (undercut, top cut, stub removal) • Chain saw with anti-kickback chain and chain brake engaged between cuts • Ground crew clear of drop zone; voice/radio communication before each cut • Inspect canopy for widow-makers before and during cutting • Never reach across or work between energized conductors • Use insulated tools rated for voltage present when within MAD |
| 5. Chipper Operations | • Entanglement / caught-in chipper feed mechanism • Struck-by ejected material from discharge chute • Noise exposure (>100 dBA typical) • Flying debris eye/face injuries | HIGH | • Dedicated spotter positioned at chipper infeed at all times • No loose clothing, jewelry, or lanyards near chipper • Feed brush butt-end first; stand to the side of infeed • Emergency stop bar functional — test before each use • Position discharge chute away from personnel and traffic • Hearing protection required (NRR 25+ minimum) • Face shield and safety glasses required during chipper operation |
| 6. Site Cleanup & Restoration | • Manual handling strain (heavy limbs/debris) • Tripping on debris during cleanup • Traffic exposure during barricade removal • Residual overhead hazards (partially cut limbs) | MEDIUM | • Team lifting for debris >50 lbs; use mechanical assistance when possible • Systematic cleanup — work outward from trunk to perimeter • Final overhead inspection before releasing work zone • Remove traffic control devices in reverse order of placement • Document completion on JHA close-out section |

PERSONAL PROTECTIVE EQUIPMENT (PPE) REQUIREMENTS

| PPE ITEM | STANDARD | NOTES |
| --- | --- | --- |
| Hard Hat (Type I, Class E) | ANSI/ISEA Z89.1-2014 | Electrical rated; required at all times in work zone |
| Safety Glasses + Face Shield | ANSI/ISEA Z87.1-2020 | Face shield for chipper / chain saw operations |
| Chain Saw Chaps (8+ layers) | ASTM F1897 / UL Certified | Required for all ground-level chain saw operations |
| Chain Saw Protective Gloves | ASTM F2675-19 | Left-hand cut protection minimum |
| Steel-Toe / Chain Saw Boots | ASTM F2413-18 / F2413-11 | Cut-resistant chain saw boots for ground operations |
| High-Vis Safety Vest (Class 2/3) | ANSI/ISEA 107-2020 | Class 3 for flaggers; Class 2 minimum for all crew |
| Fall Protection Harness | ANSI Z359.11-2021 | Full-body harness for aerial lift / climbing operations |
| Hearing Protection (NRR 25+) | ANSI S3.19 / S12.6 | Required during chipper and chain saw operations |
| Dielectric Gloves (if applicable) | ASTM D120 / OSHA Table R-6 | Voltage-rated rubber gloves for line-clearance qualified workers |
| Climbing Saddle / Lanyard | ANSI Z133-2017 Sec. 8 | For tree climbing operations when aerial lift not feasible |

EMERGENCY PROCEDURES

Emergency Contact: 911 for all life-threatening emergencies

First Aid Station: Located at _________________ (identify before each shift)

Electrocution / Electrical Contact: DO NOT touch the victim if still in contact with energized source. Call 911 immediately. De-energize line if possible (utility dispatch: ___________). Begin CPR/AED only after source is de-energized or victim is clear. All crew trained in CPR/AED required on site.

Fall from Height: Do not move injured person unless in immediate danger. Stabilize C-spine if suspected injury. Call 911. Provide first aid as trained.

Chain Saw Laceration: Apply direct pressure to wound. Use tourniquet for severe arterial bleeding per Stop the Bleed protocol. Call 911.

Chipper Entanglement: Hit emergency stop immediately. Call 911. Do not attempt to reverse-free a victim—wait for emergency services.

Struck-by Falling Limb: Assess injuries. Call 911 for head, neck, or back injuries. Maintain airway. Provide first aid as trained.

Nearest Hospital: _________________________________ (identify before each shift)

CREW ACKNOWLEDGMENT & SIGN-OFF

By signing below, I acknowledge that I have reviewed this JHA, understand the identified hazards and required control measures, and agree to follow all safety procedures. Line-clearance qualified workers must be identified with (Q) next to their name.

| # | PRINTED NAME | SIGNATURE | COMPANY / TRADE | DATE | UNDERSTOOD (INITIALS) |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |
| 6 |  |  |  |  |  |
| 7 |  |  |  |  |  |
| 8 |  |  |  |  |  |
| 9 |  |  |  |  |  |
| 10 |  |  |  |  |  |

REVIEW & APPROVAL

| ROLE | SIGNATURE | DATE | PRINTED NAME |
| --- | --- | --- | --- |
| Prepared By: |  |  |  |
| Reviewed By (Supervisor): |  |  |  |
| Approved By (Safety Mgr): |  |  |  |
| Client/GC Representative: |  |  |  |

DISCLAIMER

This JHA is a template intended to be customized for site-specific conditions. It does not replace a qualified safety professional's assessment or utility coordination. Employers remain responsible for compliance with all applicable OSHA standards, ANSI Z133, and local regulations. Only line-clearance qualified workers may work within minimum approach distances of energized power lines. This document should be reviewed and updated whenever work conditions change.