FORBES SAFETY SOLUTIONS

JOB HAZARD ANALYSIS

Waterproofing / Below-Grade Work

REGULATORY REFERENCES

• 29 CFR 1926 Subpart P — Excavations

• 29 CFR 1926.55 — Gases, Vapors, Fumes, Dusts, and Mists

• 29 CFR 1926.651 — Specific Excavation Requirements

• 29 CFR 1926.652 — Requirements for Protective Systems

• 29 CFR 1926.150-155 — Fire Protection and Prevention

• OSHA Technical Manual, Section V, Ch. 2 — Excavations

SCOPE & DESCRIPTION

This JHA covers waterproofing and below-grade construction operations including excavation access, surface preparation, primer and membrane application (cold-applied and torch/hot-applied), seam sealing, drainage board installation, and backfill coordination. Work occurs adjacent to or within excavations, creating combined hazards of cave-in, solvent vapor exposure, thermal burns, falls, and struck-by incidents from overhead operations.

PROJECT INFORMATION

| Project Name: |  | Date: |  |
| --- | --- | --- | --- |
| Project Number: |  | Location: |  |
| Contractor: |  | Supervisor: |  |
| JHA Prepared By: |  | Reviewed By: |  |

JOB HAZARD ANALYSIS

| Task Step | Hazard | Risk Level (H/M/L) | Control Measures |
| --- | --- | --- | --- |
| 1. Excavation Access | Cave-in / wall collapse Falls into excavation Struck by falling objects from surface Hazardous atmosphere (gases in soil) | H | Competent person must inspect excavation daily and after rain Shoring, sloping, or shielding per Subpart P Ladder access within 25 ft of all workers Barricades and warning signs around excavation perimeter Atmospheric testing before entry (O2, LEL, H2S, CO) Hard hats mandatory for all workers near excavation |
| 2. Surface Preparation | Dust inhalation from grinding/cleaning Hand/arm vibration from power tools Struck by debris Proximity to excavation edge | M | Dust mask / P100 respirator for grinding operations Anti-vibration gloves; limit continuous tool use to 30 min Face shield and safety glasses for grinding Maintain 2 ft setback from unprotected excavation edge Secure tools to prevent dropping into excavation |
| 3. Primer / Membrane Application | Solvent vapor inhalation (high VOC primers) Skin/eye contact with bituminous products Slippery surfaces from adhesive overspray Torch/hot-applied membrane — burn hazard | H | Organic vapor respirator in below-grade applications Forced-air ventilation to maintain vapors below PEL Chemical-resistant gloves, long sleeves, splash goggles Non-slip footwear; clean overspray immediately Hot work permit for torch-applied membranes Fire extinguisher within 10 ft of torch operations Fire watch for 30 minutes after torch use |
| 4. Seam Sealing | Solvent/adhesive vapor exposure Burns from heat welding equipment Ergonomic strain (kneeling, bending) | M | Ventilation and respiratory protection as per primer application Heat-resistant gloves for heat-welded seams Knee pads and anti-fatigue mats for extended kneeling Rotate workers to minimize repetitive strain Inspect all seams per manufacturer quality requirements |
| 5. Drainage Board Installation | Manual handling of heavy boards/rolls Cuts from utility knives Falls from ladders/staging Struck by overhead debris during installation | M | Two-person lifts for heavy rolls; mechanical aids when possible Cut-resistant gloves; retractable utility knives Ladder safety: 3-point contact, secured at top Hard hat required; overhead protection barriers Coordinate with crane/overhead operations |
| 6. Backfill Coordination | Struck by heavy equipment Workers trapped between wall and equipment Excavation collapse during backfill Vibration damage to fresh membrane | H | Dedicated spotter for all backfill equipment operations No workers between wall and equipment — enforced exclusion zone Competent person verifies membrane cure before backfill Backfill in controlled lifts per geotechnical spec High-visibility vests mandatory for all ground personnel Communication protocol between equipment operator and crew |

PERSONAL PROTECTIVE EQUIPMENT (PPE)

| PPE Required |  |
| --- | --- |
| ☐ Hard Hat (ANSI Z89.1 Type I, Class E) | ☐ Organic Vapor Respirator / P100 Combination |
| ☐ Chemical-Resistant Gloves (nitrile or neoprene) | ☐ Heat-Resistant Gloves (torch operations) |
| ☐ Splash-Proof Safety Goggles | ☐ Steel-Toe Safety Boots (puncture-resistant sole) |
| ☐ High-Visibility Vest (Class 2 minimum) | ☐ Knee Pads (membrane application) |
| ☐ Face Shield (grinding operations) | ☐ Hearing Protection (equipment operations) |

EMERGENCY PROCEDURES

1. EXCAVATION COLLAPSE / CAVE-IN: Call 911 immediately. Do NOT enter the excavation to attempt rescue. Shut down all equipment. Initiate trench rescue protocol. Only trained trench rescue personnel may enter.

2. SOLVENT VAPOR OVEREXPOSURE: Remove worker to fresh air. Administer oxygen if available. If unconscious, place in recovery position. Call 911. Provide SDS to responders.

3. THERMAL BURN (TORCH/HOT MEMBRANE): Cool burn with clean, cool water for 10+ minutes. Do not remove adhered membrane from skin. Cover with sterile dressing. Seek immediate medical attention for burns larger than palm size.

4. FALL INTO EXCAVATION: Call 911. Do not move victim if spinal injury suspected. Lower rescue equipment, not personnel, unless trained. Keep victim warm and monitor vitals.

5. STRUCK BY EQUIPMENT/DEBRIS: Secure the area. Administer first aid. Call 911 for serious injuries. Preserve scene for incident investigation.

6. EMERGENCY CONTACTS: 911 for all emergencies. Site Safety Officer: ____________. Poison Control: 1-800-222-1222.

ADDITIONAL NOTES / SITE-SPECIFIC REQUIREMENTS

• Competent Person for excavation must be identified and on-site during all below-grade work.

• Daily excavation inspection logs must be maintained and available for review.

• Waterproofing manufacturer's application guidelines must be followed for warranty compliance.

• All torch/hot-work operations require an active Hot Work Permit.

• Coordination meetings with backfill crew must occur before any heavy equipment approaches the work zone.

WORKER SIGN-OFF

By signing below, I confirm that I have reviewed and understand this JHA, including all identified hazards and required control measures.

| # | Print Name | Signature | Date | Company/Trade |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |
| 9 |  |  |  |  |
| 10 |  |  |  |  |

REVIEW & APPROVAL

This JHA has been reviewed and approved by the following authorized personnel:

| Role | Signature | Date |
| --- | --- | --- |
| Prepared By (Safety Professional) |  |  |
| Reviewed By (Site Supervisor) |  |  |
| Approved By (Project Manager) |  |  |
| Client Representative (if required) |  |  |

REVISION HISTORY

| Revision | Date | Description | Approved By |
| --- | --- | --- | --- |
| 0 |  | Initial Release |  |
| 1 |  |  |  |

DISCLAIMER: This Job Hazard Analysis is a guideline for identifying hazards and implementing controls. It does not replace site-specific safety plans, competent person assessments, or regulatory requirements. All workers must comply with applicable OSHA standards and company safety policies. This JHA must be reviewed and updated whenever work conditions change.