FORBES SAFETY SOLUTIONS

JOB HAZARD ANALYSIS

| JHA INFORMATION |  |  |  |
| --- | --- | --- | --- |
| JHA Title: | Welding & Cutting Operations | JHA Number: | FSS-JHA-028 |
| Project Name: |  | Date: |  |
| Location: |  | Revision: | 0 |
| Regulatory References: | 29 CFR 1926.350-354 (Welding & Cutting); ANSI Z49.1 (Safety in Welding, Cutting, and Allied Processes); NFPA 51B (Fire Prevention During Welding) |  |  |
| Prepared By: |  | Approved By: |  |
| Competent Person: |  | Supervisor: |  |

JOB HAZARD ANALYSIS — TASK BREAKDOWN

| Task Step | Hazard | Risk Level (H/M/L) | Control Measures |
| --- | --- | --- | --- |
| 1. Hot Work Area Preparation | • Fire from combustible materials nearby • Explosion near flammable storage • Inadequate ventilation • Unauthorized area access | H | • Survey 35-foot radius for combustible/flammable materials • Remove combustibles or protect with fire-resistant blankets/shields • Verify no flammable vapors present — use combustible gas detector • Ensure adequate ventilation (natural or mechanical) • Wet down combustible floors/surfaces if they cannot be relocated • Barricade hot work area and post warning signs • Coordinate with other trades to eliminate ignition sources |
| 2. Equipment Inspection & Setup | • Gas leaks from regulators/hoses • Electrical shock from damaged cables • Cylinder tip-over • Flashback into cylinders | H | • Inspect all hoses, regulators, gauges, and connections for damage • Check welding leads and electrode holders for damaged insulation • Secure gas cylinders upright with chains/straps to a stable structure • Verify flashback arrestors installed on oxy-fuel equipment • Test all connections with soapy water for leaks — no open flame test • Confirm ground clamp securely attached to workpiece • Ensure fire extinguisher (minimum 20 lb ABC) within 20 feet |
| 3. Fire Watch Establishment | • Undetected fire ignition • Sparks reaching hidden combustibles • Delayed response to fire • Fire watch leaving post prematurely | H | • Assign trained fire watch with no other duties during hot work • Equip fire watch with appropriate extinguisher and communication device • Fire watch must have clear view of all areas where sparks may reach • Fire watch authorized to stop work if unsafe conditions develop • Maintain fire watch for minimum 30 minutes after hot work ceases • Brief fire watch on alarm procedures and evacuation routes • Document fire watch assignments on hot work permit |
| 4. Welding / Cutting Operations | • UV/IR radiation eye burns (arc eye) • Burns from molten metal/sparks • Toxic fume inhalation (manganese, hexavalent chromium, zinc) • Electrical shock • Noise exposure exceeding 85 dBA | H | • Wear proper shade lens (Shade 10-14 for arc; Shade 3-5 for oxy-fuel) • Use welding screens/curtains to protect nearby workers from UV • Position body to minimize fume inhalation — stay upwind • Use local exhaust ventilation or fume extraction at source • Wear dry insulated welding gloves — never touch electrode with bare hands • Keep body insulated from workpiece and ground • Monitor for fume fever symptoms (zinc) — seek medical attention if ill • Wear hearing protection in high-noise operations |
| 5. Post-Work Fire Watch (30 Min Minimum) | • Smoldering materials • Hidden fire behind walls/ceilings • Delayed ignition of combustibles | M | • Continue fire watch for minimum 30 minutes after completion • Inspect all areas where sparks and heat may have traveled • Check behind walls, below floors, and above ceilings for hot spots • Use thermal imaging camera if available for hidden heat detection • Do not leave area unattended until fire watch period complete • Document fire watch completion time on hot work permit |
| 6. Equipment Shutdown & Area Cleanup | • Residual heat on materials • Gas leaks from improperly closed valves • Trip hazards from leads/hoses • Improper cylinder storage | L | • Close cylinder valves, bleed lines, and release regulator pressure • Disconnect welding leads and coil neatly for storage • Allow hot materials to cool before handling — mark with "HOT" signs • Return cylinders to proper storage (oxygen separated from fuel by 20 ft or firewall) • Replace cylinder caps for transport/storage • Clean work area of slag, spatter, and electrode stubs • Close out hot work permit and file with safety records |

REQUIRED PERSONAL PROTECTIVE EQUIPMENT (PPE)

| PPE Required | Specification / ANSI Standard | Condition / Notes |
| --- | --- | --- |
| Welding Helmet (Auto-Darkening or Fixed Shade) | ANSI/ISEA Z87.1+; Shade 10-14 (arc) | Shade selected based on amperage/process |
| Safety Glasses (worn under helmet) | ANSI/ISEA Z87.1+ with side shields | Clear or shade 2 for under-helmet use |
| Flame-Resistant Welding Jacket/Sleeves | ANSI/ISEA 105; ASTM F2302 | Leather or FR cotton — no synthetic materials |
| Welding Gloves (Insulated Leather) | ANSI/ISEA 105; EN 12477 | Gauntlet style; dry at all times |
| Steel-Toe Safety Boots (Metatarsal Guard) | ASTM F2413 (I/75 C/75 Mt/75) | Leather uppers; no lace-up eyelets exposed |
| Respiratory Protection | NIOSH 42 CFR Part 84; ANSI Z88.2 | P100 half-face minimum; SAR for confined areas |
| Hearing Protection | ANSI S3.19 / S12.6 (NRR ≥ 25 dB) | Required during grinding and high-amp welding |
| FR Skull Cap / Beanie | NFPA 2112 | Protect scalp from spark/spatter |

EMERGENCY PROCEDURES

Fire During Hot Work:

• Fire watch activates alarm and attempts extinguishment with provided extinguisher

• If fire exceeds extinguisher capability, evacuate area immediately

• Call 911 — provide location, type of fire, and materials involved

• Shut off gas cylinders if safe to do so

• Activate site fire alarm and evacuate to assembly point

• Account for all personnel at assembly area

Burn Injury:

• Remove victim from heat source to safe area

• Cool burns immediately with clean, cool water (20 minutes minimum)

• Remove clothing/jewelry near burn (do not remove if adhered to skin)

• Cover with sterile, non-adherent dressing

• Call 911 for second-degree burns larger than palm size, or any third-degree burns

• Do not apply butter, ointment, or ice to burns

Fume Inhalation / Respiratory Distress:

• Move victim to fresh air immediately

• Call 911 if victim has difficulty breathing, dizziness, or confusion

• Administer supplemental oxygen if trained and available

• Monitor for metal fume fever symptoms (flu-like onset 4-12 hours post-exposure)

• Provide EMS with welding rod/base metal SDS information

Emergency Contact Information:

• Emergency Services: 911

• Site Safety Manager: [Insert Name & Phone]

• Nearest Hospital: [Insert Name, Address & Phone]

• Poison Control: 1-800-222-1222

• Fire Department Non-Emergency: [Insert Local Number]

WORKER SIGN-OFF — I HAVE READ AND UNDERSTAND THIS JHA

| # | Print Name | Signature | Company / Trade | Date |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |
| 9 |  |  |  |  |
| 10 |  |  |  |  |

REVIEW & APPROVAL

| Role | Name / Signature | Date | Initials |
| --- | --- | --- | --- |
| Prepared By (Author) |  |  |  |
| Reviewed By (Safety Manager) |  |  |  |
| Approved By (Project Manager) |  |  |  |
| Client / Owner Rep |  |  |  |

This JHA must be reviewed and updated whenever job conditions change, after any incident, or at minimum annually.