FORBES SAFETY SOLUTIONS

Asbestos Awareness Program

Identification, Protection & Compliance

| Company: | [COMPANY NAME] |
| --- | --- |
| Prepared By: | [NAME / TITLE] |
| Effective Date: | [DATE] |
| Revision: | [REV NUMBER] |
| Regulatory Ref: | 29 CFR 1926.1101 / 29 CFR 1910.1001 |

This document is the property of [COMPANY NAME]. Unauthorized reproduction or distribution is prohibited.

# Table of Contents

# 1. Purpose & Scope

[COMPANY NAME] is committed to protecting employees from the hazards associated with asbestos exposure. This program establishes procedures for identifying asbestos-containing materials (ACM), minimizing exposure, and ensuring compliance with OSHA standards for both general industry and construction.

This program applies to all employees, contractors, and visitors at [COMPANY NAME] worksites where asbestos-containing materials may be present or disturbed.

# 2. Regulatory References

- 29 CFR 1926.1101 — Asbestos (Construction Industry)

- 29 CFR 1910.1001 — Asbestos (General Industry)

- 40 CFR 61 Subpart M — National Emission Standards for Asbestos

- EPA AHERA — Asbestos Hazard Emergency Response Act

- OSHA Fact Sheet: Asbestos

# 3. Definitions

## 3.1 Types of Asbestos

- Chrysotile (White Asbestos): Most commonly used form (~95% of asbestos in buildings). Serpentine fiber with curly structure.

- Amosite (Brown Asbestos): Amphibole fiber commonly found in insulation, ceiling tiles, and thermal products.

- Crocidolite (Blue Asbestos): Most hazardous amphibole fiber, primarily found in pipe insulation and spray-on coatings.

## 3.2 Common Locations in Construction

- Thermal system insulation (pipes, boilers, ducts)

- Floor tiles and mastic/adhesive

- Roofing materials (felts, shingles)

- Siding and transite panels

- Pipe wrap and lagging

- Fireproofing and spray-applied materials

- Joint compound and texture coatings

- Gaskets and packing materials

## 3.3 Exposure Classifications

| Class | Description | Activities |
| --- | --- | --- |
| Class I | Highest risk | Removal of thermal system insulation or surfacing ACM |
| Class II | High risk | Removal of other ACM (flooring, siding, roofing, gaskets) |
| Class III | Moderate risk | Repair and maintenance of ACM where ACM may be disturbed |
| Class IV | Lower risk | Custodial activities in areas with ACM (cleanup of ACM debris) |

- Maintenance Activities: Any work that disturbs or may disturb ACM. Maintenance workers must receive Class III training at minimum.

# 4. Permissible Exposure Limits

OSHA has established the following exposure limits for airborne asbestos fibers:

| Limit Type | Concentration | Averaging Period |
| --- | --- | --- |
| PEL (Permissible Exposure Limit) | 0.1 f/cc | 8-hour TWA |
| Excursion Limit | 1.0 f/cc | 30-minute average |

NOTE: There is no safe level of asbestos exposure. All exposures shall be reduced to the lowest feasible level regardless of the PEL.

# 5. Roles & Responsibilities

## 5.1 Management

- Provide resources for asbestos identification, abatement, and training

- Ensure compliance with federal, state, and local asbestos regulations

- Designate a competent person for asbestos-related work

- Maintain records of all asbestos surveys, monitoring, and training

## 5.2 Competent Person

A designated competent person must be present on-site during any Class I, II, III, or IV asbestos work. The competent person shall:

- Be trained in asbestos identification, hazards, and control measures

- Have the authority to take prompt corrective action

- Conduct frequent and regular inspections of the work area

- Ensure proper engineering controls, work practices, and PPE are in use

- For Class I/II work: complete EPA-accredited training (Contractor/Supervisor course)

## 5.3 Supervisors

- Ensure work does not disturb suspect or known ACM without proper protocols

- Stop work if unexpected ACM is encountered

- Enforce regulated area boundaries and access controls

## 5.4 Employees

- Attend all required asbestos awareness or operations training

- Do not disturb suspected asbestos-containing materials

- Report any damaged or deteriorating materials that may contain asbestos

- Use all required PPE and follow decontamination procedures

## 5.5 Safety Department

- Administer and coordinate asbestos program activities

- Arrange for exposure monitoring and building surveys

- Maintain training records, monitoring data, and medical surveillance files

# 6. Exposure Monitoring

## 6.1 Initial Monitoring

[COMPANY NAME] shall conduct initial exposure monitoring for all employees who may be exposed to airborne asbestos at or above the PEL. Monitoring shall be representative of the 8-hour TWA and 30-minute short-term exposure.

## 6.2 Periodic Monitoring

- Required at least every 6 months for employees exposed at or above the PEL or excursion limit

- May be reduced to every 6 months if results are below action level (0.1 f/cc)

- Monitoring may be discontinued if two consecutive samples taken 7 days apart are below the action level

## 6.3 Additional Monitoring

Additional monitoring shall be performed whenever changes in processes, controls, or personnel may result in new or increased exposures.

## 6.4 Employee Notification

Employees shall be notified of monitoring results within 15 working days of receipt. Notification must be in writing and include the corrective action being taken if the PEL is exceeded.

# 7. Regulated Areas

Regulated areas shall be established wherever airborne asbestos concentrations exceed or are expected to exceed the PEL. Requirements include:

- Demarcation with warning signs and barrier tape

- Access limited to authorized, trained, and properly equipped personnel

- Warning signs displaying: "DANGER — ASBESTOS — CANCER AND LUNG DISEASE HAZARD — AUTHORIZED PERSONNEL ONLY"

- No eating, drinking, smoking, or applying cosmetics in regulated areas

# 8. Engineering & Work Practice Controls

## 8.1 Engineering Controls

- Wet methods — keep ACM adequately wet to suppress fiber release

- Local exhaust ventilation with HEPA filtration

- Isolation and enclosure of work areas (negative pressure enclosures for Class I)

- Glove bags for small-scale pipe insulation removal

- HEPA-filtered vacuum equipment for cleanup

## 8.2 Prohibited Practices

- High-speed abrasive disc saws without HEPA exhaust

- Dry sweeping or shoveling of asbestos debris

- Compressed air to remove asbestos from surfaces

- Employee rotation as a means to reduce exposure

# 9. Respiratory Protection

When engineering controls alone cannot reduce exposure below the PEL, respiratory protection shall be provided in accordance with 29 CFR 1910.134 and the [COMPANY NAME] Respiratory Protection Program.

| Exposure Level | Minimum Respirator |
| --- | --- |
| Up to 1.0 f/cc | Half-face APR with P100 filters |
| Up to 10 f/cc | Full-face APR with P100 filters |
| Above 10 f/cc or unknown | PAPR with HEPA filters or supplied air respirator |
| IDLH or emergency | Full-face pressure-demand SCBA or combination SCBA/SAR |

All respirator users must be medically evaluated, fit tested annually, and trained in proper use, maintenance, and storage.

# 10. Medical Surveillance

Medical surveillance shall be provided at no cost to employees who are or will be exposed to airborne asbestos at or above the PEL or excursion limit for 30 or more days per year. The program includes:

- Pre-placement examination (medical/work history, chest X-ray, pulmonary function test)

- Annual examination (updated history, chest X-ray as recommended, pulmonary function test)

- Termination examination (within 30 days of last day of employment if not examined within the previous year)

- Physician's written opinion provided to employer and employee

# 11. Decontamination Procedures

## 11.1 Personnel Decontamination

For Class I and II work, a decontamination unit shall be established adjacent to the regulated area with:

- Clean room (for donning clean clothing and PPE)

- Shower room (with water to remove fibers from skin)

- Equipment room (for removal of contaminated PPE)

Workers shall shower before leaving the regulated area. Disposable clothing shall be bagged wet for disposal.

## 11.2 Equipment Decontamination

- All tools and equipment shall be HEPA-vacuumed or wet-wiped before removal from regulated area

- Reusable PPE shall be decontaminated after each use

# 12. Disposal Requirements

All asbestos waste shall be handled as regulated waste:

- Double-bagged in 6-mil polyethylene bags or sealed in leak-tight containers

- Labeled with asbestos warning labels per OSHA and DOT requirements

- Transported by licensed asbestos waste hauler

- Disposed of at permitted landfill in accordance with EPA 40 CFR 61 Subpart M

- Waste manifests retained for minimum of 30 years

# 13. Training Requirements

## 13.1 Asbestos Awareness Training (All Employees)

Employees who may work in areas where ACM is present but will not disturb it shall receive awareness training covering:

- Health effects of asbestos exposure

- Locations of ACM in the workplace

- Recognition of damage or deterioration

- Proper response to fiber release episodes

## 13.2 Training by Exposure Class

| Class | Training Hours | Content Requirements |
| --- | --- | --- |
| Class I | 32 hours + 8 hr annual | EPA-accredited contractor/supervisor course; all hazards and controls |
| Class II | 8 hours minimum | Hazard identification, controls, PPE for specific ACM type |
| Class III | 16 hours minimum | Asbestos hazards, O&M procedures, work practice controls |
| Class IV | 2 hours minimum | Awareness training plus cleanup procedures |

## 13.3 Refresher Training

Annual refresher training is required for all classes. Training records shall be maintained for the duration of employment plus 30 years.

# 14. Recordkeeping

- Exposure monitoring records — retained for 30 years per 29 CFR 1910.1020

- Medical surveillance records — retained for duration of employment plus 30 years

- Training records — retained for duration of employment plus 30 years

- Building asbestos surveys — retained for the life of the building

- Abatement project records — retained for 30 years

- Waste disposal manifests — retained for 30 years

# 15. Program Review

This program shall be reviewed annually or whenever there is a change in processes, regulations, or after any asbestos-related incident. Reviews shall be documented and approved by the Safety Department.

# Appendix A: Asbestos Identification Checklist

Use this checklist to identify potential ACM during building surveys and pre-renovation inspections.

| Item | Location | Material Type | Condition | Presumed ACM? | Sample Taken? |
| --- | --- | --- | --- | --- | --- |
|  |  |  | ☐ Good ☐ Fair ☐ Poor | ☐ Yes ☐ No | ☐ Yes ☐ No |
|  |  |  | ☐ Good ☐ Fair ☐ Poor | ☐ Yes ☐ No | ☐ Yes ☐ No |
|  |  |  | ☐ Good ☐ Fair ☐ Poor | ☐ Yes ☐ No | ☐ Yes ☐ No |
|  |  |  | ☐ Good ☐ Fair ☐ Poor | ☐ Yes ☐ No | ☐ Yes ☐ No |
|  |  |  | ☐ Good ☐ Fair ☐ Poor | ☐ Yes ☐ No | ☐ Yes ☐ No |
|  |  |  | ☐ Good ☐ Fair ☐ Poor | ☐ Yes ☐ No | ☐ Yes ☐ No |
|  |  |  | ☐ Good ☐ Fair ☐ Poor | ☐ Yes ☐ No | ☐ Yes ☐ No |
|  |  |  | ☐ Good ☐ Fair ☐ Poor | ☐ Yes ☐ No | ☐ Yes ☐ No |

Surveyed By: ______________________________     Date: ____________

Building / Location: ______________________________

Notes: ________________________________________________________________________

# Appendix B: Worker Notification Form

This form must be completed and provided to employees within 15 working days of receiving exposure monitoring results.

Date of Notification: ____________________

Employee Name: ______________________________     Employee ID: ____________

Job Title: ______________________________

Work Location / Area: ______________________________

Monitoring Results:

Date(s) of Monitoring: ____________________

8-Hour TWA Result: ____________ f/cc

30-Minute Excursion Result: ____________ f/cc

| Result | Check |
| --- | --- |
| Below Action Level (0.1 f/cc) | ☐ |
| At or Above PEL (0.1 f/cc TWA) | ☐ |
| Above Excursion Limit (1.0 f/cc / 30 min) | ☐ |

Corrective Actions Taken (if above PEL):

________________________________________________________________________

________________________________________________________________________

Supervisor Signature: ______________________________     Date: ____________

Employee Signature: ______________________________     Date: ____________