FORBES SAFETY SOLUTIONS

Spill Prevention & Response Plan

Prevention, Containment, Response & Reporting

| Company: | [COMPANY NAME] |
| --- | --- |
| Prepared By: | [NAME / TITLE] |
| Effective Date: | [DATE] |
| Revision: | [REV NUMBER] |
| Regulatory Ref: | 40 CFR 112 (SPCC) / 29 CFR 1910.120 / EPA |

This document is the property of [COMPANY NAME]. Unauthorized reproduction or distribution is prohibited.

# Table of Contents

# 1. Purpose & Scope

[COMPANY NAME] is committed to preventing spills of hazardous materials, petroleum products, and chemicals, and to ensuring a rapid and effective response when spills do occur. This plan establishes procedures for spill prevention, containment, response, cleanup, reporting, and post-incident investigation.

This plan applies to all [COMPANY NAME] facilities, operations, and employees, as well as contractors and visitors who may handle, store, or transport regulated materials.

# 2. Regulatory References

- 40 CFR 112 — Spill Prevention, Control, and Countermeasure (SPCC) Rule

- 29 CFR 1910.120 — Hazardous Waste Operations and Emergency Response (HAZWOPER)

- 40 CFR 302 — Designation, Reportable Quantities, and Notification (CERCLA)

- 40 CFR 355 — Emergency Planning and Notification (EPCRA)

- Clean Water Act — Section 311

- State and local environmental regulations (facility-specific)

# 3. Definitions

- Spill: Any unplanned or uncontrolled release of oil, petroleum, chemical, or hazardous substance.

- Reportable Quantity (RQ): The threshold amount of a hazardous substance that triggers federal reporting when released.

- Secondary Containment: A system designed to capture spills from primary storage (berms, dikes, double-walled tanks).

- SPCC Plan: Spill Prevention, Control, and Countermeasure Plan — required under 40 CFR 112 for facilities storing >1,320 gallons of oil above ground.

- NRC: National Response Center — federal agency receiving spill notifications (1-800-424-8802).

- SDS: Safety Data Sheet — document providing hazard information for chemicals.

# 4. Roles & Responsibilities

## 4.1 Management

- Provide resources for spill prevention and response equipment

- Ensure compliance with SPCC and HAZWOPER requirements

- Designate a Spill Response Coordinator

- Maintain relationships with environmental agencies and spill response contractors

## 4.2 Spill Response Coordinator

- Oversee spill prevention measures and equipment inspections

- Coordinate spill response activities and external notifications

- Manage chemical inventory and SDS library

- Conduct post-spill investigations and implement corrective actions

- Ensure spill kits are stocked and strategically located

## 4.3 Supervisors

- Ensure employees are trained in spill prevention and response

- Enforce proper chemical storage, handling, and transfer procedures

- Initiate spill response and notify the Spill Response Coordinator

## 4.4 Employees

- Follow all chemical handling, storage, and transfer procedures

- Report spills, leaks, and potential releases immediately

- Know the location of spill kits, SDS binder, and emergency equipment

- Participate in spill response only within their level of training

# 5. Spill Prevention Measures

## 5.1 Secondary Containment

- All bulk oil/chemical storage areas shall have secondary containment capable of holding 110% of the largest container or 10% of total volume, whichever is greater

- Containment berms, dikes, or double-walled tanks shall be inspected monthly

- Drain valves on containment systems shall remain closed at all times except during controlled drainage of rainwater

- Portable containment (drip pans, berms) for all transfer and dispensing operations

## 5.2 Proper Storage

- Segregate incompatible chemicals per SDS compatibility guidelines

- Label all containers with contents and hazard information

- Keep containers closed when not actively dispensing

- Store containers on stable, level surfaces away from traffic areas

- Protect outdoor storage from weather, temperature extremes, and stormwater runoff

## 5.3 Transfer Procedures

- Use grounding and bonding for flammable liquid transfers

- Attend all transfer operations — never leave transfer unattended

- Use drip pans under all connection and disconnection points

- Verify container integrity before and after transfer

- Prevent overfills — tanks shall not be filled beyond 95% capacity

## 5.4 Inspections

- Monthly inspection of storage areas, secondary containment, and containers

- Daily visual checks of active storage and transfer areas

- Annual integrity testing for bulk storage tanks (per SPCC)

- Document all inspections and corrective actions

# 6. Chemical Inventory & SDS Location

[COMPANY NAME] shall maintain a current chemical inventory and Safety Data Sheet (SDS) library:

- Chemical inventory updated at least quarterly and whenever new chemicals are introduced

- SDS library maintained in [LOCATION — e.g., break room, safety office, electronic system]

- SDS available 24/7 to all employees

- Chemical inventory includes: product name, manufacturer, quantity on-site, storage location, and reportable quantity

- A copy of the chemical inventory is included as Appendix B

# 7. Spill Response Procedures

## 7.1 Immediate Actions (All Spills)

Upon discovering any spill:

- Ensure personal safety first — do not enter spill area without proper PPE

- Identify the material if possible (check labels, SDS, placards)

- If safe, stop the source of the spill (close valve, upright container, plug leak)

- Warn others in the immediate area

- Notify supervisor and Spill Response Coordinator immediately

- Begin containment efforts if trained and equipped (deploy absorbents, berms)

- Prevent spill from reaching drains, waterways, or storm sewers

## 7.2 Response by Material Type

### 7.2.1 Petroleum Spills (Oil, Diesel, Fuel, Hydraulic Fluid)

- Deploy oil-only absorbent pads, socks, or booms

- Dam or block drains and waterways

- Do NOT use water to flush petroleum spills

- Collect all contaminated absorbents in labeled waste containers

### 7.2.2 Chemical Spills (Acids, Bases, Solvents)

- Consult SDS for specific response guidance and PPE requirements

- Use chemical-specific or universal absorbents

- For acids: neutralizing absorbents may be used if trained

- For reactive chemicals: evacuate area and call hazmat response

- Ensure adequate ventilation — avoid inhaling vapors

### 7.2.3 Unknown Material Spills

- Do NOT attempt to identify unknown substances by smell or taste

- Evacuate the immediate area and establish a perimeter

- Notify the Spill Response Coordinator and emergency services

- Wait for hazmat-trained responders to identify and manage the spill

NOTE: Employees shall only respond to spills within their training level. HAZWOPER-trained responders handle spills exceeding incidental release levels.

# 8. Spill Kit Contents & Locations

## 8.1 Standard Spill Kit Contents

| Item | Quantity (per kit) |
| --- | --- |
| Oil-only absorbent pads (17" x 19") | 50 |
| Universal absorbent pads | 25 |
| Absorbent socks (3" x 48") | 8 |
| Absorbent boom (5" x 10') | 2 |
| Drain covers / mats | 2 |
| Nitrile gloves (pairs) | 4 |
| Chemical splash goggles | 2 |
| Disposal bags (6-mil) with ties | 6 |
| Caution tape (200 ft roll) | 1 |
| Instruction card / response guide | 1 |

## 8.2 Spill Kit Locations

[COMPANY NAME] shall place spill kits at the following locations (customize per facility):

- [Loading dock / receiving area]

- [Bulk chemical storage area]

- [Fuel dispensing area]

- [Maintenance shop]

- [Process area / production floor]

- [Vehicle / equipment staging area]

Spill kit locations shall be clearly marked and accessible. Kits shall be inspected monthly and restocked after each use.

# 9. Reporting Requirements

## 9.1 Internal Reporting

- ALL spills, regardless of size, shall be reported to the supervisor and Spill Response Coordinator immediately

- Complete the Spill Report Form (Appendix A) within 24 hours

## 9.2 External Reporting

[COMPANY NAME] shall report spills to regulatory agencies when reporting thresholds are met:

| Agency | Reporting Trigger | Contact / Timeframe |
| --- | --- | --- |
| National Response Center (NRC) | Any release to navigable waters; release of RQ of hazardous substance | 1-800-424-8802 — Immediate |
| State EPA / Environmental Agency | Per state-specific thresholds | [STATE AGENCY] — Within 24 hours |
| Local Emergency Planning Committee | Release of extremely hazardous substance ≥ RQ | Immediate + written within 30 days |
| OSHA | Fatality or hospitalization related to spill | 1-800-321-OSHA — 8 hours (fatality) / 24 hours (hospitalization) |

NOTE: When in doubt about whether a spill is reportable, report it. Under-reporting carries significant regulatory penalties.

# 10. Cleanup Procedures

- Use appropriate absorbents for the material type

- Work from the outside of the spill inward to prevent spreading

- Collect all contaminated absorbents, soil, and debris

- Place waste in labeled, sealed containers (6-mil bags or drums)

- Do not wash spilled materials into drains, ditches, or waterways

- Verify cleanup is complete through visual inspection and/or sampling

- Decontaminate all tools and PPE used during response

# 11. Waste Disposal

All spill waste shall be managed as follows:

- Characterize waste (hazardous vs. non-hazardous) based on the material spilled

- Label all waste containers with contents, date, and generator information

- Store waste in designated accumulation area per EPA regulations

- Dispose through licensed waste hauler with proper manifests

- Retain waste manifests and disposal records for minimum 3 years (hazardous waste: per RCRA)

# 12. Decontamination

- Personnel: remove contaminated clothing, wash affected skin with soap and water

- Equipment: clean with appropriate decontamination solution per SDS guidance

- Vehicles: prevent cross-contamination — wash before leaving the spill area

- Reusable PPE: decontaminate per manufacturer instructions or dispose

# 13. Post-Spill Investigation

A thorough investigation shall be conducted after every spill to prevent recurrence:

- Root cause analysis — determine why the spill occurred

- Review effectiveness of response actions

- Identify corrective actions (equipment, procedures, training)

- Document findings in the Spill Report Form (Appendix A)

- Update prevention measures as needed

- Share lessons learned with all affected personnel

# 14. Notification Chain

## 14.1 Internal Notifications

| Contact | Role | Phone |
| --- | --- | --- |
| [NAME] | Spill Response Coordinator | [PHONE] |
| [NAME] | Site Manager | [PHONE] |
| [NAME] | Safety Department | [PHONE] |
| [NAME] | Environmental Compliance | [PHONE] |
| [NAME] | Company Management | [PHONE] |

## 14.2 External Notifications

| Agency | Phone |
| --- | --- |
| Emergency Services | 911 |
| National Response Center (NRC) | 1-800-424-8802 |
| State Environmental Agency | [STATE NUMBER] |
| Local Emergency Planning Committee | [LEPC NUMBER] |
| Poison Control Center | 1-800-222-1222 |
| OSHA | 1-800-321-OSHA (6742) |
| Spill Response Contractor | [CONTRACTOR NUMBER] |

# 15. Training Requirements

## 15.1 Awareness Level (All Employees)

- Spill prevention measures and good housekeeping

- Spill reporting procedures

- Spill kit locations and basic use

- SDS location and how to read an SDS

## 15.2 Operations Level (Spill Response Team)

- 24-hour HAZWOPER initial training (or 8-hour refresher annually)

- Material-specific response procedures

- PPE selection and use for spill response

- Containment, cleanup, and decontamination techniques

## 15.3 Refresher Training

Annual refresher training for all employees. HAZWOPER refresher (8 hours) for spill response team members.

# 16. Recordkeeping

- Spill report forms — retained for 5 years

- SPCC plan and amendments — retained for 5 years after replacement

- Chemical inventory — retained for 3 years

- Inspection records — retained for 3 years

- Training records — retained for duration of employment plus 3 years

- Waste disposal manifests — retained per RCRA (3 years minimum; hazardous waste longer)

- External agency notifications — retained for 5 years

# 17. Program Review

This plan shall be reviewed annually, after any reportable spill, or whenever facility operations change significantly. Reviews shall be documented and the plan updated as necessary.

# Appendix A: Spill Report Form

Complete within 24 hours of any spill or release incident.

SECTION 1 — INCIDENT INFORMATION

Date of Spill: ____________     Time: ____________

Location / Area: ______________________________

Reported By: ______________________________     Phone: ____________

Weather Conditions: ______________________________

SECTION 2 — MATERIAL INFORMATION

Material Spilled: ______________________________

Quantity Released (estimated): ____________ gallons / lbs / other: ______

Source of Spill: ______________________________

Cause: ☐ Equipment failure  ☐ Human error  ☐ Overfill  ☐ Container breach  ☐ Other: ____________

SECTION 3 — IMPACT

Did material reach: ☐ Soil  ☐ Water (surface)  ☐ Storm drain  ☐ Sanitary sewer  ☐ Contained

Injuries? ☐ Yes  ☐ No     If yes, describe: ______________________________

Estimated area affected: ______________________________

SECTION 4 — RESPONSE ACTIONS

Immediate actions taken: ________________________________________________________________________

Cleanup methods: ________________________________________________________________________

Waste generated and disposal method: ________________________________________________________________________

SECTION 5 — NOTIFICATIONS

| Agency / Person | Notified? | Date / Time | Contact Name |
| --- | --- | --- | --- |
| Supervisor | ☐ Yes ☐ No |  |  |
| Spill Response Coordinator | ☐ Yes ☐ No |  |  |
| National Response Center | ☐ Yes ☐ No ☐ N/A |  |  |
| State EPA | ☐ Yes ☐ No ☐ N/A |  |  |
| LEPC | ☐ Yes ☐ No ☐ N/A |  |  |

SECTION 6 — ROOT CAUSE & CORRECTIVE ACTIONS

Root cause: ________________________________________________________________________

Corrective actions: ________________________________________________________________________

Responsible person: ______________________________     Due date: ____________

Completed By: ______________________________     Date: ____________

Reviewed By: ______________________________     Date: ____________

# Appendix B: Chemical Inventory Template

Maintain a current inventory of all chemicals stored at [COMPANY NAME] facilities.

Facility / Location: ______________________________     Date Updated: ____________

Prepared By: ______________________________

| Product Name | Manufacturer | Max Qty On-Site | Storage Location | SDS On File? | Reportable Qty |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | ☐ Yes |  |
|  |  |  |  | ☐ Yes |  |
|  |  |  |  | ☐ Yes |  |
|  |  |  |  | ☐ Yes |  |
|  |  |  |  | ☐ Yes |  |
|  |  |  |  | ☐ Yes |  |
|  |  |  |  | ☐ Yes |  |
|  |  |  |  | ☐ Yes |  |
|  |  |  |  | ☐ Yes |  |
|  |  |  |  | ☐ Yes |  |
|  |  |  |  | ☐ Yes |  |
|  |  |  |  | ☐ Yes |  |

SDS Binder Location: ______________________________

Electronic SDS System: ☐ Yes  ☐ No     System Name: ______________________________

Notes: ________________________________________________________________________