FORBES SAFETY SOLUTIONS

JOB HAZARD ANALYSIS

| JHA INFORMATION |  |  |  |
| --- | --- | --- | --- |
| JHA Title: | Confined Space Entry | JHA Number: | FSS-JHA-027 |
| Project Name: |  | Date: |  |
| Location: |  | Revision: | 0 |
| Regulatory References: | 29 CFR 1910.146 (Permit-Required Confined Spaces); 29 CFR 1926 Subpart AA (Confined Spaces in Construction) |  |  |
| Prepared By: |  | Approved By: |  |
| Competent Person: |  | Supervisor: |  |

JOB HAZARD ANALYSIS — TASK BREAKDOWN

| Task Step | Hazard | Risk Level (H/M/L) | Control Measures |
| --- | --- | --- | --- |
| 1. Space Identification & Hazard Assessment | • Unidentified permit-required conditions • Unknown atmospheric hazards • Engulfment potential • Energy sources not identified | H | • Survey and inventory all confined spaces on site • Classify each space as permit-required or non-permit per 1910.146 • Post warning signs at all confined space entries ("DANGER — Confined Space") • Review historical data, SDS, and process info for each space • Identify all energy sources requiring lockout/tagout • Conduct pre-entry hazard assessment with competent person |
| 2. Atmospheric Testing | • Oxygen deficiency (<19.5%) • Oxygen enrichment (>23.5%) • Toxic gas exposure (H₂S, CO) • Flammable atmosphere (>10% LEL) | H | • Calibrate multi-gas detector before each use per manufacturer specs • Test atmosphere before entry in this order: O₂, combustibles, toxics • Test at multiple levels (top, middle, bottom) of space • Continuous monitoring required during all entry operations • Establish action levels: O₂ 19.5-23.5%, LEL <10%, H₂S <10ppm, CO <25ppm • Document all readings on entry permit • Evacuate immediately if readings exceed action levels |
| 3. Ventilation Setup | • Inadequate air circulation • Introduction of contaminants via ventilation • Recirculation of hazardous atmosphere • Noise exposure from blowers | M | • Position blower intake in clean air source — away from exhaust/contaminants • Direct forced air to lowest point of space for heavier-than-air gases • Allow adequate ventilation time before entry (minimum 15 minutes) • Re-test atmosphere after ventilation before allowing entry • Maintain continuous mechanical ventilation during all entry operations • Use intrinsically safe ventilation equipment in flammable atmospheres |
| 4. Entry Permit Completion | • Incomplete hazard communication • Missing authorization • Inadequate rescue planning • Untrained personnel entry | M | • Complete all sections of confined space entry permit • Verify atmospheric test results recorded on permit • Confirm rescue plan and rescue team availability • List all authorized entrants, attendants, and entry supervisor • Verify all entrants have current confined space training • Entry supervisor signs permit — post at entry point • Permit valid for one shift only; renew for subsequent shifts |
| 5. Entry & Work Operations | • Atmospheric changes during work • Engulfment by stored materials • Entrapment in converging geometry • Electrical/mechanical energy contact | H | • Attendant stationed at entry point at all times — no exceptions • Maintain continuous communication (visual, voice, radio, or hardwired) • Entrants wear retrieval harness with lifeline attached to tripod/davit • Continuous atmospheric monitoring with audible/visual alarms • All energy sources locked out/tagged out before entry • Limit number of entrants to essential personnel only • Entrants authorized to self-evacuate at any time without question |
| 6. Exit & Permit Closeout | • Residual hazards left in space • Incomplete documentation • Failure to secure space after exit | L | • All entrants exit space and are accounted for • Remove all tools, equipment, and materials from space • Close and secure space entry point to prevent unauthorized entry • Entry supervisor cancels permit and documents closeout • Review any incidents or near-misses during entry • File completed permit with project safety records • Debrief crew on lessons learned |

REQUIRED PERSONAL PROTECTIVE EQUIPMENT (PPE)

| PPE Required | Specification / ANSI Standard | Condition / Notes |
| --- | --- | --- |
| Hard Hat | ANSI/ISEA Z89.1, Type I or II | Bump cap may be acceptable in low-clearance spaces |
| Safety Glasses / Goggles | ANSI/ISEA Z87.1+ | Splash-proof goggles if liquid hazards present |
| Full-Body Harness with Retrieval D-Ring | ANSI/ASSP Z359.1 | Dorsal D-ring for fall arrest; sternal for retrieval |
| Supplied Air Respirator (SAR) or SCBA | NIOSH 42 CFR Part 84; ANSI Z88.2 | Required when atmosphere cannot be made safe |
| Chemical-Resistant Gloves | ANSI/ISEA 105; EN 374 | Select based on chemical exposure present |
| Steel-Toe Safety Boots | ASTM F2413 (I/75 C/75) | Chemical-resistant soles if needed |
| High-Visibility Vest | ANSI/ISEA 107, Class 2 | Required for surface crew visibility |
| Hearing Protection | ANSI S3.19 / S12.6 (NRR ≥ 25 dB) | Required when ventilation noise exceeds 85 dBA |

EMERGENCY PROCEDURES

Atmospheric Emergency / Alarm:

• All entrants evacuate space IMMEDIATELY upon alarm activation

• Attendant initiates rescue procedures — does NOT enter the space

• Attendant calls 911 and activates on-site rescue team

• Account for all entrants by name — report to rescue team

• Provide rescue team with atmospheric data and space configuration

• On-site rescue team enters only with proper equipment and training

Entrant Down / Unresponsive:

• Attendant attempts non-entry rescue using retrieval system (tripod/davit)

• Call 911 and activate confined space rescue team

• NEVER enter the space to attempt rescue without proper equipment and backup

• If extracted, begin CPR/AED and treat for atmospheric exposure

• Provide EMS with SDS information on potential chemical exposures

Engulfment Emergency:

• Shut off all material flow sources immediately

• Call 911 — request confined space rescue with engulfment capability

• Do not attempt to pull victim from engulfment material

• Maintain communication with victim if conscious

• Provide rescue team with material type and estimated depth

Emergency Contact Information:

• Emergency Services: 911

• Confined Space Rescue Team: [Insert Name & Phone]

• Site Safety Manager: [Insert Name & Phone]

• Nearest Hospital: [Insert Name, Address & Phone]

• Poison Control: 1-800-222-1222

WORKER SIGN-OFF — I HAVE READ AND UNDERSTAND THIS JHA

| # | Print Name | Signature | Company / Trade | Date |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |
| 9 |  |  |  |  |
| 10 |  |  |  |  |

REVIEW & APPROVAL

| Role | Name / Signature | Date | Initials |
| --- | --- | --- | --- |
| Prepared By (Author) |  |  |  |
| Reviewed By (Safety Manager) |  |  |  |
| Approved By (Project Manager) |  |  |  |
| Client / Owner Rep |  |  |  |

This JHA must be reviewed and updated whenever job conditions change, after any incident, or at minimum annually.