FORBES SAFETY SOLUTIONS

JOB HAZARD ANALYSIS

Underground Vault Entry

Regulatory References: 29 CFR 1910.146, 29 CFR 1910.269(e)

Document No: FSS-JHA-036  |  Revision: 0  |  Date: _______________

| Project Name: |  | Date: |  |
| --- | --- | --- | --- |
| Location: |  | JHA Number: |  |
| Supervisor: |  | Permit Required: | ☐ Yes ☐ No |
| Prepared By: |  | Reviewed By: |  |

JOB HAZARD ANALYSIS TABLE

| Task Step | Hazard | Risk Level (H/M/L) | Control Measures |
| --- | --- | --- | --- |
| 1. Vault Identification & Access | Traffic near vault opening Unauthorized access Unknown vault condition | H M H | Set up traffic control (cones, signs, flaggers) per MUTCD Verify vault location against utility maps Barricade work area with minimum 10-ft perimeter Post confined space entry signage Remove vault cover using proper lifting techniques/tools |
| 2. Atmospheric Testing | Toxic atmosphere (H2S, CO) Oxygen deficiency/enrichment Explosive atmosphere (LEL) | H H H | Test atmosphere BEFORE entry at all levels (top, middle, bottom) Calibrate multi-gas detector per manufacturer specs O2: 19.5%-23.5%, LEL: <10%, CO: <25 ppm, H2S: <10 ppm Continuous atmospheric monitoring throughout entry Document all readings on confined space entry permit |
| 3. Ventilation Setup | Inadequate air supply Recirculation of contaminated air Noise from blower equipment | H M M | Position forced-air ventilation blower at vault opening Ensure fresh air intake is upwind and away from exhaust sources Supply ductwork to bottom of vault for positive ventilation Run ventilation minimum 15 minutes before entry Retest atmosphere after ventilation and before entry |
| 4. Entry & Work Operations | Falls during entry/exit Ergonomic strain (cramped space) Electrical contact Insects/vermin | H M H M | Use fixed ladder or approved entry/egress equipment Attendant stationed at vault opening at all times Entrant wears full-body harness connected to retrieval system Verify all circuits de-energized and LOTO applied before work Rotate workers to minimize time in cramped positions Inspect vault for insects/vermin before and during entry |
| 5. Cable/Equipment Work | Electrical contact with energized cables Pinch points Cuts from cable/tools Chemical exposure (cable insulation) | H M M L | Confirm zero-energy state on all cables before handling Use voltage-rated insulating gloves and tools Identify and mark ALL conductors in the vault Use proper cable-cutting tools; secure cable ends Wear chemical-resistant gloves if handling splicing compounds Maintain communication with surface attendant at all times |
| 6. Exit & Securing Vault | Falls during exit Leaving vault unsecured Tool/equipment left below | M M L | Inventory all tools and materials before exiting Exit using fixed ladder with three-point contact Replace vault cover securely; verify flush with grade Remove traffic control devices only after vault is secured Complete confined space permit closeout documentation Debrief crew on any issues encountered |

REQUIRED PERSONAL PROTECTIVE EQUIPMENT (PPE)

| ☐ Hard Hat (ANSI Z89.1 Type I, Class E) | ☐ Safety Glasses (ANSI Z87.1) |
| --- | --- |
| ☐ FR-Rated Clothing | ☐ Steel-Toe Boots (ASTM F2413) |
| ☐ Rubber Insulating Gloves (Class appropriate) | ☐ Full-Body Harness with Retrieval Line |
| ☐ Multi-Gas Detector (LEL, O2, CO, H2S) | ☐ High-Visibility Vest (ANSI/ISEA 107 Class 2) |
| ☐ Hearing Protection (if noise >85 dBA) | ☐ Respiratory Protection (SCBA/SAR if required) |

EMERGENCY PROCEDURES

1. If atmospheric alarm sounds, immediately evacuate the vault using the retrieval system. Do NOT re-enter until atmosphere is retested and verified safe.

2. In case of water intrusion or flooding, evacuate immediately. Notify supervisor and de-energize all electrical sources if safe to do so.

3. If a worker becomes unresponsive, activate the retrieval system from the surface. Do NOT enter the vault for rescue without proper equipment and training.

4. For electrical contact, de-energize the source if safe. Call 911 immediately. Begin CPR if the victim is unresponsive and not breathing.

5. Administer first aid for insect bites/stings. Seek medical attention for allergic reactions. Ensure EpiPen is available if known allergies exist.

6. Report all incidents to site supervisor immediately. Complete incident report within 24 hours.

| Emergency Contact: | 911 / Site-specific: _______________ |
| --- | --- |
| Assembly Point: |  |
| Nearest Hospital: |  |
| First Aid Kit Location: |  |

WORKER SIGN-OFF

By signing below, I acknowledge that I have reviewed and understand this Job Hazard Analysis, the associated hazards, and the required control measures.

| # | Print Name | Signature | Company | Date |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |
| 9 |  |  |  |  |
| 10 |  |  |  |  |

REVIEW & APPROVAL

This JHA has been reviewed and approved by the following authorized personnel:

| Role | Name / Signature | Date |
| --- | --- | --- |
| Prepared By (Safety Professional) |  |  |
| Reviewed By (Site Supervisor) |  |  |
| Approved By (Project Manager) |  |  |
| Client Representative (if required) |  |  |

| Next Review Date: |  |
| --- | --- |
| Revision Notes: |  |

FORBES SAFETY SOLUTIONS  |  Committed to Zero Incidents

This document is valid only when all sections are completed and signatures obtained.