FORBES SAFETY SOLUTIONS

Permit-Required Confined Space Entry Program

Enhanced/Comprehensive

Prepared for:

[COMPANY NAME]

Regulatory References:

29 CFR 1910.146 | 29 CFR 1926 Subpart AA

Effective Date: [DATE]

Review Date: [ANNUAL REVIEW DATE]

# Table of Contents

# 1. Purpose and Scope

This Permit-Required Confined Space Entry Program establishes comprehensive procedures for the safe identification, evaluation, and entry of permit-required confined spaces at [COMPANY NAME] worksites. This program applies to all employees, contractors, and subcontractors who may enter, supervise, or support confined space entry operations.

This enhanced program exceeds minimum regulatory requirements by incorporating best practices for atmospheric monitoring, rescue planning, contractor coordination, and annual program review to ensure the highest level of worker protection.

# 2. Regulatory References

- 29 CFR 1910.146 — Permit-Required Confined Spaces (General Industry)

- 29 CFR 1926 Subpart AA — Confined Spaces in Construction

- ANSI/ASSE Z117.1 — Safety Requirements for Confined Spaces

- NFPA 350 — Guide for Safe Confined Space Entry and Work

- OSHA Confined Space CPL Directive

# 3. Definitions

Confined Space:

A space that: (1) is large enough for an employee to enter and perform work; (2) has limited or restricted means of entry or exit; and (3) is not designed for continuous employee occupancy.

Permit-Required Confined Space (PRCS):

A confined space that has one or more of the following characteristics: (1) contains or has the potential to contain a hazardous atmosphere; (2) contains a material that has the potential to engulf an entrant; (3) has an internal configuration such that an entrant could be trapped or asphyxiated; or (4) contains any other recognized serious safety or health hazard.

Atmospheric Hazard:

An atmosphere that may expose employees to the risk of death, incapacitation, impairment of ability to self-rescue, injury, or acute illness from one or more of the following: flammable gas/vapor/mist >10% LEL, airborne combustible dust, oxygen <19.5% or >23.5%, atmospheric concentration of any substance exceeding its PEL.

Entry:

The action by which any part of an employee's body breaks the plane of an opening into a permit-required confined space.

Alternate Entry Procedures:

Entry procedures used when the employer can demonstrate that the only hazard in the space is a controlled atmospheric hazard that can be eliminated through continuous forced air ventilation alone.

# 4. Roles and Responsibilities

## 4.1 Employer / Program Administrator

- Develop, implement, and maintain this written confined space program

- Designate a Confined Space Program Administrator

- Ensure adequate resources for equipment, training, and rescue services

- Conduct annual program reviews and updates

- Maintain the confined space inventory

## 4.2 Entry Supervisor

- Verify all entry conditions and authorize entry by signing the permit

- Ensure all pre-entry testing is completed and documented

- Ensure rescue services are available and have been notified

- Cancel the entry permit and terminate entry when conditions warrant

- Remove unauthorized individuals from the area

- Ensure acceptable entry conditions are maintained throughout entry

## 4.3 Authorized Entrant

- Know the hazards and symptoms of exposure

- Use equipment properly (PPE, communication, monitoring)

- Communicate with attendant continuously

- Alert the attendant when warning signs or symptoms are recognized

- Exit the space immediately when ordered or when hazards are detected

## 4.4 Attendant

- Remain at the entry point at all times during entry operations

- Monitor entrants and conditions inside and outside the space

- Maintain an accurate count of all entrants in the space

- Order evacuation when conditions warrant

- Summon rescue services when needed — never enter the space for rescue

- Prevent unauthorized entry

## 4.5 Rescue Team

- Be trained and equipped for confined space rescue operations

- Practice rescue from representative spaces at least annually

- Be available within an appropriate response time

- Ensure at least one member is trained in first aid and CPR

# 5. Confined Space Identification and Inventory

A comprehensive survey shall be conducted of all worksite areas to identify confined spaces. Each identified space shall be:

- Documented in the Confined Space Inventory Log (Appendix B)

- Evaluated for permit-required characteristics

- Classified as permit-required or non-permit confined space

- Marked with appropriate warning signage: "DANGER — PERMIT-REQUIRED CONFINED SPACE — DO NOT ENTER"

- Re-evaluated whenever conditions change or at least annually

Common confined spaces include but are not limited to: manholes, vaults, tanks, vessels, silos, bins, hoppers, pits, pipelines, trenches, excavations, tunnels, and ductwork.

# 6. Hazard Assessment

Prior to any entry, a thorough hazard assessment shall be conducted for each permit-required confined space. The assessment shall evaluate:

- Atmospheric hazards (toxic, flammable, oxygen-deficient/enriched)

- Physical hazards (engulfment, entrapment, moving parts, energized equipment)

- Chemical hazards (residual materials, cleaning agents)

- Biological hazards (mold, bacteria, animal waste)

- Thermal hazards (extreme heat/cold)

- Noise hazards

- Configuration hazards (converging walls, sloped floors)

The hazard assessment shall be documented on the entry permit and reviewed with all entry personnel before entry begins.

# 7. Atmospheric Testing and Monitoring

## 7.1 Testing Requirements

Atmospheric testing shall be conducted before entry and continuously during entry operations. Testing shall be performed in the following order:

- Oxygen (O₂) — test first, as other sensors require adequate oxygen to function

- Combustible gases (LEL) — test second

- Toxic gases (H₂S, CO, and others as identified) — test third

Testing shall be conducted at multiple levels within the space (top, middle, bottom) to account for stratification of gases.

## 7.2 Acceptable Atmospheric Levels

| Parameter | Acceptable Level | Action Level | IDLH |
| --- | --- | --- | --- |
| Oxygen (O₂) | 19.5% – 23.5% | < 19.5% or > 23.5% | < 16% |
| LEL (Combustible Gas) | < 10% LEL | ≥ 10% LEL | ≥ 20% LEL |
| Hydrogen Sulfide (H₂S) | < 10 ppm | ≥ 10 ppm | 100 ppm |
| Carbon Monoxide (CO) | < 25 ppm | ≥ 25 ppm | 1,200 ppm |
| Sulfur Dioxide (SO₂) | < 2 ppm | ≥ 2 ppm | 100 ppm |
| Ammonia (NH₃) | < 25 ppm | ≥ 25 ppm | 300 ppm |

## 7.3 Instrument Calibration

All atmospheric monitoring instruments shall be calibrated according to manufacturer specifications and at minimum before each day of use with known concentration calibration gas. Calibration records shall be maintained.

# 8. Ventilation Requirements

Mechanical ventilation shall be used to control atmospheric hazards within permit-required confined spaces:

- Forced air ventilation shall supply clean, breathable air to the space

- Ventilation equipment shall be positioned to prevent recirculation of contaminated air

- Air supply intake shall be located in a clean air zone away from exhaust or contamination sources

- Ventilation shall operate continuously during entry when used as an atmospheric control measure

- Ventilation alone does not eliminate the need for atmospheric monitoring

Ventilation rate shall be sufficient to maintain atmospheric conditions within acceptable levels as defined in Section 7.2.

# 9. Entry Permit System

No employee shall enter a permit-required confined space without a valid, signed entry permit. The permit shall include:

- Space identification and location

- Purpose of entry and authorized duration

- Names of authorized entrants, attendants, and entry supervisor

- Hazards of the space and control measures

- Atmospheric monitoring results (pre-entry and continuous)

- Rescue and emergency procedures and contact information

- Communication procedures

- Equipment required (PPE, ventilation, lighting, barriers)

- Hot work permit number (if applicable)

- Lockout/tagout verification

Permits are valid for one shift only and shall be posted at the entry point. Completed permits shall be retained for at least one year for annual program review. A comprehensive entry permit form is provided in Appendix A.

# 10. Entry Procedures

## 10.1 Pre-Entry Checklist

- Conduct a pre-entry briefing with all entry team members

- Complete lockout/tagout/isolation of all energy sources

- Verify rescue services are available and have been notified

- Test the atmosphere — O₂, LEL, H₂S, CO (in that order, at multiple levels)

- Verify ventilation equipment is operational

- Ensure all required PPE and equipment is available and inspected

- Complete and sign the entry permit

- Post the permit at the entry point

- Establish communication between entrant(s) and attendant

- Verify barriers and signage are in place

## 10.2 During Entry

- Continuous atmospheric monitoring shall be maintained

- The attendant shall maintain communication with entrants at all times

- The attendant shall maintain an accurate count of entrants

- The entry supervisor shall periodically verify conditions remain acceptable

- If any atmospheric reading exceeds acceptable levels, all entrants shall immediately evacuate

## 10.3 Post-Entry

- All tools and equipment shall be removed from the space

- A head count of all entrants shall be verified

- The space shall be secured against unauthorized re-entry

- The entry permit shall be closed out with time and signature

- The completed permit shall be filed for record retention

# 11. Communication Procedures

Effective communication shall be established and maintained between the attendant and entrants throughout the entry operation. Acceptable communication methods include:

- Voice communication (direct or hands-free radio)

- Visual contact

- Hardwired intercom or telephone

- Rope signals (tug-line) — as a secondary backup only

Communication shall be tested before entry begins. If communication is lost at any time, entrants shall immediately evacuate the space.

# 12. Rescue and Emergency Services

## 12.1 Rescue Planning

Rescue planning shall be completed before any entry operation begins. The entry employer shall evaluate and select the appropriate rescue method:

- Self-rescue: entrant exits the space independently using retrieval systems

- Non-entry rescue: entrant is retrieved without another person entering the space (e.g., retrieval line and tripod)

- Entry rescue: trained rescuers enter the space to recover the entrant

## 12.2 On-Site Rescue Team

When on-site rescue is designated, the rescue team shall:

- Be trained in confined space rescue and first aid/CPR

- Practice making permit space rescues at least once every 12 months using representative simulated conditions

- Be equipped with appropriate PPE and rescue equipment

- Be notified prior to each entry and remain on standby

## 12.3 Off-Site Rescue Service

When relying on off-site rescue services, the employer shall:

- Evaluate the rescue service's ability to respond in a timely manner

- Document response time — rescue must be achievable within a timeframe appropriate to the identified hazards

- Provide the rescue service with access to the space for practice and planning

- Ensure the rescue service is informed of the hazards of each space

- Verify annually that the service is capable and willing to respond

## 12.4 Retrieval Systems

Each authorized entrant shall use a chest or full-body harness with a retrieval line attached to a mechanical retrieval device or fixed anchor point when entering vertical spaces deeper than 5 feet. Wristlets may be used as an alternative if the employer can demonstrate that a harness is infeasible or creates a greater hazard.

# 13. Alternate Entry Procedures (Reclassification)

A permit-required confined space may be reclassified as a non-permit confined space under the following conditions (per 29 CFR 1910.146(c)(7)):

- The space poses no actual or potential atmospheric hazards, OR

- All hazards within the space are eliminated without entry, OR

- The only hazard is a controlled atmospheric hazard that can be eliminated by continuous forced air ventilation alone (alternate entry per 1910.146(c)(5))

For alternate entry procedures:

- Continuous forced air ventilation shall be maintained throughout entry

- Atmospheric testing shall demonstrate the space is safe before entry

- A full entry permit is not required, but documentation of conditions and ventilation shall be maintained

- If hazards arise during entry, all entrants shall exit and the space reverts to permit-required status

# 14. Contractor Coordination

When contractors perform permit-required confined space entry at [COMPANY NAME] worksites:

## 14.1 Host Employer Responsibilities

- Inform contractors of permit-required confined spaces and their hazards

- Inform contractors of any precautions or procedures that the host has implemented for the protection of employees in or near the spaces

- Coordinate entry operations when both host and contractor employees will be working in or near the space

- Debrief the contractor at the conclusion of entry operations regarding any hazards encountered

## 14.2 Contractor Responsibilities

- Obtain all available information from the host employer about the confined space hazards

- Ensure contractor employees are properly trained and equipped

- Inform the host employer of the permit space program the contractor will follow

- Inform the host employer of any hazards encountered or created during entry

# 15. Training

All employees involved in confined space operations shall receive training before initial assignment and whenever there is a change in duties, a change in space conditions, or inadequacies in the program are identified. Training shall cover:

## 15.1 All Affected Employees

- Hazards of confined spaces and recognition of symptoms

- This written program and its requirements

- Prohibition against unauthorized entry

## 15.2 Authorized Entrants

- All items above plus:

- Proper use of atmospheric monitoring equipment

- Proper use of PPE and rescue equipment

- Communication procedures

- Emergency and evacuation procedures

## 15.3 Attendants

- Duties and responsibilities of attendants

- Emergency notification procedures

- Non-entry rescue techniques

- When to order evacuation

## 15.4 Entry Supervisors

- All entrant and attendant training

- Permit completion and authorization

- Hazard assessment and control evaluation

- Permit cancellation and termination of entry

## 15.5 Rescue Team Members

- All entrant training

- Confined space rescue techniques

- First aid and CPR (current certification)

- Annual practice rescue drills from representative spaces

# 16. Annual Program Review

[COMPANY NAME] shall review this confined space program at least annually, using the cancelled entry permits retained from the previous 12 months. The review shall evaluate:

- Whether the program adequately protects employees

- Any deficiencies identified during entry operations

- Incidents, near-misses, or rescue events

- Changes in confined space inventory

- Employee feedback and training effectiveness

- Equipment condition and adequacy

The review shall be documented and any revisions to the program shall be implemented promptly.

# 17. Recordkeeping

- Entry permits: retained for at least one year

- Training records: maintained for duration of employment

- Atmospheric monitoring calibration records: maintained for 3 years

- Confined space inventory: updated annually and when conditions change

- Annual program review documentation: retained for 5 years

- Rescue service evaluations: retained for 3 years

- Incident and near-miss reports: retained for 5 years

# Appendix A: Confined Space Entry Permit

(Comprehensive Fillable Form)

| Permit Number: |  |
| --- | --- |
| Date: |  |
| Space Location/ID: |  |
| Purpose of Entry: |  |
| Entry Supervisor: |  |
| Phone/Radio: |  |
| Estimated Entry Time: |  |
| Estimated Exit Time: |  |

## Authorized Personnel

| Role | Name | Signature |
| --- | --- | --- |
| Entrant #1 |  |  |
| Entrant #2 |  |  |
| Entrant #3 |  |  |
| Attendant #1 |  |  |
| Attendant #2 |  |  |
| Entry Supervisor |  |  |

## Hazard Assessment

| Hazard | Present? (Y/N) | Controls in Place |
| --- | --- | --- |
| Atmospheric (toxic/flammable) |  |  |
| Oxygen deficiency/enrichment |  |  |
| Engulfment |  |  |
| Entrapment |  |  |
| Mechanical/moving parts |  |  |
| Electrical |  |  |
| Thermal extremes |  |  |
| Other: |  |  |

## Pre-Entry Atmospheric Monitoring

| Parameter | Reading #1 (Top) | Reading #2 (Mid) | Reading #3 (Bottom) | Acceptable? |
| --- | --- | --- | --- | --- |
| O₂ (%) |  |  |  |  |
| LEL (%) |  |  |  |  |
| CO (ppm) |  |  |  |  |
| H₂S (ppm) |  |  |  |  |
| Other: |  |  |  |  |

Instrument Make/Model: ________________________  Serial #: ______________  Last Calibration: ______________

## Required Equipment Checklist

| Equipment | Available? (Y/N) | Equipment | Available? (Y/N) |
| --- | --- | --- | --- |
| Atmospheric monitor |  | Ventilation blower |  |
| Retrieval system/tripod |  | Communication equipment |  |
| Full-body harness |  | Fire extinguisher |  |
| SCBA/SAR |  | Barricades/signage |  |
| Lighting |  | GFCI for all electrical |  |

## Lockout/Tagout Verification

All energy sources isolated and locked out? ☐ Yes  ☐ No

LOTO verified by: ________________________  Time: ______________

## Rescue Provisions

☐ On-site rescue team on standby     ☐ Off-site rescue service notified

Rescue Service Contact: ________________________  Phone: ______________

Estimated Response Time: ______________

## Authorization

I have reviewed conditions and authorize entry into this confined space.

Entry Supervisor Signature: ________________________  Date/Time: ______________

## Permit Closure

All entrants have exited: ☐ Yes     Space secured: ☐ Yes

Closed by: ________________________  Date/Time: ______________

Conditions encountered / notes: ________________________________________

# Appendix B: Confined Space Inventory Log

Document all identified confined spaces at the worksite.

Facility / Worksite: ________________________     Conducted by: ________________________     Date: ______________

| Space ID | Location / Description | Type | PRCS? (Y/N) | Known Hazards | Last Assessment Date |
| --- | --- | --- | --- | --- | --- |
| CS-001 |  |  |  |  |  |
| CS-002 |  |  |  |  |  |
| CS-003 |  |  |  |  |  |
| CS-004 |  |  |  |  |  |
| CS-005 |  |  |  |  |  |
| CS-006 |  |  |  |  |  |
| CS-007 |  |  |  |  |  |
| CS-008 |  |  |  |  |  |
| CS-009 |  |  |  |  |  |
| CS-010 |  |  |  |  |  |

Type codes: MH = Manhole, TK = Tank/Vessel, VT = Vault, PT = Pit, PP = Pipeline, TR = Trench, OT = Other

Reviewed by: ________________________  Date: ______________

# Appendix C: Pre-Entry Atmospheric Monitoring Record

Use this form to document continuous atmospheric monitoring during entry operations.

| Permit Number: |  |
| --- | --- |
| Space Location/ID: |  |
| Date: |  |
| Monitor Operator: |  |
| Instrument Make/Model: |  |
| Serial Number: |  |
| Calibration Date: |  |

## Monitoring Readings

| Time | O₂ (%) | LEL (%) | CO (ppm) | H₂S (ppm) | Other | Initials |
| --- | --- | --- | --- | --- | --- | --- |

## Action Taken for Out-of-Range Readings

| Time | Parameter | Reading | Action Taken |
| --- | --- | --- | --- |

Monitoring completed by: ________________________  Signature: ________________________

Entry Supervisor review: ________________________  Date: ______________