HAZARD COMMUNICATION

(HazCom) PROGRAM

Written Safety Program

Per 29 CFR 1910.1200 — Globally Harmonized System (GHS) Aligned

[COMPANY NAME]

[Company Address]

Effective Date: _______________

Revision Date: _______________

Prepared by Forbes Safety Solutions

# Table of Contents

1.0  Purpose and Scope

2.0  Responsibilities

3.0  Hazard Classification

4.0  Safety Data Sheets (SDS)

5.0  Labeling Requirements

6.0  Chemical Inventory

7.0  Employee Training

8.0  Non-Routine Tasks

9.0  Contractor Notification

10.0  Multi-Employer Worksites

11.0  Recordkeeping

Appendix A: Chemical Inventory Template

Appendix B: SDS Request Letter Template

Appendix C: Training Sign-Off Sheet

Appendix D: GHS Pictogram Reference Chart

# 1.0  Purpose and Scope

This Hazard Communication (HazCom) Program has been established to comply with OSHA's Hazard Communication Standard, 29 CFR 1910.1200, as aligned with the Globally Harmonized System of Classification and Labelling of Chemicals (GHS). The purpose of this program is to ensure that the hazards of all chemicals used, stored, or handled at [Company Name] worksites are evaluated, and that information concerning these hazards is transmitted to all affected employees.

This program applies to all employees who may be exposed to hazardous chemicals under normal working conditions or during foreseeable emergencies. This includes all work areas, job classifications, and operations where hazardous chemicals are present.

This written program is available to all employees and their designated representatives upon request. A copy is maintained at [location / office].

# 2.0  Responsibilities

## 2.1  Program Administrator

The HazCom Program Administrator is: [Name / Title]. This person is responsible for:

- Implementing and maintaining this written program

- Maintaining the chemical inventory and SDS library

- Ensuring proper labeling of all containers

- Coordinating employee training

- Communicating hazard information to contractors

- Reviewing and updating the program annually

## 2.2  Supervisors / Foremen

- Ensure employees receive HazCom training before working with hazardous chemicals

- Ensure proper labeling of all workplace containers

- Ensure Safety Data Sheets are accessible to all employees on every shift

- Enforce proper use of PPE when handling hazardous chemicals

## 2.3  Employees

- Attend required HazCom training

- Read and follow SDS information and label warnings

- Use required PPE when working with hazardous chemicals

- Report unlabeled containers, missing SDSs, or chemical spills to their supervisor immediately

# 3.0  Hazard Classification

[Company Name] relies on the hazard classification provided by chemical manufacturers and importers on their Safety Data Sheets and product labels. The company will not perform independent hazard classification.

Under the GHS-aligned HazCom standard, hazards are classified into two major groups:

Physical Hazards: Flammable gases, flammable liquids, flammable solids, oxidizers, compressed gases, explosives, self-reactive chemicals, pyrophoric materials, organic peroxides, corrosive to metals, and others.

Health Hazards: Acute toxicity, skin corrosion/irritation, serious eye damage/irritation, respiratory or skin sensitization, germ cell mutagenicity, carcinogenicity, reproductive toxicity, specific target organ toxicity (single and repeated exposure), and aspiration hazard.

When a new chemical is introduced to the workplace, the SDS must be obtained and reviewed before the chemical is used. The Program Administrator will update the chemical inventory and ensure proper labeling.

# 4.0  Safety Data Sheets (SDS)

[Company Name] will maintain a complete and current SDS for every hazardous chemical used at the worksite. SDSs will be readily accessible to all employees during each work shift.

## 4.1  SDS Management

- SDSs are maintained at: [Location — e.g., safety office, SDS binder in each work area, electronic system]

- New SDSs are obtained before any new chemical is brought to the worksite

- SDSs are reviewed and updated when new or revised versions become available

- If an SDS is not available for a chemical, the Program Administrator will contact the manufacturer/supplier using the SDS Request Letter (Appendix B)

- Electronic SDSs are acceptable provided employees can access them without barriers during each shift

## 4.2  The 16-Section SDS Format

Under the GHS, all SDSs must follow a standardized 16-section format:

- Section 1: Identification — Product name, manufacturer, emergency phone

- Section 2: Hazard(s) Identification — GHS classification, signal word, pictograms, hazard statements

- Section 3: Composition / Information on Ingredients

- Section 4: First-Aid Measures

- Section 5: Fire-Fighting Measures

- Section 6: Accidental Release Measures

- Section 7: Handling and Storage

- Section 8: Exposure Controls / Personal Protection

- Section 9: Physical and Chemical Properties

- Section 10: Stability and Reactivity

- Section 11: Toxicological Information

- Section 12: Ecological Information

- Section 13: Disposal Considerations

- Section 14: Transport Information

- Section 15: Regulatory Information

- Section 16: Other Information

# 5.0  Labeling Requirements

## 5.1  Manufacturer Labels

All containers of hazardous chemicals received at the worksite must have a GHS-compliant label that includes:

- Product identifier (chemical name)

- Signal word ("Danger" or "Warning")

- Hazard statement(s) describing the nature of the hazard

- Precautionary statement(s) describing recommended measures

- GHS pictogram(s) — red-bordered diamond symbols

- Manufacturer/supplier name, address, and phone number

## 5.2  Workplace (Secondary) Container Labels

When chemicals are transferred to secondary containers, the container must be labeled with at minimum:

- Product identifier (matching the SDS)

- Words, pictures, symbols, or a combination providing general information about the hazards

Exception: Portable containers for immediate use by the employee who performs the transfer do not require labeling, provided the chemical is under that employee's control during the entire shift.

## 5.3  Label Maintenance

Labels must not be defaced or removed. If a label becomes unreadable, the supervisor must ensure the container is relabeled immediately. Unlabeled containers must be removed from service until properly labeled.

# 6.0  Chemical Inventory

A master chemical inventory is maintained by the Program Administrator and updated whenever a new chemical is introduced or an existing chemical is removed from use. The inventory includes:

- Chemical / product name (matching the SDS)

- Manufacturer / supplier name

- Work area where the chemical is used or stored

- Hazard classification (physical and health)

- SDS on file (Yes / No)

- Date added to inventory

The Chemical Inventory Template is included in Appendix A.

# 7.0  Employee Training

All employees who may be exposed to hazardous chemicals must receive HazCom training. Training is provided:

- At the time of initial assignment to a work area with hazardous chemicals

- When a new chemical hazard is introduced to the work area

- Annually as a refresher

## 7.1  Training Content

Training must cover the following topics:

- An overview of the requirements of 29 CFR 1910.1200 and this written program

- Location and availability of the written HazCom Program and chemical inventory

- How to read and interpret GHS labels, including pictograms, signal words, hazard statements, and precautionary statements

- How to access, read, and use Safety Data Sheets

- Physical and health hazards of the chemicals in the work area

- Protective measures (PPE, engineering controls, work practices) to reduce exposure

- Emergency procedures for chemical spills, leaks, or exposures

- Procedures for non-routine tasks involving hazardous chemicals

The Training Sign-Off Sheet (Appendix C) must be completed for all training sessions.

# 8.0  Non-Routine Tasks

Before employees perform non-routine tasks that involve potential exposure to hazardous chemicals (e.g., cleaning tanks, entering confined spaces, emergency repairs), the supervisor must:

- Identify the hazardous chemicals that may be involved

- Review the applicable SDS(s) with the affected employees

- Ensure appropriate PPE and safety measures are in place

- Brief employees on the specific hazards and emergency procedures for the task

- Conduct atmospheric monitoring if required by the task or hazard

# 9.0  Contractor Notification

When contractors work at [Company Name] facilities or jobsites, the following information must be provided:

- The identity of hazardous chemicals to which they may be exposed while on site

- The location and availability of SDSs for those chemicals

- Precautionary measures the contractor's employees should take to protect themselves

- The location of this written HazCom Program and the chemical inventory

Contractors must also provide [Company Name] with SDSs for any hazardous chemicals they bring onto the worksite. Contractors are responsible for training their own employees on the hazards of their chemicals.

# 10.0  Multi-Employer Worksites

On multi-employer worksites (including construction projects), the host employer or general contractor will:

- Inform all employers of the location and availability of SDSs at the worksite

- Implement a system to communicate chemical hazards introduced by any employer

- Ensure that chemical inventory information is shared and accessible to all employers

- Coordinate labeling systems so all workers can understand the hazard information

Each employer on the multi-employer worksite remains responsible for training their own employees and maintaining their own HazCom program.

# 11.0  Recordkeeping

The following records must be maintained and made available upon request:

- This written Hazard Communication Program

- Chemical Inventory (updated as chemicals change)

- Safety Data Sheets for all hazardous chemicals (current and historical)

- Employee training records (including dates, topics, attendees, and trainer)

- SDS request letters sent to manufacturers/suppliers

Training records must be maintained for the duration of employment plus 3 years.

# Appendix A: Chemical Inventory Template

Use this template to track all hazardous chemicals present at the worksite. Update whenever chemicals are added or removed.

| Product Name | Manufacturer | Work Area / Location | Hazard Class | SDS on File? | Date Added |
| --- | --- | --- | --- | --- | --- |

# Appendix B: SDS Request Letter Template

[Date]

[Manufacturer/Supplier Name]

[Address]

[City, State, ZIP]

Re: Request for Safety Data Sheet(s)

Dear Sir or Madam:

Under the OSHA Hazard Communication Standard (29 CFR 1910.1200), employers are required to maintain Safety Data Sheets (SDSs) for all hazardous chemicals present in the workplace. We have been unable to locate the SDS for the following product(s) in our files:

[Product Name / Product #]

Please forward the current SDS for the above product(s) to:

[Company Name]

[Attention: Program Administrator Name]

[Address]

[City, State, ZIP]

[Email Address]

If you have any questions, please contact us at [Phone Number]. We appreciate your prompt attention to this matter.

Sincerely,

_________________________________

[Name, Title]

[Company Name]

# Appendix C: Training Sign-Off Sheet

Hazard Communication (HazCom) Training

| Training Date: |  |
| --- | --- |
| Trainer Name: |  |
| Location / Project: |  |
| Topics Covered: | ☐ Written HazCom Program ☐ Chemical Inventory ☐ SDS Access & Use ☐ GHS Labels ☐ PPE Requirements ☐ Spill Response ☐ Non-Routine Tasks |

By signing below, I acknowledge that I have received training on the company's Hazard Communication Program, including how to read labels, access Safety Data Sheets, and protect myself from chemical hazards.

| # | Print Name | Company / Employer | Signature | Date |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |
| 9 |  |  |  |  |
| 10 |  |  |  |  |
| 11 |  |  |  |  |
| 12 |  |  |  |  |
| 13 |  |  |  |  |
| 14 |  |  |  |  |
| 15 |  |  |  |  |

# Appendix D: GHS Pictogram Reference Chart

The Globally Harmonized System (GHS) uses standardized pictograms on labels and SDSs to communicate specific hazards. Each pictogram is a red-bordered diamond shape with a black symbol on a white background. Below is a description of each pictogram and its meaning.

| Pictogram Symbol | Pictogram Name | Hazard Description |
| --- | --- | --- |
| 🔥 Flame | Flammable | Flammable gases, liquids, solids, aerosols. Self-reactive substances. Pyrophoric materials. Self-heating substances. Substances that emit flammable gases when in contact with water. Organic peroxides. |
| ⭕ Flame Over Circle | Oxidizer | Oxidizing gases, liquids, and solids. May cause or intensify fire. May cause explosion. |
| 💀 Skull and Crossbones | Acute Toxicity (Severe) | Chemicals that are fatal, toxic, or highly toxic if swallowed, inhaled, or absorbed through the skin. Categories 1-3 acute toxicity. |
| ⚠️ Exclamation Mark | Irritant / Harmful | Skin and eye irritation. Skin sensitization. Acute toxicity (Category 4 — harmful). Narcotic effects. Respiratory tract irritation. Hazardous to ozone layer. |
| 🫁 Health Hazard (Silhouette) | Serious Health Hazard | Carcinogenicity. Respiratory sensitization. Reproductive toxicity. Specific target organ toxicity (single and repeated exposure). Germ cell mutagenicity. Aspiration hazard. |
| 🧪 Corrosion | Corrosive | Skin corrosion / burns. Serious eye damage. Corrosive to metals. |
| 💨 Gas Cylinder | Compressed Gas | Compressed gases. Liquefied gases. Refrigerated liquefied gases. Dissolved gases. May explode if heated. |
| 💣 Exploding Bomb | Explosive | Explosives. Self-reactive substances. Organic peroxides. Unstable explosives. Divisions 1.1 through 1.6. |
| 🌊 Environment | Environmental Hazard | Acute and chronic aquatic toxicity. Hazardous to the aquatic environment. Not mandated by OSHA but may appear on labels/SDSs. |

Signal Words: "DANGER" indicates more severe hazards. "WARNING" indicates less severe hazards. A product will display only one signal word — the most severe applicable.

Hazard Statements: Standardized phrases describing the nature and degree of hazard (e.g., "Highly flammable liquid and vapor," "Causes serious eye damage"). These appear on both labels and SDSs.

Precautionary Statements: Recommended measures to minimize or prevent adverse effects from exposure, storage, or handling (e.g., "Keep away from heat/sparks/open flames," "Wear protective gloves/eye protection").