FORBES SAFETY SOLUTIONS

Personal Protective
Equipment (PPE) Program

Comprehensive PPE Hazard Assessment, Selection & Training

Prepared for:

[COMPANY NAME]

Reference Standards:

29 CFR 1910.132-138 | 29 CFR 1926.95-106

Date: [DATE]  |  Revision: [REV NUMBER]

# Table of Contents

# 1. Purpose

This Personal Protective Equipment (PPE) Program establishes the requirements for the selection, use, maintenance, and training associated with personal protective equipment at [COMPANY NAME]. The purpose is to protect employees from workplace hazards that cannot be eliminated through engineering or administrative controls, in compliance with OSHA standards 29 CFR 1910.132-138 and 29 CFR 1926.95-106.

PPE is the last line of defense in the hierarchy of controls and shall be used when hazards cannot be adequately controlled by other means.

# 2. Scope

This program applies to all employees, contractors, temporary workers, and visitors at [COMPANY NAME] facilities and job sites where workplace hazards require the use of personal protective equipment.

This program covers the following categories of PPE:

- Head protection (hard hats, bump caps)

- Eye and face protection (safety glasses, goggles, face shields)

- Hand protection (gloves by hazard type)

- Foot protection (safety-toed footwear, metatarsal guards)

- Hearing protection (earplugs, earmuffs)

- Respiratory protection (see separate Respiratory Protection Program)

- Body/skin protection (high-visibility vests, protective clothing)

- Fall protection (see separate Fall Protection Program)

# 3. Responsibilities

## 3.1 Management

- Provide funding for all required PPE at no cost to employees

- Ensure hazard assessments are conducted and certified

- Support enforcement of PPE requirements

- Review and update this program at least annually

## 3.2 Safety Manager / Competent Person

- Conduct and certify workplace hazard assessments

- Select appropriate PPE based on identified hazards

- Coordinate PPE training for all affected employees

- Maintain records of hazard assessments, training, and PPE issuance

- Evaluate new PPE products and technologies

- Conduct periodic audits of PPE use and condition

## 3.3 Supervisors

- Enforce PPE requirements within their work areas

- Monitor employees for proper PPE use, fit, and condition

- Ensure new employees receive PPE training before beginning work

- Report PPE deficiencies and request replacement equipment

## 3.4 Employees

- Wear required PPE at all times when exposed to hazards

- Inspect PPE before each use and report defects

- Maintain and store PPE properly according to training

- Attend all required PPE training sessions

- Notify supervisor of any changes in hazard conditions

# 4. Hazard Assessment

## 4.1 Hazard Assessment Process

The Safety Manager shall conduct a hazard assessment of each work area and job task to identify sources of hazards that require PPE. The assessment shall follow the requirements of 29 CFR 1910.132(d) and include:

- Walk-through survey of each work area

- Identification of hazard sources (impact, penetration, compression, chemical, heat, harmful dust, light/radiation, biohazards, electrical, fall, noise)

- Organization of findings by job classification and work area

- Selection of appropriate PPE for each identified hazard

- Written certification of the hazard assessment

## 4.2 Job-by-Job Hazard Assessment

Each job task shall be individually assessed. The following table provides a framework for documenting hazards by job task:

| Job Task / Area | Hazard Type | Body Part at Risk | PPE Required |
| --- | --- | --- | --- |
| Welding Operations | Optical radiation, sparks, fumes | Eyes, face, hands, body | Welding helmet, safety glasses, leather gloves, FR clothing |
| Grinding / Cutting | Flying particles, noise | Eyes, face, ears, hands | Safety glasses w/ side shields, face shield, hearing protection, gloves |
| Chemical Handling | Splash, vapor, skin absorption | Eyes, face, hands, body | Chemical splash goggles, face shield, chemical-resistant gloves, apron |
| Overhead Work | Falling objects, impact | Head, feet | Hard hat (Type I or II), safety-toed footwear |
| Electrical Work | Shock, arc flash, burn | Head, eyes, hands, body | Dielectric hard hat, arc-rated face shield, insulated gloves, arc-rated clothing |
| Concrete / Masonry | Dust, abrasion, impact | Eyes, lungs, hands, knees | Safety glasses, dust mask/respirator, leather gloves, knee pads |
| Elevated Work (>6 ft) | Falls | Full body | Full body harness, lanyard, anchor (see Fall Protection Program) |
| [Add Job Tasks] | [Identify Hazards] | [Body Parts] | [Select PPE] |

## 4.3 Certification of Hazard Assessment

Per 29 CFR 1910.132(d)(2), the hazard assessment must be certified in writing. The certification shall include:

- Identification of the workplace evaluated

- Name of the person performing the assessment

- Date(s) of the assessment

- Document identification as a certification of hazard assessment

See Appendix A for the PPE Hazard Assessment Certification Form.

# 5. PPE Selection

## 5.1 PPE Selection Table by Hazard Type

The following table provides guidance for selecting appropriate PPE based on the type of hazard identified during the assessment:

| Hazard Category | Hazard Examples | PPE Type | Applicable Standard |
| --- | --- | --- | --- |
| Impact | Flying objects, falling tools | Hard hat, safety glasses w/ side shields, safety-toed shoes | ANSI Z89.1, ANSI Z87.1, ASTM F2413 |
| Penetration | Sharp objects, nails | Puncture-resistant soles, cut-resistant gloves, safety glasses | ASTM F2413, ANSI/ISEA 105 |
| Compression (rollover) | Heavy equipment, rolling objects | Safety-toed footwear (metatarsal guard) | ASTM F2413 |
| Chemical exposure | Splashes, vapors, mists | Chemical splash goggles, face shield, chemical-resistant gloves, apron | ANSI Z87.1, manufacturer specs |
| Heat / Burns | Hot surfaces, sparks, flames | Leather gloves, FR clothing, welding helmet, face shield | NFPA 2112, ASTM F2302 |
| Harmful dust | Silica, wood dust, metals | Dust mask or respirator, safety goggles | NIOSH 42 CFR Part 84 |
| Optical radiation | Welding arc, lasers, UV | Welding helmet (proper shade), laser safety glasses | ANSI Z87.1, ANSI Z136 |
| Noise (>85 dBA) | Machinery, power tools | Earplugs (NRR 25+), earmuffs (NRR 25+) | ANSI S3.19 / S12.6 |
| Electrical | Shock, arc flash | Insulated gloves (class rated), dielectric footwear, arc-rated PPE | ASTM D120, NFPA 70E |
| Fall hazards | Elevated work >6 ft | Full body harness, SRL, lanyard, anchor | ANSI Z359.1 |

# 6. PPE Requirements by Body Part

## 6.1 Head Protection

Hard hats are required in all areas where there is a danger of head injury from falling objects, impact, or electrical contact. Per 29 CFR 1910.135 and 1926.100:

- Type I hard hats protect the top of the head from impact

- Type II hard hats protect the top and sides from impact

- Class E (Electrical) rated to 20,000 volts

- Class G (General) rated to 2,200 volts

- Class C (Conductive) provides no electrical protection

Hard hats shall be replaced when cracked, dented, or after any significant impact. Suspension systems shall be replaced per manufacturer guidelines. Hard hats shall not be painted, drilled, or modified.

## 6.2 Eye and Face Protection

Eye and face protection is required per 29 CFR 1910.133 and 1926.102 wherever there is a reasonable probability of injury from flying particles, molten metal, liquid chemicals, acids, caustic liquids, chemical gases/vapors, or light radiation.

- Safety glasses with side shields: minimum requirement for general hazard areas

- Safety goggles: chemical splash, dust, fine particles

- Face shields: used OVER safety glasses/goggles for additional protection

- Welding helmets: minimum shade per OSHA/ANSI welding shade guide

All eye/face protection shall meet ANSI Z87.1 and be marked with the manufacturer's mark and Z87+ for high-impact rating.

## 6.3 Hand Protection

Per 29 CFR 1910.138, appropriate hand protection shall be selected based on the specific hazards identified:

| Hazard | Glove Type | Key Considerations |
| --- | --- | --- |
| Cuts / Abrasion | Cut-resistant (ANSI/ISEA A2–A9 rating) | Match cut level to hazard severity |
| Chemical exposure | Chemical-resistant (nitrile, neoprene, butyl, PVA) | Check permeation data for specific chemicals |
| Heat / Flame | Leather, Kevlar, aluminized | Temperature rating must exceed exposure |
| Electrical | Insulated gloves (Class 00–4) | Must be tested per ASTM D120 every 6 months |
| Cold weather | Insulated, waterproof | Maintain dexterity for task performance |
| Welding | Leather welding gloves | Minimum gauntlet-style for arc welding |

## 6.4 Foot Protection

Safety-toed footwear meeting ASTM F2413 is required in all areas where there is a danger of foot injuries from falling objects, compression, puncture, or electrical hazards. Requirements include:

- Impact-resistant toe cap (I/75 or I/50 rating)

- Compression-resistant toe (C/75 or C/50 rating)

- Puncture-resistant sole plate when required

- Metatarsal guards for heavy material handling

- Electrical hazard (EH) rated footwear in electrical work areas

## 6.5 Hearing Protection

Hearing protection is required for all employees exposed to noise levels at or above 85 dBA as an 8-hour TWA per 29 CFR 1910.95 and the company Hearing Conservation Program. Options include:

- Foam earplugs (NRR 25–33): disposable or reusable

- Pre-molded earplugs (NRR 22–27): reusable, custom-fit available

- Earmuffs (NRR 22–31): over-the-head or cap-mounted

- Dual protection (plugs + muffs): required when exposure exceeds 100 dBA

## 6.6 Respiratory Protection

Respiratory protection is addressed in the separate [COMPANY NAME] Respiratory Protection Program per 29 CFR 1910.134. When respiratory hazards are identified during the PPE hazard assessment, refer to the Respiratory Protection Program for proper selection, fit testing, medical evaluation, and training requirements.

# 7. Proper Fit and Use

All PPE must properly fit each affected employee. Where manufacturer sizing is available, employees shall be fitted according to manufacturer guidelines. Key requirements:

- PPE shall not create additional hazards (e.g., loose clothing near machinery)

- PPE must be compatible when multiple types are worn simultaneously (e.g., hard hat + earmuffs + safety glasses)

- Prescription eyewear wearers shall use safety glasses with prescription inserts, over-the-glasses (OTG) safety glasses, or prescription safety glasses meeting ANSI Z87.1

- Employees with medical conditions affecting PPE use shall be evaluated by occupational health

# 8. Inspection and Maintenance

Employees shall inspect PPE before each use. PPE that is damaged, defective, or has exceeded its service life shall be removed from service immediately.

## 8.1 Inspection Criteria

- Hard hats: Check shell for cracks, dents, penetration, UV degradation (chalky appearance); check suspension for fraying, cracking, or loss of elasticity

- Safety glasses/goggles: Check for scratched lenses, broken frames, missing side shields, damaged seals

- Gloves: Check for tears, punctures, chemical degradation, loss of flexibility

- Safety footwear: Check for exposed toe caps, sole separation, worn tread, damaged eyelets

- Fall protection harnesses: Follow manufacturer inspection criteria (webbing, buckles, D-rings, labels)

- Hearing protection: Check for hardening, deformation, cracking, broken headbands

## 8.2 Maintenance and Storage

- Clean PPE according to manufacturer instructions

- Store PPE in a clean, dry location away from sunlight and contaminants

- Do not modify or alter PPE in any way

- Replace PPE at manufacturer-recommended intervals or when defects are found

- Maintain an adequate inventory of replacement PPE

# 9. Employee-Owned PPE

Per 29 CFR 1910.132(h), where employees provide their own PPE, the employer is responsible to ensure that it is adequate, properly maintained, and sanitary.

- Employee-owned PPE must meet the same standards as employer-provided PPE

- The employer shall verify the adequacy of employee-owned PPE

- The employer is not required to reimburse employees for non-specialty safety-toed footwear, prescription safety glasses, or logging boots required by 29 CFR 1910.266(d)(1)(v)

- All other required PPE shall be provided by [COMPANY NAME] at no cost to employees

# 10. Training

Per 29 CFR 1910.132(f), each employee required to use PPE shall be trained to know at a minimum:

- When PPE is necessary

- What PPE is required for their specific job tasks

- How to properly put on, take off, adjust, and wear PPE

- The limitations of the PPE

- The proper care, maintenance, useful life, and disposal of the PPE

## 10.1 Training Frequency

- Initial training: before job assignment requiring PPE

- Retraining required when:

  - Changes in the workplace render previous training inadequate

  - Changes in the types of PPE render previous training inadequate

  - Employee demonstrates lack of understanding or skill in proper PPE use

## 10.2 Training Verification

Each employee who has been trained shall demonstrate understanding of the training elements. Training shall be documented and include the employee's name, date, trainer's name, and topics covered.

# 11. Program Evaluation

[COMPANY NAME] shall evaluate the effectiveness of this program at least annually, or whenever conditions change that may affect PPE needs. Evaluation shall include:

- Review of workplace hazard assessments for accuracy

- Assessment of PPE selection adequacy

- Review of injury/illness records related to PPE

- Employee feedback on PPE comfort, fit, and effectiveness

- Audit of PPE use, condition, and compliance

- Review of new PPE technologies and products

# 12. Recordkeeping

The following records shall be maintained:

- Written hazard assessment certifications

- PPE issue records (type, date, employee)

- Training records (date, attendees, topics, trainer)

- Inspection and maintenance logs (as applicable)

- Program evaluation records

Records shall be retained for the duration of employment plus three (3) years.

# Appendix A: PPE Hazard Assessment Certification Form

CERTIFICATION OF HAZARD ASSESSMENT FOR PPE

Per 29 CFR 1910.132(d)(2)

Company: _______________________________

Work Area / Job Site: _______________________________

Assessment Date: _______________________________

Assessed By: _______________________________

Title: _______________________________

Hazards Identified:

| Job Task / Area | Hazard Source | Hazard Type | Body Part(s) | PPE Required |
| --- | --- | --- | --- | --- |

Certification Statement:

I certify that a hazard assessment has been performed for the workplace identified above, that the assessment identified the hazards requiring personal protective equipment, and that the PPE specified above has been selected to protect affected employees from those hazards.

Assessor Name (Print): ________________________________________

Assessor Signature: ________________________________________

Date: ________________________________________

Reviewed By (Management): ________________________________________

Date: ________________________________________

# Appendix B: PPE Issue Record

PERSONAL PROTECTIVE EQUIPMENT ISSUE RECORD

Employee Name: _______________________________

Employee ID / Number: _______________________________

Department / Crew: _______________________________

Job Title: _______________________________

| Date | PPE Item Issued | Size | Qty | Condition | Reason (New/Replace) | Issued By | Employee Initials |
| --- | --- | --- | --- | --- | --- | --- | --- |

Employee Acknowledgment:

I have received the PPE listed above and have been trained on its proper use, care, and maintenance. I understand that I am responsible for wearing the PPE as required and for reporting any defects or damage.

Employee Signature: ________________________________________

Date: ________________________________________