FORBES SAFETY SOLUTIONS

JOB HAZARD ANALYSIS

| JHA INFORMATION |  |  |  |
| --- | --- | --- | --- |
| JHA Title: | Mobile Crane Lift Operations | JHA Number: | FSS-JHA-030 |
| Project Name: |  | Date: |  |
| Location: |  | Revision: | 0 |
| Regulatory References: | 29 CFR 1926 Subpart CC (Cranes & Derricks in Construction); ASME B30.5 (Mobile and Locomotive Cranes); OSHA 29 CFR 1926.1400-1442 |  |  |
| Prepared By: |  | Approved By: |  |
| Competent Person: |  | Supervisor: |  |

JOB HAZARD ANALYSIS — TASK BREAKDOWN

| Task Step | Hazard | Risk Level (H/M/L) | Control Measures |
| --- | --- | --- | --- |
| 1. Lift Planning & Load Chart Review | • Crane overload / tip-over • Incorrect load weight estimation • Unanticipated boom/load radius • Inadequate lift plan for critical lifts | H | • Conduct lift planning meeting with operator, rigger, and signal person • Determine actual load weight (weigh or calculate — add rigging weight) • Reference crane load chart for configuration, radius, and boom length • Ensure lift does not exceed 75% of rated capacity (critical lift threshold) • Develop written Critical Lift Plan for loads >75% capacity or special conditions • Review swing path, travel route, and landing zone for obstructions • Verify operator certification (NCCCO or equivalent) is current |
| 2. Ground Preparation & Crane Setup | • Ground failure under outriggers • Crane instability on slopes • Underground voids / utilities • Insufficient outrigger extension | H | • Evaluate ground conditions — bearing capacity must support crane + load reactions • Place outrigger pads/mats sized to distribute ground bearing pressure • Extend all outriggers fully and level crane within 1% grade • Call 811 / utility locate before setting up over unknown subsurface • Confirm level using crane's built-in level indicator • Barricade swing radius to prevent unauthorized access • Set up on compacted, stable surface — no soft fill or excavation edges |
| 3. Rigging & Load Attachment | • Rigging failure / sling breakage • Improper hitch causing load shift • Struck by rigging hardware • Pinch points during attachment | H | • Qualified rigger selects rigging based on load weight and sling angles • Inspect all slings, shackles, and hardware before each lift — reject damaged gear • Match sling capacity to load — account for sling angle reduction factor • Use proper hitch type (basket, choker, vertical) for load geometry • Ensure load is properly balanced and center of gravity identified • Rig tag lines to control load orientation during lift • Never stand under suspended load — stay out of fall zone |
| 4. Load Pick & Travel | • Tip-over during lift or swing • Overhead power line contact • Struck by swinging load • Two-blocking the hoist • Loss of load control in wind | H | • Use designated signal person with clear communication (hand signals or radio) • Perform initial test lift — raise load 6 inches, hold, verify stability • Maintain minimum 20-ft clearance from power lines (or de-energize / use spotter) • Anti-two-block device must be functional — test at start of shift • Monitor wind speed — cease operations if wind exceeds crane chart limits • Clear all personnel from swing radius and load travel path • Operator must have clear line of sight to load at all times • Never travel with load over occupied areas |
| 5. Load Placement & Unhooking | • Pinch/crush during placement • Load shift on landing • Premature unhooking • Unstable load support | M | • Land load on adequate dunnage/cribbing to support weight evenly • Use tag lines to guide load into final position — never push/pull by hand • Do not unhook until load is confirmed stable and properly supported • Rigger signals all clear before operator slacks rigging • Keep hands and body clear of pinch points during landing • Block and secure load against movement before releasing rigging • Verify load placement matches engineered drawings if applicable |
| 6. Crane Breakdown & Demobilization | • Boom contact with structures during retraction • Outrigger retraction on unsupported crane • Transport hazards on roadway • Improper securing for transport | M | • Retract boom fully and secure in boom rest/travel position • Retract outriggers only after boom is fully secured • Verify swing lock is engaged before transport • Inspect crane for damage and document in operator log • Ensure all loose components are secured for transport • Develop transport plan for oversized loads (permits, escorts) • Restore site — remove outrigger pads, fill any ground depressions • File completed lift plan and inspection records with project documentation |

REQUIRED PERSONAL PROTECTIVE EQUIPMENT (PPE)

| PPE Required | Specification / ANSI Standard | Condition / Notes |
| --- | --- | --- |
| Hard Hat | ANSI/ISEA Z89.1, Type I, Class E | Required for all personnel in crane operating zone |
| Safety Glasses | ANSI/ISEA Z87.1+ | Impact-rated with side shields |
| High-Visibility Vest | ANSI/ISEA 107, Class 2 minimum | Required for all ground personnel |
| Work Gloves (Rigger's Gloves) | ANSI/ISEA 105, Cut Level A4+ | Leather palm for rigging work |
| Steel-Toe Safety Boots | ASTM F2413 (I/75 C/75) | Puncture-resistant soles recommended |
| Fall Protection Harness | ANSI/ASSP Z359.1 | Required when working at heights on crane or load |
| Hearing Protection | ANSI S3.19 / S12.6 (NRR ≥ 25 dB) | Required near crane engine operations |
| Communication Radio | N/A — operational requirement | Dedicated channel for crane operator, signal person, rigger |

EMERGENCY PROCEDURES

Crane Tip-Over / Structural Failure:

• Sound alarm — all personnel evacuate crane operating zone immediately

• Call 911 — report crane incident with potential trapped persons

• Establish exclusion zone (minimum 1.5x boom length radius)

• Account for all personnel at assembly point

• Do not approach overturned crane — risk of fuel/hydraulic spill and secondary collapse

• Notify crane manufacturer/engineer for recovery planning

• Preserve scene for OSHA investigation

Overhead Power Line Contact:

• Operator stays in cab — DO NOT exit crane (equipotential zone)

• If crane is drivable, move away from power line contact

• If must exit: JUMP clear — do not step down (step potential danger)

• Call 911 and utility emergency — report power line contact

• Keep all personnel at least 35 feet from crane and any grounded equipment

• Do not touch crane or load until utility confirms line is de-energized

Rigging Failure / Dropped Load:

• Operator sets brakes and secures crane — do not attempt to recover

• Evacuate drop zone and establish exclusion perimeter

• Account for all personnel — check for injuries

• Call 911 if any injuries or structural damage to occupied areas

• Inspect all rigging and crane before any further lifts

• Conduct incident investigation and root cause analysis

Emergency Contact Information:

• Emergency Services: 911

• Crane Company Emergency: [Insert Name & Phone]

• Site Safety Manager: [Insert Name & Phone]

• Nearest Hospital: [Insert Name, Address & Phone]

• Utility Emergency: [Insert Local Utility Number]

WORKER SIGN-OFF — I HAVE READ AND UNDERSTAND THIS JHA

| # | Print Name | Signature | Company / Trade | Date |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |
| 9 |  |  |  |  |
| 10 |  |  |  |  |

REVIEW & APPROVAL

| Role | Name / Signature | Date | Initials |
| --- | --- | --- | --- |
| Prepared By (Author) |  |  |  |
| Reviewed By (Safety Manager) |  |  |  |
| Approved By (Project Manager) |  |  |  |
| Client / Owner Rep |  |  |  |

This JHA must be reviewed and updated whenever job conditions change, after any incident, or at minimum annually.