[  COMPANY LOGO  ]

JOB HAZARD ANALYSIS

Rigging & Crane Lift Operations

Applicable Standards: 29 CFR 1926 Subpart CC (1926.1400–1926.1442) — Cranes & Derricks; ASME B30.5 — Mobile Cranes; ASME B30.9 — Slings; OSHA 1926.1431 — Hoisting Personnel

| Project Name: ____________________________ | Date: ____________________________ |
| --- | --- |
| Location: ____________________________ | Supervisor: ____________________________ |
| Crew Members: _______________________________________________________________________________ |  |

| Step # | Job Steps | Potential Hazards | Hazard Controls | Responsible Person |
| --- | --- | --- | --- | --- |
| 1 | Conduct pre-lift planning — review lift plan, load weight, crane capacity at required radius | Crane overloading (structural failure/tip-over); incorrect load weight estimate; inadequate ground conditions | Qualified rigger calculates total load (object + rigging + block). Verify crane load chart at planned radius and boom length — stay below 75% of rated capacity for routine lifts. Review ground bearing capacity. Document lift plan — critical lifts (>75% capacity) require engineered lift plan. |  |
| 2 | Set up crane on firm, level ground — extend outriggers and verify LMI/anti-two-block function | Crane tip-over from inadequate ground support; outrigger pad failure; LMI malfunction leading to undetected overload | Level crane within 1% grade (or per manufacturer spec). Outriggers fully extended with timber mats rated for ground pressure. Function-test LMI (Load Moment Indicator) and anti-two-block device before first lift. Daily crane inspection per 1926.1412. |  |
| 3 | Inspect all rigging hardware — slings, shackles, hooks, and spreader beams | Rigging failure from damaged or overloaded hardware; dropped load; struck-by from failed rigging component | Qualified rigger inspects all rigging per ASME B30.9 — reject any sling with visible damage, broken wires, or missing ID tags. Verify hardware WLL (Working Load Limit) exceeds calculated sling loads. Use proper sling angles (never <30° included angle). Document inspection. |  |
| 4 | Rig the load — attach slings, connect shackles, verify center of gravity, install taglines | Crush injuries during rigging attachment; pinch points between load and rigging; unbalanced load; tag line entanglement | Only qualified riggers attach hardware to loads. Hands clear of pinch points — use push/pull sticks. Verify CG with trial lift (raise 6-12 inches and pause). Install taglines of sufficient length — never wrap around hands. Use softeners on sling contact edges. |  |
| 5 | Clear the lift zone — establish swing radius barricades and verify power line clearance | Workers struck by suspended load or crane components; boom contact with overhead power lines (electrocution); unauthorized entry into lift zone | Barricade swing radius plus 10 ft. Assign signal person with dedicated radio channel. Verify min 20-ft clearance from power lines per 1926.1408 (or use table for voltage >350kV). No personnel under suspended load at any time. Air horn for lift initiation warning. |  |
| 6 | Execute the lift — signal person directs crane operator through pick, swing, and travel | Struck-by from load swing; crane tip-over during dynamic loading; loss of communication between operator and signal person; two-blocking | Standard hand signals or radio per 1926.1419–1926.1421. Smooth, controlled movements — no sudden stops or swings. Monitor LMI continuously. One signal person at a time (designated). Stop lift immediately for any "all stop" signal from any worker. |  |
| 7 | Set and land the load at designated location | Crush injuries during landing; workers caught between load and structure; loss of load control in wind; rigging snagging on structure | Tagline operators guide load to within 12 inches. Landing zone crew clear until load is within 2 ft of landing. Use dunnage/cribbing at set point. Workers approach from upwind side only. Release load tension before disconnecting rigging. Wind speed limit: suspend lifts if sustained winds >20 mph (or per lift plan). |  |
| 8 | Disconnect rigging from landed load | Load shift after tension release; falling hardware when disconnecting; residual energy in rigging (snap-back) | Verify load is stable and fully supported before disconnecting. Rigger confirms no residual tension on slings. Remove rigging from topside when possible. Stack rigging hardware properly — never leave on ground in walkways. Inspect rigging for damage before next lift. |  |
| 9 | Multi-crane lifts (when applicable) — coordinated dual-crane operations | Unequal load sharing leading to overload; boom collision; communication failure between operators; loss of synchronization | Engineered lift plan mandatory for all multi-crane lifts. Each crane limited to 75% of capacity at max radius. Dedicated signal person for each crane + lift director. Pre-lift meeting with all operators. Continuous radio communication. Abort procedure agreed upon and rehearsed. |  |
| 10 | Secure crane at end of shift — boom stow, lockout, and documentation | Unattended crane movement; boom interference with other operations; theft/vandalism | Boom lowered and secured per manufacturer procedure. Swing lock engaged. Main disconnect locked out. Remove keys. Complete operator daily log — note any deficiencies. Report maintenance items to crane coordinator. Barricade crane perimeter if left on-site. |  |

⚠ This JHA must be reviewed and signed by all crew members before work begins. Conditions on the jobsite may change — reassess hazards throughout the shift and update this JHA as necessary.

REVIEW & APPROVAL

| ________________________________________ Reviewed By (Print Name) | ________________________________________ Signature | ________________________________________ Date |
| --- | --- | --- |