FORBES SAFETY SOLUTIONS

Mobile Crane
Operation Program

OSHA Compliance Program

Prepared for:

[COMPANY NAME]

Regulatory References:

29 CFR 1926 Subpart CC | ASME B30.5

Date: [DATE]     |     Revision: [REV NUMBER]

Prepared by: [SAFETY MANAGER NAME / TITLE]

# Table of Contents

# 1. Purpose and Scope

This Mobile Crane Operation Program establishes the requirements and procedures for the safe operation, inspection, and maintenance of mobile cranes used by [COMPANY NAME]. This program has been developed to comply with OSHA's Cranes and Derricks in Construction standard (29 CFR 1926 Subpart CC) and ASME B30.5 (Mobile and Locomotive Cranes).

This program applies to all employees, operators, riggers, signal persons, and subcontractors involved in mobile crane operations at [COMPANY NAME] work sites. It covers all types of mobile cranes, including lattice boom crawler cranes, hydraulic truck cranes, rough terrain cranes, all-terrain cranes, and carry deck cranes.

## 1.1 Regulatory References

- 29 CFR 1926 Subpart CC – Cranes and Derricks in Construction

- ASME B30.5 – Mobile and Locomotive Cranes

- ASME B30.3 – Construction Tower Cranes (where applicable)

- 29 CFR 1926.1400-1442 – Specific crane operation standards

- ANSI/NCCCO – Crane Operator Certification Standards

- Manufacturer specifications and load charts for each crane in service

# 2. Roles and Responsibilities

## 2.1 Crane Program Administrator

[SAFETY MANAGER NAME / TITLE] is responsible for:

- Implementing and maintaining this Mobile Crane Operation Program

- Verifying crane operator certifications are current and valid

- Ensuring all crane inspections are conducted and documented

- Reviewing and approving lift plans for critical/complex lifts

- Investigating crane-related incidents and near-misses

- Coordinating crane-related training

## 2.2 Crane Operators

- Hold a valid NCCCO, NCCER, or equivalent operator certification for the type and capacity of crane being operated

- Perform daily/shift pre-operation inspections per this program

- Operate the crane in accordance with manufacturer specifications, load charts, and this program

- Refuse to make a lift if conditions are unsafe (operator has stop-work authority)

- Maintain awareness of load weight, radius, capacity, and environmental conditions

## 2.3 Riggers

- Be qualified per 29 CFR 1926.1404 (qualified rigger criteria)

- Select and inspect rigging hardware before each use

- Attach loads in accordance with rigging best practices and the lift plan

- Communicate with the operator and signal person throughout the lift

## 2.4 Signal Persons

- Be qualified per 29 CFR 1926.1428 (qualified signal person criteria)

- Use standard ASME hand signals or radio communication

- Maintain clear line of sight with the operator at all times

- Stop the lift if any unsafe condition is observed

## 2.5 Competent Person

A designated competent person shall be assigned to each crane operation and is responsible for evaluating ground conditions, supervision of assembly/disassembly, power line clearance compliance, and hazard assessment.

# 3. Operator Certification Requirements

All crane operators at [COMPANY NAME] shall meet the operator certification requirements of 29 CFR 1926.1427:

## 3.1 Certification Standards

- Operators shall be certified by an accredited testing organization (NCCCO, NCCER, CIC, or OECP) for the type and capacity of crane being operated.

- Certification must be current and not expired, suspended, or revoked.

- Operators must carry their certification card on-site during crane operations.

- A copy of the certification shall be maintained in the operator's personnel file.

## 3.2 Medical Fitness

Crane operators shall meet the medical fitness requirements, including adequate vision, hearing, and physical capability to safely operate the crane. Medical evaluations shall be conducted in accordance with ASME B30.5 requirements.

## 3.3 Familiarization

Before operating any crane for the first time, operators must receive familiarization training on the specific make, model, and configuration, including:

- Operating controls and instrumentation

- Safety devices (LMI, anti-two-block, boom length/angle indicators)

- Emergency procedures

- Load chart review and interpretation for the specific crane configuration

# 4. Crane Selection and Lift Planning

## 4.1 Crane Selection for the Job

The appropriate crane shall be selected based on the following factors:

- Load weight (including rigging weight, headache ball, and below-the-hook devices)

- Lift radius (distance from center of rotation to the load)

- Lift height required

- Ground conditions and available setup area

- Crane capacity at the required radius (per the load chart)

- Boom length and configuration needed

- Counterweight configuration

- Proximity to power lines, structures, and other cranes

## 4.2 Load Charts – Radius and Capacity

The load chart is the most critical reference for safe crane operation. All lifts shall be planned using the crane's load chart, which is specific to the crane's make, model, boom length, configuration, and counterweight.

NOTE: The crane's rated capacity at any radius is the MAXIMUM load that may be lifted. [COMPANY NAME] requires that no lift exceed 75% of the crane's rated capacity unless a critical lift plan is in place.

Key load chart considerations:

- Always use the correct chart for the actual crane configuration (boom type, jib, outrigger/crawler position)

- Account for the weight of all rigging, blocks, headache balls, and below-the-hook lifting devices

- Capacity decreases as radius increases – never exceed the rated capacity at any radius

- On outriggers: capacity may vary depending on whether outriggers are fully or partially extended

- On rubber: capacity is significantly reduced compared to operation on outriggers

# 5. Ground Conditions Assessment

A competent person shall evaluate ground conditions before crane setup to ensure the ground can support the crane, outrigger loads, and the maximum anticipated lifted load.

## 5.1 Ground Condition Requirements

- Ground must be firm, level, drained, and adequately compacted to support the crane's maximum outrigger/track loading.

- Outrigger pads, cribbing, or mats shall be used to distribute outrigger loads. Pad size shall be calculated based on soil bearing capacity.

- Underground utilities, vaults, tanks, and recently backfilled trenches shall be identified and avoided.

- Slopes, edges of excavations, and soft soil areas shall be evaluated and mitigated.

- The crane shall be level within manufacturer-specified tolerances (typically ±1% unless otherwise specified).

## 5.2 Outrigger Load Calculations

Maximum outrigger load shall be calculated using manufacturer data or engineering analysis. Soil bearing capacity at the setup location shall be verified before crane operations begin.

# 6. Assembly and Disassembly

Crane assembly and disassembly shall be performed under the direction of a qualified and competent A/D (Assembly/Disassembly) Director in accordance with 29 CFR 1926.1404.

## 6.1 Requirements

- An A/D Director shall be designated for each assembly/disassembly operation.

- The A/D Director shall review the manufacturer's assembly/disassembly procedures before work begins.

- The crew shall be briefed on the sequence, hazards, and each person's role.

- Adequate clearance from power lines shall be maintained (see Section 9).

- Only personnel essential to the operation shall be in the work area.

- Wind speed limits for assembly/disassembly shall be per manufacturer requirements.

# 7. Operational Procedures

## 7.1 Pre-Lift Verification

Before any lift, the operator and lift supervisor shall verify:

- Load weight is known and documented (actual weight, not estimated).

- Rigging is inspected, rated for the load, and properly configured.

- Crane is level and set up per the load chart (outriggers fully extended and set, or on crawlers as specified).

- Load chart confirms adequate capacity for the maximum radius during the lift.

- Swing path is clear of personnel and obstructions.

- Signal person is assigned and positioned (required when operator cannot see the load or the point of landing).

- Tag lines are attached to control load rotation and swing.

## 7.2 During the Lift

- No personnel shall be under a suspended load at any time.

- The operator shall not leave the controls while the load is suspended.

- Loads shall not be swung over occupied areas.

- Multi-part line lifts shall be reeved per manufacturer requirements.

- The operator shall monitor the LMI (Load Moment Indicator) and rated capacity at all times.

## 7.3 Anti-Two-Block Systems

All mobile cranes shall be equipped with a functioning anti-two-block (A2B) device. The A2B system prevents the hook block from being drawn up into the boom tip sheaves, which can result in structural failure and dropped loads.

- The A2B warning and lockout system shall be tested daily before operations begin.

- The A2B system shall not be bypassed, disabled, or overridden at any time.

- If the A2B system activates during a lift, the operator shall stop all motions immediately and lower the load to the ground.

## 7.4 Wind Speed Limits

Crane operations shall cease when wind speeds exceed the following limits (or lower limits specified by the manufacturer):

| Condition | Maximum Wind Speed | Action Required |
| --- | --- | --- |
| Normal crane operations | 20 mph sustained (typical) | Monitor conditions; follow manufacturer limits |
| High-wind alert | 20-30 mph | Cease lifts with high-windage loads; secure boom; consult manufacturer charts |
| Crane shutdown | 30+ mph (or per manufacturer) | Cease all operations; boom down or secured per manufacturer procedure |
| Assembly / disassembly | Per manufacturer A/D procedures | Typically lower than operational limits |

NOTE: Manufacturer wind speed limits for the specific crane model always take precedence. Wind speed shall be measured at the work site using an anemometer.

# 8. Multi-Crane (Tandem) Lifts

Multi-crane lifts involve two or more cranes lifting a single load simultaneously. These are considered critical lifts and require additional planning and controls.

## 8.1 Requirements for Multi-Crane Lifts

- A detailed, written lift plan shall be prepared and approved by a qualified person (engineer or qualified lift planner).

- No crane shall be loaded beyond 75% of its rated capacity at any point during the lift.

- A single designated Lift Director shall coordinate all crane movements.

- All operators, riggers, and signal persons shall participate in a pre-lift meeting to review the lift plan, sequence, signals, and abort procedures.

- Communication between all parties shall be established and tested before the lift begins.

- Load distribution between cranes shall be calculated and verified.

- Abort/emergency procedures shall be established before the lift.

# 9. Power Line Clearance

Contact with energized power lines is one of the leading causes of crane fatalities. [COMPANY NAME] strictly enforces the following power line clearance distances per 29 CFR 1926.1408-1411:

## 9.1 Minimum Clearance Distances

| Voltage (kV) | Minimum Clearance Distance |
| --- | --- |
| Up to 50 kV | 10 feet (3.05 m) |
| Over 50 kV to 200 kV | 15 feet (4.60 m) |
| Over 200 kV to 350 kV | 20 feet (6.10 m) |
| Over 350 kV to 500 kV | 25 feet (7.62 m) |
| Over 500 kV to 750 kV | 35 feet (10.67 m) |
| Over 750 kV to 1,000 kV | 45 feet (13.72 m) |
| Over 1,000 kV | As established by utility owner/operator |

## 9.2 Power Line Safety Procedures

- Identify all overhead and underground power lines within the work area before operations begin.

- Assume all power lines are energized unless the utility owner confirms de-energization and grounding in writing.

- Maintain the minimum clearance distances (above) for all parts of the crane, load, load line, rigging, and tag lines.

- A dedicated spotter/signal person shall be assigned when operating within the Table A assessment distance.

- If the crane cannot maintain the required clearance, the power line must be de-energized and visibly grounded, or insulating barriers must be installed.

- Pre-lift planning shall include a power line hazard assessment for every lift within the work area.

NOTE: If any part of the crane, load line, or load contacts a power line, all personnel shall stay clear of the crane and load. The operator shall remain in the cab and attempt to swing or drive the crane clear. If unable to move clear, the operator shall remain in the cab until the utility confirms the line is de-energized.

# 10. Crane Inspection Requirements

[COMPANY NAME] shall ensure all mobile cranes are inspected in accordance with OSHA regulations and manufacturer requirements.

## 10.1 Inspection Categories

| Inspection Type | Frequency | Performed By | Key Items |
| --- | --- | --- | --- |
| Pre-Shift / Daily | Before each shift | Crane Operator | Controls, safety devices (LMI, A2B), wire rope, hydraulic systems, tires/tracks, fluid levels, structural components, load chart legibility |
| Monthly | Every 30 days | Competent Person | All daily items plus: hooks (for deformation, cracks), wire rope (reeving, end connections), sheaves, boom and jib structural members, outrigger components |
| Annual | Every 12 months | Qualified Inspector | Comprehensive inspection per manufacturer and ASME B30.5: structural, mechanical, electrical, hydraulic systems, safety devices calibration |
| Comprehensive | Per manufacturer interval (typically every 4 years or 8,000 hours) | Qualified Inspector / Manufacturer Rep | Disassembly-level inspection of structural and mechanical components. Non-destructive testing (NDT) of critical welds and pins. |

## 10.2 Deficiency Reporting

Any deficiencies identified during inspection shall be:

- Reported to the crane owner and operator immediately.

- Documented on the inspection form with the nature and severity of the deficiency.

- Classified as safety-critical (crane removed from service immediately) or non-critical (repair at next scheduled maintenance).

- Corrected before the crane is returned to service (for safety-critical items).

# 11. Training

[COMPANY NAME] shall ensure all personnel involved in crane operations are qualified and trained.

## 11.1 Operator Training

- All operators shall hold a valid, accredited crane operator certification.

- Familiarization training shall be provided for each specific crane make/model before the operator's first use.

- Annual refresher training on crane safety, load charts, and this program.

## 11.2 Rigger Training

- All riggers shall meet the qualified rigger criteria per 29 CFR 1926.1404.

- Training shall cover sling selection, load weight estimation, hitch types, rigging hardware inspection, and capacity calculations.

## 11.3 Signal Person Training

- All signal persons shall be qualified per 29 CFR 1926.1428.

- Training shall cover standard ASME hand signals, radio communication protocols, and positioning.

## 11.4 Awareness Training

All employees working in or around crane operations shall receive awareness-level training covering:

- Staying clear of the crane's swing radius and overhead loads

- Recognizing and avoiding struck-by hazards

- Power line awareness and clearance distances

- Emergency and evacuation procedures during crane operations

# 12. Recordkeeping and Documentation

[COMPANY NAME] shall maintain the following crane-related records:

- Operator certification copies (current and valid)

- Daily/shift crane inspection records (Appendix A)

- Monthly, annual, and comprehensive inspection reports

- Lift plans (routine and critical lifts – Appendix B)

- Crane maintenance and repair records

- Training records for operators, riggers, signal persons, and awareness training

- Incident and near-miss investigation reports

| Record Type | Retention Period |
| --- | --- |
| Daily inspection logs | Minimum 3 months (recommended 1 year) |
| Monthly inspections | Minimum 1 year |
| Annual / comprehensive inspections | Life of the crane |
| Lift plans | Duration of project + 1 year |
| Operator certifications | Duration of employment + 3 years |
| Training records | Duration of employment + 3 years |
| Incident reports | Minimum 5 years |

# Appendix A: Daily Crane Operator Inspection Checklist

[COMPANY NAME]

Daily / Pre-Shift Crane Inspection Checklist

Date: __________________________________________________

Crane Make / Model: __________________________________________________

Crane Serial / Unit Number: __________________________________________________

Operator Name: __________________________________________________

Operator Certification #: __________________________________________________

Project / Site Name: __________________________________________________

Hours on Meter (start of shift): __________________________________________________

Mark each item: ✓ = OK    X = Deficiency    N/A = Not Applicable

## Cab / Controls

☐  All controls labeled and functioning properly

☐  Cab windows clean and unobstructed

☐  Seat belt / operator restraint functional

☐  Horn / audible warning device working

☐  Fire extinguisher present and current inspection

☐  Load chart present, legible, and accessible

## Safety Devices

☐  LMI (Load Moment Indicator) tested and functional

☐  Anti-two-block (A2B) device tested and functional

☐  Boom angle / length indicator functional

☐  Outrigger / stabilizer controls functional

☐  Swing brake / lock functional

## Structural / Mechanical

☐  Boom sections – no visible damage, cracks, or deformation

☐  Jib (if applicable) – pins secured, no damage

☐  Turntable / swing bearing – no unusual play or noise

☐  Hook – safety latch present, no cracks or deformation, free rotation

☐  Sheaves – rotate freely, no cracks, rope guards in place

## Wire Rope

☐  No visible broken wires, kinking, crushing, or bird-caging

☐  Properly reeved per load chart

☐  End connections secure

☐  No excessive wear or corrosion

## Hydraulic / Pneumatic

☐  No visible leaks (hoses, fittings, cylinders)

☐  Hydraulic fluid level adequate

☐  Outrigger cylinders and pads in good condition

## Carrier / Travel System

☐  Engine oil, coolant, and fuel levels adequate

☐  Tires / tracks in good condition (pressure, wear, damage)

☐  Brakes functional (service and parking)

☐  Lights and reflectors working

☐  Counterweight secure and as specified

## Ground Conditions / Setup

☐  Ground firm, level, and adequate for crane loading

☐  Outriggers fully extended and set on adequate pads/cribbing

☐  Crane level within manufacturer tolerances

☐  Swing area clear of obstructions and personnel

☐  Power line clearance distances maintained

## Deficiencies / Comments

________________________________________________________________________

________________________________________________________________________

________________________________________________________________________

Crane Status:  ☐ Fit for Service    ☐ Restricted Use (explain)    ☐ Out of Service

________________________________________    ________________

Crane Operator Signature                                                            Date

________________________________________    ________________

Supervisor Acknowledgment                                                            Date

# Appendix B: Lift Plan Form

[COMPANY NAME]

Lift Plan Form

☐ Routine Lift    ☐ Critical Lift (≥75% capacity, multi-crane, personnel hoist, near power lines)

Date: __________________________________________________

Project / Site Name: __________________________________________________

Lift Description: __________________________________________________

Lift Director / Supervisor: __________________________________________________

## Crane Information

Crane Make / Model: __________________________________________________

Crane Capacity (max rated): __________________________________________________

Boom Length / Configuration: __________________________________________________

Counterweight Configuration: __________________________________________________

Outrigger Position (fully/partially extended): __________________________________________________

## Load Information

Load Description: __________________________________________________

Load Weight (verified): __________________________________________________

Rigging Weight: __________________________________________________

Total Lifted Weight (load + rigging + block): __________________________________________________

Pick Radius: __________________________________________________

Set Radius: __________________________________________________

Maximum Radius During Lift: __________________________________________________

Crane Capacity at Maximum Radius: __________________________________________________

Percent of Capacity (%): __________________________________________________

## Lift Environment

Wind Speed at Time of Lift: __________________________________________________

Maximum Allowable Wind Speed: __________________________________________________

Power Lines Present?  ☐ Yes   ☐ No

If Yes, Voltage: ________________  Clearance Distance: ________________

Ground Conditions: __________________________________________________

## Personnel

Crane Operator: __________________________________________________

Operator Cert #: __________________________________________________

Signal Person: __________________________________________________

Rigger(s): __________________________________________________

## Rigging Plan

Sling Type / Size / Capacity: __________________________________________________

Hitch Type (vertical, choker, basket): __________________________________________________

Shackles / Hardware: __________________________________________________

Spreader Bar / Below Hook Device (if applicable): __________________________________________________

## Pre-Lift Checklist

☐  Load weight verified (bill of lading, calculated, or weighed)

☐  Load chart reviewed – adequate capacity confirmed

☐  Rigging inspected and rated for load

☐  Crane inspected (daily inspection complete)

☐  Ground conditions assessed – crane level and stable

☐  Swing path clear of obstructions and personnel

☐  Power line clearance verified

☐  Signal person assigned and in position

☐  Tag lines attached

☐  Pre-lift meeting conducted with all personnel

☐  LMI / A2B systems tested and functional

## Special Conditions / Notes

________________________________________________________________________

________________________________________________________________________

________________________________________    ________________

Lift Director / Supervisor                                                            Date

________________________________________    ________________

Crane Operator                                                            Date

________________________________________    ________________

Safety Manager (Critical Lifts Only)                                                            Date