FORBES SAFETY SOLUTIONS

Hydrogen Sulfide (H2S) Safety Program

Exposure Prevention, Monitoring & Emergency Response

| Company: | [COMPANY NAME] |
| --- | --- |
| Prepared By: | [NAME / TITLE] |
| Effective Date: | [DATE] |
| Revision: | [REV NUMBER] |
| Regulatory Ref: | 29 CFR 1910.1000 / 29 CFR 1926.55 / ANSI Z390.1 |

This document is the property of [COMPANY NAME]. Unauthorized reproduction or distribution is prohibited.

# Table of Contents

# 1. Purpose & Scope

[COMPANY NAME] is committed to protecting employees from the hazards of hydrogen sulfide (H2S) exposure. This program establishes procedures for H2S detection, monitoring, personal protective equipment, emergency response, and training to ensure compliance with applicable standards.

This program applies to all employees, contractors, and visitors working at or visiting [COMPANY NAME] locations where H2S exposure is possible, including oil and gas production, wastewater treatment, confined space entry, and drilling operations.

# 2. Regulatory References

- 29 CFR 1910.1000 — Air Contaminants (General Industry)

- 29 CFR 1926.55 — Gases, Vapors, Fumes, Dusts, and Mists (Construction)

- ANSI Z390.1 — Accepted Practices for Hydrogen Sulfide (H2S) Training Programs

- NIOSH Pocket Guide to Chemical Hazards — Hydrogen Sulfide

- 29 CFR 1910.134 — Respiratory Protection

- 29 CFR 1910.146 — Permit-Required Confined Spaces

# 3. Properties of Hydrogen Sulfide

Understanding the physical and chemical properties of H2S is critical for safe operations:

| Property | Description |
| --- | --- |
| Chemical Formula | H₂S |
| Appearance | Colorless gas |
| Odor | "Rotten egg" smell at low concentrations — DO NOT rely on odor for detection |
| Vapor Density | 1.189 (heavier than air — accumulates in low-lying areas) |
| Flammability | Flammable — LEL: 4.0%, UEL: 44.0% |
| Solubility | Soluble in water; released when agitated or heated |
| Key Hazard | Olfactory fatigue — paralyzes sense of smell at ≥50 ppm |

NOTE: NEVER rely on your sense of smell to detect H2S. Olfactory fatigue occurs rapidly at higher concentrations, and the gas becomes odorless. Always use calibrated gas detection instruments.

# 4. Exposure Limits

| Standard | Limit | Concentration |
| --- | --- | --- |
| OSHA PEL (Ceiling) | Maximum at any time | 20 ppm |
| OSHA PEL (Peak) | 10-minute maximum | 50 ppm |
| NIOSH REL (Ceiling) | 10-minute ceiling | 10 ppm |
| ACGIH TLV-TWA | 8-hour average | 1 ppm |
| ACGIH TLV-STEL | 15-minute short-term | 5 ppm |
| IDLH | Immediately Dangerous to Life/Health | 100 ppm |

# 5. Health Effects by Concentration

The following table summarizes the health effects of H2S at various concentrations:

| Concentration (ppm) | Health Effects |
| --- | --- |
| 0.01 – 0.3 | Odor threshold — faint "rotten egg" smell detectable |
| 1 – 10 | Noticeable odor; eye irritation at upper range; headache with prolonged exposure |
| 10 – 50 | Strong offensive odor; eye and respiratory irritation; headache; nausea; dizziness |
| 50 – 100 | Olfactory fatigue (loss of smell); severe eye and respiratory irritation; pulmonary edema possible |
| 100 – 200 | IDLH zone; loss of smell immediate; severe respiratory distress; rapid loss of consciousness possible |
| 200 – 500 | Rapid unconsciousness; respiratory paralysis; death possible within minutes |
| 500 – 1000 | Immediate collapse; respiratory paralysis; death within minutes without rescue |
| 1000+ | Immediate death — single breath can be fatal |

# 6. Common Sources of H2S

## 6.1 Oil & Gas Operations

- Wellheads, flow lines, and production equipment

- Tank batteries and storage vessels

- Gas sweetening and sulfur recovery units

- Drilling operations (especially in H2S-bearing formations)

## 6.2 Other Sources

- Wastewater treatment plants and collection systems

- Confined spaces (manholes, vaults, pits, tanks)

- Landfills and waste disposal facilities

- Pulp and paper mills

- Agricultural operations (manure pits, silos)

- Geothermal areas and hot springs

# 7. Roles & Responsibilities

## 7.1 Management

- Ensure H2S hazard assessments are conducted for all applicable work locations

- Provide monitoring equipment, respiratory protection, and rescue equipment

- Establish H2S danger zones, alarm systems, and evacuation routes

- Ensure adequate staffing for buddy system compliance

## 7.2 Supervisors

- Verify atmospheric monitoring before and during H2S operations

- Enforce buddy system, PPE requirements, and wind direction awareness

- Ensure all personnel on-site are H2S-trained before work begins

- Brief all workers on evacuation routes, alarm signals, and assembly points

## 7.3 Employees

- Attend and complete required H2S safety training

- Use personal and area monitors as required

- Know evacuation routes and emergency assembly points

- Never enter an H2S-contaminated area without proper PPE and authorization

- Report all alarms, leaks, and suspected H2S exposures immediately

# 8. Atmospheric Monitoring

## 8.1 Equipment Requirements

- Calibrated personal H2S monitors shall be worn by all personnel in H2S areas

- Area monitors shall be placed at strategic locations around work zones

- Monitors shall be bump-tested daily and calibrated per manufacturer specifications

- Spare batteries and sensor replacements shall be available on-site

## 8.2 Alarm Set Points

| Alarm Level | Concentration | Action |
| --- | --- | --- |
| Low Alarm | 10 ppm | Alert — increase awareness, check wind direction, prepare to evacuate |
| High Alarm | 20 ppm (PEL Ceiling) | Evacuate — move upwind and crosswind to assembly point |
| TWA Alarm | 10 ppm (8-hr average) | Evacuate — cumulative exposure limit reached |

## 8.3 Wind Direction Awareness

- Windsocks shall be installed and visible at all H2S work locations

- Workers shall be aware of wind direction at all times

- Evacuation routes shall be upwind and crosswind of the H2S source

- Briefings shall include wind direction and potential changes

# 9. Buddy System

No person shall work alone in areas where H2S concentrations may exceed 10 ppm:

- Minimum two-person buddy system required in all H2S danger areas

- Each buddy must maintain visual and/or voice contact

- Each buddy must carry a personal H2S monitor and have access to escape SCBA

- Buddy pairs shall monitor each other for signs of H2S exposure

# 10. Escape Route Planning

- Primary and secondary evacuation routes shall be established and communicated

- Routes shall be upwind and/or crosswind from potential H2S sources

- Evacuation maps posted at visible locations around the work area

- Wind direction indicators (windsocks) at assembly points

- Muster/assembly points at safe distances upwind of the source

- Regular drills to ensure all personnel are familiar with evacuation procedures

# 11. Respiratory Protection

Respiratory protection for H2S shall follow this hierarchy:

| Exposure Level | Required Respiratory Protection |
| --- | --- |
| Below 10 ppm | No respiratory protection required (monitoring continues) |
| 10 – 50 ppm | Full-face APR with appropriate cartridges (only for escape, not for entry) |
| 50 – 100 ppm | Full-face SAR (Supplied Air Respirator) in pressure-demand mode |
| Above 100 ppm (IDLH) | Full-face pressure-demand SCBA or combination SCBA/SAR |
| Unknown concentration | Full-face pressure-demand SCBA |

NOTE: APRs (Air Purifying Respirators) do NOT protect against IDLH concentrations. SCBA is mandatory for concentrations above 100 ppm or unknown atmospheres.

Escape-only SCBA (5–10 minute units) shall be readily available at all H2S work locations.

# 12. Rescue Procedures

In the event of an H2S exposure emergency:

## 12.1 Self-Rescue

- Hold your breath, don escape SCBA if immediately available

- Move upwind and crosswind to the designated assembly point

- Alert others by radio, horn, or alarm

- Do NOT re-enter the area to assist others unless wearing SCBA

## 12.2 Rescue of Others

IMPORTANT: Attempting rescue without proper SCBA is the leading cause of multiple H2S fatalities.

- Do NOT enter an H2S atmosphere without full-face pressure-demand SCBA

- Contact emergency services (911) immediately

- Only trained rescue personnel wearing SCBA shall attempt rescue

- Remove the victim to fresh air upwind of the source

- Begin CPR if the victim is not breathing (H2S may cause respiratory arrest)

- Administer supplemental oxygen if trained and equipment is available

- Stay with the victim until EMS arrives

# 13. Medical Surveillance

Medical surveillance shall be provided for employees routinely working in H2S environments:

- Pre-assignment baseline health assessment

- Annual medical evaluation (focused on respiratory and neurological function)

- Post-exposure medical evaluation following any exposure above the PEL

- Pulmonary function tests, chest X-ray, and neurological screening as directed by the physician

- Medical records retained for duration of employment plus 30 years

# 14. Training Requirements

## 14.1 Initial Training (ANSI Z390.1 Compliant)

All employees who may be exposed to H2S shall complete initial training covering:

- Properties, sources, and health hazards of H2S

- Exposure limits and effects at various concentrations

- Detection and monitoring equipment operation

- Respiratory protection selection and use

- Buddy system and wind direction awareness

- Evacuation routes and alarm procedures

- Rescue procedures including CPR and first aid

- Hands-on equipment use (monitor, SCBA donning/doffing)

## 14.2 Refresher Training

- Annual refresher training for all H2S-exposed employees

- Retraining when job conditions change or new H2S sources are identified

- Retraining after any H2S exposure incident

# 15. Recordkeeping

- Atmospheric monitoring records — retained for 30 years

- Personal exposure monitoring results — retained for 30 years

- Medical surveillance records — retained for duration of employment plus 30 years

- Training records — retained for duration of employment plus 3 years

- Equipment calibration and maintenance records — retained for 3 years

- Incident and near-miss reports — retained for 5 years

# 16. Program Review

This program shall be reviewed annually or after any H2S exposure incident, near-miss, or change in operations that may affect H2S exposure risk.

# Appendix A: H2S Monitoring Log

Use this log to document atmospheric monitoring results at H2S work locations.

Site / Location: ______________________________     Date: ____________

Instrument Type: ______________________________     Serial #: ____________

Last Calibration Date: ____________     Bump Test: ☐ Pass  ☐ Fail

| Time | Location / Area | H2S (ppm) | Wind Dir. | Action Taken | Initials |
| --- | --- | --- | --- | --- | --- |

Monitor: ______________________________     Reviewed By: ______________________________

# Appendix B: H2S Emergency Action Checklist

Post at all H2S work locations. Review with all personnel before work begins.

IF H2S ALARM SOUNDS OR H2S IS DETECTED:

| Step | Action | ✓ |
| --- | --- | --- |
| 1 | STOP work immediately | ☐ |
| 2 | Hold your breath — don escape SCBA if immediately available | ☐ |
| 3 | Move UPWIND and CROSSWIND to the assembly point | ☐ |
| 4 | Alert others by radio, horn, or verbal warning | ☐ |
| 5 | Account for all personnel at assembly point (buddy check) | ☐ |
| 6 | Call 911 and notify the Site Supervisor | ☐ |
| 7 | DO NOT re-enter the area without SCBA | ☐ |
| 8 | If rescue needed — only trained rescuers with SCBA | ☐ |
| 9 | Administer first aid / CPR if trained | ☐ |
| 10 | Remain at assembly point until "All Clear" is given | ☐ |

EMERGENCY CONTACTS:

Emergency Services: 911

Site Supervisor: ______________________________     Phone: ____________

Safety Department: ______________________________     Phone: ____________

Poison Control: 1-800-222-1222

Assembly Point Location: ______________________________

Primary Evacuation Route: ______________________________

Secondary Evacuation Route: ______________________________

Wind Direction (current): ____________     Windsock Location: ____________