JOB HAZARD ANALYSIS

Directional Drilling / Boring

Regulatory References: 29 CFR 1926.651, CGA Best Practices, 811 Call Before You Dig

PROJECT INFORMATION

| Project Name: |  | Date: |  |
| --- | --- | --- | --- |
| Location: |  | JHA Number: | JHA-032 |
| Supervisor: |  | Reviewed By: |  |
| Trade/Activity: | Directional Drilling / Boring | Permit Required: | ☐ Yes ☐ No |

JOB HAZARD ANALYSIS

| Task Step | Hazard | Risk Level | Control Measures |
| --- | --- | --- | --- |
| 1. Utility Locate Verification | Striking unmarked utilities Inaccurate locate markings Incomplete records | H H M | Verify 811 ticket is current and all utilities have responded Pothole/vacuum excavate to physically expose utilities within 5 ft of bore path Compare field marks against utility maps and as-built drawings Mark all exposed utilities with flags/stakes and record depths If discrepancies found, stop work and re-request locates Use ground-penetrating radar (GPR) for added verification in congested areas |
| 2. Drill Rig Setup | Heavy equipment tip-over Caught-in/between Noise exposure Traffic hazards | H M M H | Position rig on firm, level ground; use crane mats if needed Establish traffic control plan with signs, cones, flaggers per MUTCD Set exclusion zone around rig — minimum 10 ft from rotating components Inspect all rig safety guards, E-stops, and shutdown devices Verify pipe rack is secure and pipe handling area is clear Conduct safety briefing with all crew before startup |
| 3. Pilot Bore | Striking existing utilities Drilling fluid release (frac-out) Entanglement in drill string | H H H | Use electronic locating/sonde to track bore head position and depth continuously Monitor drilling fluid pressure — do NOT exceed formation fracture pressure Maintain 2 ft minimum separation from known utilities Station spotter at bore path to watch for surface heaving or fluid emergence Never reach into rotating drill string; use wrenches designed for breakout Wear hearing protection; rig produces 85-95 dB |
| 4. Back-Ream | Entanglement Fluid spill High torque / sudden release Noise | H M M M | Ensure reamer guards are installed and rotating parts are shielded Maintain safe distance from reamer and drill string during rotation Monitor mud returns — loss of returns indicates potential frac-out Use proper pipe handling tools (no hands near threads under torque) Keep mud containment pits/tanks maintained and not overflowing Have vacuum truck on standby for fluid management |
| 5. Product Pullback | Pipe whip/snap Heavy load handling Trench collapse at exit pit Entanglement | H M H H | Inspect swivel, pulling head, and product pipe connections before pullback Keep personnel clear of pipe under tension — establish exclusion zone Shore or slope exit pit per 29 CFR 1926.652 Use tag lines on suspended pipe — never guide by hand near rotating joints Monitor pull force — stop if resistance exceeds design parameters Spotter at exit pit to guide product emergence |
| 6. Cleanup & Demobilization | Slips/trips on drilling mud Heavy equipment movement Environmental contamination | M M M | Pump out and properly dispose of all drilling fluid per local regulations Backfill entry/exit pits and compact to original grade Clean up all spilled drilling fluid from surrounding areas Restore ground surface, seed/sod as required by permit Secure all equipment for transport; check load securement Complete as-built documentation with bore depth and alignment records |

Additional Notes: All utility locates must be verified and valid (within 10-14 business days depending on jurisdiction). Pothole/vacuum excavate to expose all utilities within 5 ft of bore path. Maintain bore logs with depth and fluid pressure records.

PERSONAL PROTECTIVE EQUIPMENT (PPE)

| Required PPE |  |
| --- | --- |
| ☑ Hard Hat (ANSI Z89.1) | ☑ Safety Glasses (ANSI Z87.1) |
| ☑ High-Visibility Vest (Class 2/3) | ☑ Steel/Composite Toe Boots |
| ☑ Hearing Protection (NRR 25+) | ☑ Cut-Resistant Gloves |
| ☑ Face Shield (during fluid ops) | ☑ Rubber Boots (wet conditions) |

EMERGENCY PROCEDURES

• Utility strike (gas): Evacuate area immediately. Do NOT operate switches/ignition. Call 911 and gas utility emergency line.

• Utility strike (electric): Stay clear of equipment. Assume all ground in area is energized. Call 911 and utility provider.

• Utility strike (water/sewer): Shut down drill. Notify utility. Contain flow to prevent environmental damage.

• Drilling fluid release (frac-out): Stop drilling immediately. Contain fluid with berms/vacuum truck. Notify environmental coordinator.

• Entanglement injury: Activate E-stop on drill rig immediately. Do NOT attempt to reverse rotation. Call 911.

• Report all incidents, near-misses, and utility contacts to supervisor and 811 immediately.

WORKER SIGN-OFF

By signing below, I acknowledge that I have reviewed and understand this Job Hazard Analysis, the associated hazards, and the required control measures for the work to be performed.

| # | Print Name | Signature | Company | Date | Time |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |
| 6 |  |  |  |  |  |
| 7 |  |  |  |  |  |
| 8 |  |  |  |  |  |
| 9 |  |  |  |  |  |
| 10 |  |  |  |  |  |

REVIEW & APPROVAL

| Role | Name / Signature | Title | Date |
| --- | --- | --- | --- |
| Prepared By |  |  |  |
| Reviewed By |  |  |  |
| Approved By |  |  |  |
| Site Supervisor |  |  |  |

REVISION HISTORY

| Rev. | Date | Description | Author |
| --- | --- | --- | --- |
| 0 |  | Initial Release |  |
| 1 |  |  |  |