[  COMPANY LOGO  ]

JOB HAZARD ANALYSIS

Excavation & Trenching Operations

Applicable Standards: 29 CFR 1926 Subpart P (1926.650–1926.652); OSHA Excavation Standard; 29 CFR 1926.651(c) — Utility Locates

| Project Name: ____________________________ | Date: ____________________________ |
| --- | --- |
| Location: ____________________________ | Supervisor: ____________________________ |
| Crew Members: _______________________________________________________________________________ |  |

| Step # | Job Steps | Potential Hazards | Hazard Controls | Responsible Person |
| --- | --- | --- | --- | --- |
| 1 | Conduct pre-excavation site survey and review utility locate markings (811 call confirmation) | Utility strikes (gas, electric, water, fiber); unmarked underground hazards | Verify 811 utility locate completed (min. 48 hrs prior). Review as-built drawings. Hand-dig within 18" of marked utilities. Use ground-penetrating radar for unknown areas. |  |
| 2 | Establish excavation perimeter barricades and traffic control | Struck-by from vehicles or mobile equipment; unauthorized entry into excavation zone | Install high-visibility barricades and warning signs per MUTCD standards. Assign flaggers for active roadways. Maintain 6-ft setback for spoil piles from trench edge. |  |
| 3 | Perform soil classification by competent person (visual and manual tests) | Incorrect soil classification leading to inadequate protective systems; cave-in risk | Competent person conducts thumb penetration, plasticity, and dry strength tests per 29 CFR 1926.652 Appendix A. Document soil type (A, B, or C) in writing. Reassess after rain events. |  |
| 4 | Excavate trench using hydraulic excavator | Equipment tip-over; struck-by from bucket swing; overhead power line contact | Ensure operator is trained/certified. Maintain 10-ft clearance from power lines (20-ft for >350kV). No workers in swing radius. Ground guide with radio communication required. Spotters for blind spots. |  |
| 5 | Install protective system — shoring, shielding, or sloping per soil classification | Cave-in / trench collapse; engulfment; crushing injuries | Install system before any worker enters trench >5 ft deep (OSHA 1926.652). Use tabulated data for shoring specs. Hydraulic shores installed from top down. Trench boxes rated for soil type and depth. |  |
| 6 | Place access/egress ladders within 25 ft of lateral travel for all workers | Inability to escape during emergency; falls from ladder; ladder displacement | Position ladders per 1926.651(c)(2) — max 25 ft travel distance. Ladders extend 3 ft above trench edge. Secure at top to prevent displacement. Inspect daily before use. |  |
| 7 | Monitor atmospheric conditions in trench (>4 ft deep) | Oxygen deficiency; toxic gases (H₂S, CO, methane); hazardous atmosphere from nearby operations | Continuous atmospheric monitoring with 4-gas meter (O₂, LEL, CO, H₂S). Calibrate instrument daily. Ventilate with mechanical blowers if readings are abnormal. Rescue plan and retrieval system in place. |  |
| 8 | Perform work activities within the trench (pipe installation, utility repair, etc.) | Struck-by from falling materials/tools; manual handling injuries; water accumulation | Keep materials/tools 2 ft back from trench edge. Use buckets or tool lanyards to lower items. Pump standing water continuously. No workers under suspended loads. Competent person inspects trench daily and after weather events. |  |
| 9 | Backfill operations and compaction | Workers caught between equipment and trench wall; uncontrolled soil movement; vibration exposure | Clear all personnel from trench before backfilling. Remove shoring bottom-up. Backfill in lifts and compact per engineer specs. Operator maintains visual contact with spotter. Anti-vibration gloves for compaction equipment. |  |
| 10 | Demobilize equipment and restore site | Struck-by during equipment movement; unguarded open excavations; trip hazards | Barricade any open excavations left overnight with steel plates or fencing. Restore traffic control. Final inspection by competent person. Document JHA completion and any incidents. |  |

⚠ This JHA must be reviewed and signed by all crew members before work begins. Conditions on the jobsite may change — reassess hazards throughout the shift and update this JHA as necessary.

REVIEW & APPROVAL

| ________________________________________ Reviewed By (Print Name) | ________________________________________ Signature | ________________________________________ Date |
| --- | --- | --- |