FORBES SAFETY SOLUTIONS

JOB HAZARD ANALYSIS

Pipeline / Underground Utility Installation

Applicable Standards: 29 CFR 1926 Subpart P (Excavations) • 29 CFR 1926.651 (General Requirements) • 29 CFR 1926.652 (Protective Systems) • CGA Best Practices (Common Ground Alliance 811) • MUTCD (Traffic Control) • 29 CFR 1926.353 (Ventilation for Welding) • 29 CFR 1910.146 (Permit-Required Confined Spaces)

PROJECT INFORMATION

| Company Name: |  | Date: |  |
| --- | --- | --- | --- |
| Project/Site: |  | JHA Number: |  |
| Supervisor: |  | Work Activity: |  |
| Location/Address: |  | Permit #: |  |

JOB HAZARD ANALYSIS — TASK BREAKDOWN

| TASK STEP | HAZARD | RISK (H/M/L) | CONTROL MEASURES |
| --- | --- | --- | --- |
| 1. Utility Locate & Potholing (811 Verification) | • Struck-by existing utilities (gas, electric, fiber, water) • Electrocution from unmarked power lines • Damage to pressurized gas line • Inadequate locate markings | H | • Call 811 minimum 48–72 hours before excavation — verify ticket is active and valid • Obtain utility locate maps and cross-reference with as-built drawings • Pothole (soft-dig) with vacuum excavation or hand tools within 24" of marked utilities • Competent person verifies all locates before mechanical excavation • Mark tolerance zones with white paint per CGA Best Practices • Photograph all markings before work begins for documentation |
| 2. Excavation & Trench Setup | • Cave-in / trench collapse • Engulfment • Falls into excavation • Hazardous atmosphere (methane, CO, H₂S, low O₂) • Contact with groundwater • Heavy equipment struck-by | H | • Competent person performs soil classification per 29 CFR 1926.651 • Install trench protective system: sloping, benching, shoring, or trench shield per soil type • Excavations >4 ft. deep require protective system; >20 ft. requires PE-stamped design • Atmospheric monitoring before entry and continuously in deep trenches (>4 ft.) • Install barricades, guardrails, and warning signs around excavation perimeter • Ladders within 25 ft. of all workers for emergency egress • Spotter required for all heavy equipment near trench edge • Daily trench inspection by competent person; re-inspect after rain or changed conditions |
| 3. Pipe Laying / Welding | • Struck-by pipe or materials during lifting • Caught-between during pipe placement • Welding burns and UV radiation • Fire/explosion from hot work near gas utilities • Ergonomic strain (heavy lifting) • Fume inhalation | H | • Rigging inspection before each lift — competent rigger directs crane/excavator • Tag lines on all suspended loads; no workers under suspended pipe • Hot work permit required for all welding/cutting operations • Welding screens/curtains to protect nearby workers from UV/sparks • Fire watch with extinguisher during and 30 min. after hot work • Welding fume extraction or natural ventilation for trench welding • Proper lifting techniques; mechanical assists for pipe >50 lbs. • Gas detection monitor in continuous operation near gas utilities |
| 4. Backfill & Compaction | • Struck-by heavy equipment • Crushing/caught-between compaction equipment and trench walls • Damaged pipe from improper backfill • Noise exposure >85 dBA • Whole-body vibration | M | • Spotter for all heavy equipment operations near trench • Verify no workers in trench during machine backfill operations • Follow engineered backfill specifications (lift thickness, material type) • Compaction testing per project specifications • Hearing protection required near compaction equipment • Anti-vibration gloves and equipment operator rest intervals • Maintain minimum safe distance from trench edge for equipment (per soil type) |
| 5. Pressure Testing | • High-pressure line failure / rupture • Projectile hazard from fitting failure • Exposure to test medium (water, air, or inert gas) • Noise from pressurized release | H | • Written pressure test procedure approved by engineer • Establish exclusion zone — no personnel within blast radius during pressurization • Inspect all fittings, joints, and connections before test initiation • Gradual pressurization in stages with hold periods per specification • Calibrated pressure gauges — minimum two independent gauges • Bleed pressure safely before disconnecting any fittings • Hearing protection during controlled pressure release • Emergency shutoff valve accessible at all times |
| 6. Site Restoration & Demobilization | • Traffic exposure on roadways • Uneven ground / trip hazards • Remaining excavation openings • Improper waste disposal | M | • Traffic control plan (TTC) per MUTCD — flaggers, cones, signs, arrow boards • High-visibility Class 3 vests for all workers near traffic • Verify all excavations backfilled and compacted to grade • Install permanent surface restoration (asphalt, concrete, sod) per permit requirements • Remove all construction debris and dispose per environmental regulations • Conduct final site walk-through with supervisor — document with photos • Update as-built drawings with actual utility locations |

FORBES SAFETY SOLUTIONS

PERSONAL PROTECTIVE EQUIPMENT (PPE) REQUIREMENTS

| PPE ITEM | SPECIFICATION / STANDARD | WHEN REQUIRED |
| --- | --- | --- |
| Hard Hat | ANSI/ISEA Z89.1, Type I, Class E (electrical rated) | All times within excavation work zone |
| High-Visibility Vest | ANSI/ISEA 107, Class 3 (for roadway/traffic exposure) | All workers on site; Class 3 required near traffic |
| Steel/Composite Toe Boots | ASTM F2413 — puncture-resistant, metatarsal guard recommended | All times on site; waterproof if groundwater present |
| Safety Glasses | ANSI Z87.1 — impact-rated, anti-fog | All times; goggles for grinding, cutting, or dusty conditions |
| Hearing Protection | ANSI S3.19 — NRR 25+ dB (plugs or muffs) | Near heavy equipment, compaction, or pneumatic tools (>85 dBA) |
| Cut-Resistant Gloves | ANSI/ISEA 105, Cut Level A4 minimum | Pipe handling, material moving, general excavation tasks |
| Welding Hood / Shield | ANSI Z87.1, Shade 10-12 for arc welding | All welding and cutting operations |
| Welding Gloves | ANSI/ISEA 105 — heat and flame resistant | All welding and hot work operations |
| Respiratory Protection | NIOSH-approved P100 or supplied air as required | Trench atmospheres with contaminants; welding fume exposure |
| Fall Protection Harness | ANSI Z359.11 — full body harness with lanyard | Work adjacent to excavations >6 ft. deep without guardrails |
| Gas Detection Monitor | 4-gas monitor: O₂, LEL, CO, H₂S — bump tested daily | All personnel entering excavations >4 ft. deep |

EMERGENCY PROCEDURES

EMERGENCY PROCEDURES

Trench Rescue: If a cave-in occurs, call 911 immediately. Do NOT enter the trench to rescue — this is a confined space rescue requiring trained technical rescue teams. Protect remaining trench walls from further collapse.

Struck Utility — Gas: Evacuate upwind immediately. Establish 300 ft. exclusion zone. Call gas company emergency line. Do NOT attempt to cap or repair. Eliminate ignition sources.

Struck Utility — Electric: Assume all conductors are energized. Do NOT touch the line or equipment in contact with it. Call utility emergency number. Establish exclusion zone equal to wire span length.

Hazardous Atmosphere: Remove worker from trench immediately if gas monitor alarms. Ventilate with mechanical blower. Do not re-enter until atmosphere is confirmed safe by competent person with calibrated monitor.

Medical Emergency: Administer first aid. Call 911 and provide exact location and nature of injury. Designate someone to guide emergency vehicles to site. First aid kit and AED on site at all times.

Emergency Contacts: Competent Person: _____________ ¦ 811 Ticket #: _____________ ¦ Local EMS: 911 ¦ Gas Co. Emergency: _____________ ¦ Electric Co. Emergency: _____________

FORBES SAFETY SOLUTIONS

WORKER ACKNOWLEDGMENT & SIGN-OFF

By signing below, I acknowledge that I have reviewed this JHA, understand the identified hazards and control measures, and agree to follow all safety procedures outlined herein. I have been trained on the use of required PPE and emergency procedures for this work activity.

| # | PRINTED NAME | SIGNATURE | DATE |
| --- | --- | --- | --- |
| 1 |  |  |  |
| 2 |  |  |  |
| 3 |  |  |  |
| 4 |  |  |  |
| 5 |  |  |  |
| 6 |  |  |  |
| 7 |  |  |  |
| 8 |  |  |  |
| 9 |  |  |  |
| 10 |  |  |  |

JHA REVIEW & APPROVAL

This JHA must be reviewed and approved before work commences. Review is required whenever scope, conditions, personnel, or site conditions change. The competent person must re-assess the trench and protective systems at the start of each shift.

| ROLE | NAME | SIGNATURE | DATE |
| --- | --- | --- | --- |
| Prepared By (Safety Professional) |  |  |  |
| Reviewed By (Competent Person) |  |  |  |
| Approved By (Project Manager) |  |  |  |
| Approved By (HSE Manager) |  |  |  |

REVISION HISTORY

| REV # | DATE | DESCRIPTION | REVISED BY |
| --- | --- | --- | --- |

DISCLAIMER: This JHA template is provided as a guide for safety planning and does not replace site-specific hazard assessments. Employers are responsible for ensuring compliance with all applicable OSHA standards, state and local regulations, utility owner requirements, and company-specific safety programs. The competent person must adapt this document to reflect actual site conditions, soil classification, protective systems, and personnel qualifications.