[  COMPANY LOGO  ]

JOB HAZARD ANALYSIS

Roofing Operations — Commercial & Residential

Applicable Standards: 29 CFR 1926 Subpart M (1926.500–1926.503) — Fall Protection; 29 CFR 1926.501(b)(10) — Roofing; OSHA 3150 — Fall Protection in Construction

| Project Name: ____________________________ | Date: ____________________________ |
| --- | --- |
| Location: ____________________________ | Supervisor: ____________________________ |
| Crew Members: _______________________________________________________________________________ |  |

| Step # | Job Steps | Potential Hazards | Hazard Controls | Responsible Person |
| --- | --- | --- | --- | --- |
| 1 | Establish roof access via ladder, scaffold, or aerial lift | Falls from height during access/egress; ladder tip-over or displacement; overreaching | Extension ladder extends 3 ft above roof edge. Secure at top and base. 4:1 ratio setup. Use ladder stabilizer on soft ground. Three-point contact rule enforced. Workers wear 100% tie-off harness when transitioning to roof. |  |
| 2 | Install perimeter fall protection (guardrails, warning line system, or personal fall arrest) | Unprotected roof edge — falls to lower level (leading cause of roofing fatalities) | Install guardrails meeting 1926.502(b) — 42" top rail, 21" mid rail, capable of 200 lb force. For low-slope roofs: warning line at 6 ft from edge + safety monitor OR personal fall arrest system. Anchor points rated 5,000 lbs per worker. |  |
| 3 | Hoist materials to roof level using crane, conveyor, or manual methods | Struck-by from falling materials; overloaded hoist; musculoskeletal injuries from manual lifting | Establish exclusion zone below hoisting area. Tag loads — no riding loads. Inspect rigging hardware pre-use. Limit manual carries to <50 lbs per person. Two-person lift for bundles. Hard hats mandatory in hoist zone. |  |
| 4 | Remove existing roofing materials (tear-off operations) | Falls through weakened decking or skylights; exposure to asbestos/lead in older buildings; nail puncture wounds | Survey for fragile decking and skylights — cover or guard per 1926.502(i). Pre-demolition hazmat survey for buildings pre-1980. Cut-resistant gloves and puncture-resistant boot insoles. Debris chute or controlled drop zone for waste. |  |
| 5 | Inspect and repair roof deck substrate | Structural collapse of weakened deck; falls through openings; splinter/cut injuries | Competent person evaluates structural integrity before loading. Cover all deck openings — secure covers labeled "HOLE" per 1926.502(i). Replace rotted decking before proceeding. Maximum material staging load per structural engineer limits. |  |
| 6 | Install underlayment, insulation, and vapor barrier | Heat illness (especially on dark roofs in summer); slips on smooth membrane; adhesive/solvent fume inhalation | Implement heat illness prevention plan — water, rest, shade per OSHA guidance. Cool-down breaks every 20 min when >90°F. Non-slip footwear required. Half-face respirator with OV cartridge for adhesive application. Ventilation for enclosed areas. |  |
| 7 | Install primary roofing system (shingles, membrane, or metal panels) | Repetitive motion injuries; nail gun injuries; burns from hot-applied materials (BUR/modified bitumen); electrical contact with rooftop equipment | Sequential trigger on nail guns only (no bump firing). Hot kettle operators trained and certified — fire extinguisher within 25 ft. Maintain 10-ft clearance from electrical equipment. Rotate task assignments to reduce repetitive strain. |  |
| 8 | Perform edge and flashing detail work (drip edge, valley, penetrations) | Falls from roof edge during low-position work; lacerations from sheet metal; dropped tools striking workers below | 100% personal fall arrest at roof edge — 6-ft max free fall. Use retractable SRL when working near edge. Cut-resistant gloves for metal flashing. Tool lanyards on all hand tools. Drop zone barricaded below. |  |
| 9 | Conduct final inspection, cleanup, and debris removal | Puncture wounds from exposed fasteners; trip hazards from debris; struck-by during waste removal | Magnetic nail sweeper on deck and ground level. Bundle debris for controlled lowering — no throwing materials. Maintain fall protection until last worker descends. Final walkthrough by supervisor. Document punch list items. |  |

⚠ This JHA must be reviewed and signed by all crew members before work begins. Conditions on the jobsite may change — reassess hazards throughout the shift and update this JHA as necessary.

REVIEW & APPROVAL

| ________________________________________ Reviewed By (Print Name) | ________________________________________ Signature | ________________________________________ Date |
| --- | --- | --- |