FORBES SAFETY SOLUTIONS

JOB HAZARD ANALYSIS

| JHA INFORMATION |  |  |  |
| --- | --- | --- | --- |
| JHA Title: | Scaffolding Erection & Dismantling | JHA Number: | FSS-JHA-029 |
| Project Name: |  | Date: |  |
| Location: |  | Revision: | 0 |
| Regulatory References: | 29 CFR 1926.451 (General Requirements — Scaffolds); 29 CFR 1926.452 (Specific Types); 29 CFR 1926.453 (Aerial Lifts); 29 CFR 1926.454 (Training) |  |  |
| Prepared By: |  | Approved By: |  |
| Competent Person: |  | Supervisor: |  |

JOB HAZARD ANALYSIS — TASK BREAKDOWN

| Task Step | Hazard | Risk Level (H/M/L) | Control Measures |
| --- | --- | --- | --- |
| 1. Site Assessment & Base Preparation | • Unstable ground conditions • Underground utilities • Overhead power line proximity • Uneven or sloped terrain • Pedestrian/vehicle traffic | H | • Competent person conducts site survey before erection begins • Verify soil bearing capacity at scaffold location — compact or improve as needed • Call 811 / utility locate to identify underground utilities • Survey for overhead power lines — maintain minimum 10 ft clearance (20 ft for >50kV) • Level ground surface; use compacted gravel fill if needed • Barricade scaffold zone to restrict unauthorized access • Establish traffic control plan if near vehicle routes |
| 2. Mud Sill / Base Plate Placement | • Point loading causing settlement • Base plate sliding/shifting • Uneven load distribution • Improper sill sizing | M | • Place mud sills (min 2x10 lumber or engineered sills) under all base plates • Mud sills must extend minimum 6 inches beyond base plate on each side • Ensure base plates are centered on mud sills • Verify level with spirit level — shim as needed (no scrap wood for shimming) • Never use concrete blocks, bricks, or unstable materials as base support • Lock screw jacks after leveling — do not exceed maximum extension |
| 3. Frame Erection & Bracing | • Falls during erection (no guardrails yet) • Scaffold collapse from improper bracing • Struck by falling components • Manual handling injuries | H | • Erectors must use personal fall arrest systems (harness + lanyard) until guardrails installed • Install cross-bracing on every frame bay — both sides • Plumb and level each lift before proceeding to next tier • Use manufacturer's coupling/locking pins — no substitutions • Two-person minimum for frame erection — use tag lines for heavy components • Secure components against wind during erection (tie-off partially erected sections) • Install tie-ins to structure per manufacturer specs (every 26 ft vertical, 30 ft horizontal) |
| 4. Platform Installation & Guardrails | • Falls from incomplete platforms • Planks sliding or tipping • Gaps between planks > 1 inch • Missing guardrail components | H | • Install scaffold planks full width — maximum 1-inch gap between planks • Planks must extend minimum 6 inches beyond support, maximum 12 inches • Secure planks to prevent displacement (cleats, hooks, or wire ties) • Install guardrails: top rail at 38-45 inches, mid rail, and toe board (4 inches min) • Guardrail must withstand 200 lb force at any point in any direction • Install access ladder or stair tower — no climbing on cross-braces • Inspect all planks for defects — reject cracked, warped, or damaged planks |
| 5. Scaffold Inspection & Tagging | • Use of uninspected scaffold • Overloading beyond rated capacity • Defects missed during inspection • Unauthorized modifications | M | • Competent person inspects scaffold before each work shift • Apply scaffold tag system: GREEN (safe), YELLOW (restrictions), RED (do not use) • Document inspection findings on scaffold inspection form • Post load capacity signage on scaffold (must not exceed 4x max intended load) • Re-inspect after any alteration, weather event, or extended non-use • Do not modify scaffold without competent person approval • Train all scaffold users on tag system and load limits per 1926.454 |
| 6. Dismantling (Reverse Sequence) | • Falls during dismantling • Scaffold instability as components removed • Struck by falling components • Premature removal of ties/bracing | H | • Dismantle in reverse order of erection — top down • Erectors wear personal fall arrest at all times during dismantling • Do not remove tie-ins until dismantling reaches that level • Lower components to ground — never throw or drop • Maintain bracing integrity at each stage of dismantling • Barricade exclusion zone below dismantling operations • Stack and organize components for transport/storage upon removal • Competent person supervises entire dismantling operation |

REQUIRED PERSONAL PROTECTIVE EQUIPMENT (PPE)

| PPE Required | Specification / ANSI Standard | Condition / Notes |
| --- | --- | --- |
| Hard Hat | ANSI/ISEA Z89.1, Type I, Class E | Chin strap required during erection/dismantling |
| Full-Body Fall Arrest Harness | ANSI/ASSP Z359.1; Z359.11 | Required for erectors until guardrails installed |
| Self-Retracting Lifeline (SRL) | ANSI/ASSP Z359.14 | Preferred over shock-absorbing lanyard for scaffold work |
| Safety Glasses | ANSI/ISEA Z87.1+ | Impact-rated with side shields |
| Work Gloves (Cut-Resistant) | ANSI/ISEA 105, Cut Level A4+ | Dexterity sufficient for pin/coupler handling |
| Steel-Toe Safety Boots | ASTM F2413 (I/75 C/75) | Ankle support; slip-resistant soles |
| High-Visibility Vest | ANSI/ISEA 107, Class 2 minimum | Required at all times on scaffold operations |
| Hearing Protection | ANSI S3.19 / S12.6 (NRR ≥ 25 dB) | Required when hammering or using power tools |

EMERGENCY PROCEDURES

Fall from Scaffold:

• Call 911 immediately — do not move the victim unless in immediate danger

• Stabilize head and neck — assume spinal injury until medically cleared

• Control bleeding with direct pressure; apply tourniquet if trained and needed

• Cover victim to prevent shock — keep warm and still

• Clear area below scaffold and restrict access

• Preserve scene for investigation — do not alter scaffold

Scaffold Collapse / Structural Failure:

• Sound alarm — evacuate all personnel from scaffold and surrounding area

• Call 911 and report potential trapped/injured workers

• Establish exclusion zone — no re-entry until structural engineer clears area

• Account for all personnel by name at assembly point

• Do not attempt rescue from collapsed structure without professional rescue team

• Preserve all evidence for OSHA and incident investigation

Struck by Falling Object:

• Clear area and provide first aid to injured worker

• Call 911 for head injuries, fractures, or significant bleeding

• Apply ice/pressure to impact site; immobilize fractures

• Monitor for signs of concussion (dizziness, nausea, confusion)

• All head impacts require medical evaluation — do not allow worker to refuse

Emergency Contact Information:

• Emergency Services: 911

• Site Safety Manager: [Insert Name & Phone]

• Competent Person: [Insert Name & Phone]

• Nearest Hospital: [Insert Name, Address & Phone]

• OSHA Area Office: [Insert Local Number]

WORKER SIGN-OFF — I HAVE READ AND UNDERSTAND THIS JHA

| # | Print Name | Signature | Company / Trade | Date |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |
| 9 |  |  |  |  |
| 10 |  |  |  |  |

REVIEW & APPROVAL

| Role | Name / Signature | Date | Initials |
| --- | --- | --- | --- |
| Prepared By (Author) |  |  |  |
| Reviewed By (Safety Manager) |  |  |  |
| Approved By (Project Manager) |  |  |  |
| Client / Owner Rep |  |  |  |

This JHA must be reviewed and updated whenever job conditions change, after any incident, or at minimum annually.