| FORBES SAFETY SOLUTIONS | JOB HAZARD ANALYSIS |
| --- | --- |

STEEL ERECTION — JOB HAZARD ANALYSIS

Reference: 29 CFR 1926 Subpart R — Steel Erection

PROJECT INFORMATION

| Company Name: | [Enter Company Name] |
| --- | --- |
| Project Name: | [Enter Project Name] |
| Date: | [MM/DD/YYYY] |
| Supervisor: | [Enter Supervisor Name] |
| JHA Number: | [JHA-SE-001] |
| Work Activity: | Steel Erection — 29 CFR 1926 Subpart R |

JOB HAZARD ANALYSIS — TASK STEPS & CONTROLS

The following analysis identifies task steps, associated hazards, risk levels, and control measures for steel erection operations. All workers must review this JHA before commencing work.

| Task Step | Hazard(s) | Risk Level (H/M/L) | Control Measures |
| --- | --- | --- | --- |
| 1. Site Preparation & Layout | Uneven terrain, underground utilities, overhead power lines, heavy equipment traffic | H | • Conduct site survey; identify/mark all utilities • Verify minimum clearance from power lines (20 ft for lines up to 350kV per Table A) • Establish equipment travel paths and barricades • Ground conditions assessed by competent person |
| 2. Unloading & Staging Steel Members | Struck-by falling steel, crushing injuries during rigging, overloaded crane, pinch points | H | • Use tag lines to control loads; never stand under suspended loads • Inspect all rigging gear before each lift; remove defective slings • Establish exclusion zone around unloading area • Store steel on level ground with adequate dunnage and chocking |
| 3. Column Erection & Plumbing | Falls from height, column tip-over, struck-by tools/bolts, structural instability | H | • 100% tie-off required above 15 ft (steel erection standard) • Anchor bolts inspected and verified per erection drawings • Columns guyed or braced immediately upon placement • Connectors use personal fall arrest systems with retractable lanyards |
| 4. Beam Setting & Connection | Falls from height, struck-by steel beams, hand/finger crush injuries, unstable structural members | H | • Double connections or equivalent per 1926.756(c)(1) • Connectors positioned with PFAS to structural steel • Beam secured with minimum 2 bolts before load released • Communication via radio between crane operator, signalman, and connectors |
| 5. Decking Installation | Falls through openings, falls from leading edge, slipping on unsecured decking, falling objects | H | • Controlled Decking Zone (CDZ) established per 1926.760 • CDZ limited to 90 ft / 3000 sq ft ahead of leading edge • Decking bundles secured immediately upon placement • Safety net systems or PFAS for workers outside CDZ |
| 6. Bolt-Up & Final Connections | Falls from height, dropped tools/bolts striking workers below, hand injuries, ergonomic strain | M | • Tool lanyards required for all hand tools at height • Tethered bolt bags; no loose hardware in pockets • 100% tie-off maintained during bolt-up operations • Torque values verified per structural engineer specifications |
| 7. Welding & Cutting Operations | Welding burns, arc flash eye injury, fire/explosion, toxic fume inhalation, hot work ignition | H | • Hot work permit obtained before each welding session • Fire watch maintained for 30 minutes after welding ceases • Welding screens/curtains deployed to protect adjacent workers • Forced ventilation or respiratory protection for confined areas • Fire extinguisher within 20 ft of welding operations |

REQUIRED PERSONAL PROTECTIVE EQUIPMENT (PPE)

☐  Hard hat (ANSI Z89.1 Type I, Class E) — required at all times on site

☐  Safety glasses with side shields (ANSI Z87.1) — required at all times

☐  Welding hood/goggles (Shade 10-14) — required during welding/cutting operations

☐  High-visibility vest (ANSI/ISEA 107 Class 2 minimum) — required at all times

☐  Steel-toed boots (ASTM F2413) with puncture-resistant soles

☐  Cut-resistant gloves (ANSI A4 minimum) for steel handling; welding gloves for hot work

☐  Personal Fall Arrest System (PFAS): full-body harness, shock-absorbing lanyard, and SRL

☐  Hearing protection (NRR 25+) when noise exceeds 85 dBA

☐  Welding leathers/flame-resistant clothing during welding and cutting operations

EMERGENCY PROCEDURES

| Emergency Contact Number: | [Enter Phone Number] |
| --- | --- |
| Nearest Hospital: | [Enter Hospital Name & Address] |
| Site Assembly Point: | [Enter Location] |
| First Aid Kit Location: | [Enter Location] |
| AED Location: | [Enter Location] |
| Fire Extinguisher Locations: | [Enter Locations] |
| Rescue Plan for Fall Arrest: | [Describe Suspension Trauma Rescue Procedure] |

CREW SIGN-OFF — I HAVE REVIEWED AND UNDERSTAND THIS JHA

By signing below, each crew member acknowledges they have reviewed this Job Hazard Analysis, understand the identified hazards and control measures, and agree to follow all safety procedures outlined herein.

| Printed Name | Signature | Date |
| --- | --- | --- |

ADDITIONAL HAZARDS / SITE-SPECIFIC NOTES

Use this section to document any additional hazards, site-specific conditions, or supplemental control measures not covered above.

JHA REVIEW & APPROVAL

| Role | Name / Signature | Date |
| --- | --- | --- |
| Prepared By (Competent Person): |  |  |
| Reviewed By (Site Superintendent): |  |  |
| Approved By (Project Manager): |  |  |

DISCLAIMER: This JHA template is provided as a starting point for site-specific hazard analysis. It must be reviewed and customized by a competent person familiar with the specific project conditions. Forbes Safety Solutions is not liable for misuse or failure to adapt this template to actual site conditions. Always consult current OSHA regulations and site-specific safety plans.