FORBES SAFETY SOLUTIONS

Fall Rescue Plan

Prompt Rescue Procedures for Suspended Workers

Prepared for:

[COMPANY NAME]

Reference Standards:

29 CFR 1926.502(d)(20) | ANSI Z359.2 | ANSI Z359.4

Date: [DATE]  |  Revision: [REV NUMBER]

# Table of Contents

# 1. Purpose

This Fall Rescue Plan establishes the procedures, equipment, and training requirements for the prompt rescue of employees who have fallen and are suspended in a personal fall arrest system. This plan ensures compliance with 29 CFR 1926.502(d)(20), which requires that the employer provide for prompt rescue of employees in the event of a fall, or assure that employees are able to rescue themselves.

Suspension in a fall arrest system is a life-threatening emergency. Without prompt rescue, a suspended worker can develop suspension trauma (orthostatic intolerance) which can cause serious injury or death. This plan ensures [COMPANY NAME] is prepared to rescue fallen workers within the critical time window.

# 2. Scope

This Fall Rescue Plan applies to all [COMPANY NAME] work activities where employees use personal fall arrest systems (PFAS), including but not limited to:

- Steel erection and structural work

- Roofing and rooftop work

- Scaffold work at heights

- Crane operations and maintenance

- Tower climbing (communication, utility)

- Elevated platform work

- Bridge and overpass work

- Excavation and confined space egress

- Any work at heights requiring fall arrest equipment

# 3. Responsibilities

## 3.1 Management

- Ensure a site-specific rescue plan is developed before any work at heights begins

- Provide rescue equipment and maintain it in ready condition

- Fund rescue training and drill programs

## 3.2 Safety Manager / Competent Person

- Develop site-specific rescue plans for each project

- Ensure rescue equipment is available, inspected, and operational

- Coordinate rescue team training and drills

- Review and update rescue plans when site conditions change

- Conduct post-rescue debriefs and documentation

## 3.3 Rescue Team Members

- Maintain current rescue training and certifications

- Participate in rescue drills at least quarterly

- Inspect rescue equipment before each shift

- Be available and capable of responding within the target rescue time

## 3.4 Supervisors

- Brief workers on the rescue plan before work at heights begins

- Ensure rescue equipment is on-site and accessible

- Initiate rescue procedures immediately upon a fall event

- Account for all workers and maintain communication

## 3.5 Employees

- Know the site-specific rescue plan and their role in it

- Carry suspension relief straps when using a fall arrest harness

- Immediately alert others if a fall occurs

- Attempt self-rescue if trained and able

# 4. Suspension Trauma / Orthostatic Intolerance

Suspension trauma (also called orthostatic intolerance or harness-induced pathology) is a life-threatening condition that occurs when a person is held motionless in an upright position in a harness.

## 4.1 How Suspension Trauma Occurs

- When suspended motionless in a harness, the leg straps compress veins and restrict blood flow

- Blood pools in the legs due to gravity and cannot return to the heart

- Reduced blood return decreases cardiac output and blood pressure

- Vital organs (brain, kidneys, heart) become deprived of oxygenated blood

- The condition can begin within minutes of suspension

## 4.2 Time Critical Nature

CRITICAL: Suspension trauma can become fatal within 30 minutes of motionless suspension. Some studies indicate serious effects can begin in as little as 3 to 5 minutes. Rescue must be treated as immediately dangerous to life and health (IDLH).

| Time Suspended | Potential Effects | Urgency Level |
| --- | --- | --- |
| 0–5 minutes | Discomfort, numbness in legs, tingling | High — initiate rescue immediately |
| 5–15 minutes | Pain, dizziness, nausea, elevated heart rate, anxiety | Critical — rescue must be underway |
| 15–30 minutes | Loss of consciousness, organ damage, cardiac arrhythmia | Life-threatening — minutes matter |
| 30+ minutes | Cardiac arrest, renal failure, death | Fatal if not rescued |

## 4.3 Contributing Factors

- Immobility — conscious workers should be instructed to pump their legs to promote circulation

- Injury from the fall — pain or injury may prevent movement

- Unconsciousness — eliminates all muscle activity

- Cold temperatures — accelerate the onset of symptoms

- Dehydration and fatigue — reduce tolerance

- Tight harness straps — increase venous pooling

## 4.4 Suspension Relief Straps

All employees using personal fall arrest systems shall carry suspension relief straps (trauma straps) attached to their harness. These straps allow a conscious suspended worker to stand in the straps, relieving pressure on the legs and promoting blood circulation while awaiting rescue.

# 5. Rescue Methods

[COMPANY NAME] shall evaluate and implement one or more of the following rescue methods for each work situation:

## 5.1 Self-Rescue

Self-rescue is the preferred first response when the worker is conscious, uninjured, and trained:

- Worker deploys suspension relief straps and stands to relieve leg pressure

- Worker uses a self-rescue device (retractable descent device) to lower themselves to a safe level

- Worker climbs back to the work surface if possible

Self-rescue training is provided as part of the fall protection training program. Not all situations allow self-rescue — workers must also know when to wait for assisted rescue.

## 5.2 Assisted Rescue (Non-Entry)

Assisted rescue is performed from an adjacent safe location without the rescuer entering the fall zone:

- Rescue pole systems — a pole with a hook mechanism to reach the fallen worker and connect a lowering line

- Retrieval systems — pre-rigged mechanical retrieval winches that can raise or lower a worker

- Reach and pull — from an adjacent elevated surface, rescuers reach the worker with a hook or strap

- Ladder access — position an extension ladder to the suspended worker's location

## 5.3 Mechanical Rescue

Mechanical rescue uses equipment to reach and lower the suspended worker:

- Aerial lifts (boom lifts, scissor lifts) positioned beneath the worker

- Crane with personnel basket (last resort, follow OSHA crane-suspended personnel platform requirements)

- Self-retracting lifeline (SRL) with rescue/retrieval capability — some SRL models include built-in rescue winches

- Davit arm or tripod with rescue winch

## 5.4 Rescue Method Selection

The rescue method shall be selected based on the specific site conditions. Factors include:

- Height of the fall arrest point

- Location accessibility (can a lift or ladder reach the worker?)

- Number of workers at height simultaneously

- Availability and proximity of rescue equipment

- Environmental conditions (wind, weather, confined areas)

- Worker condition (conscious, injured, unconscious)

# 6. Rescue Equipment Inventory

[COMPANY NAME] shall maintain the following rescue equipment, as applicable to site conditions:

| Equipment | Quantity Required | Location | Inspection Frequency |
| --- | --- | --- | --- |
| Rescue/retrieval SRL with winch | Per site-specific plan | [SPECIFY] | Before each use, annually by manufacturer |
| Suspension relief straps | One per worker in PFAS | Attached to each harness | Before each use |
| Rescue pole/hook system | 1 per work area | [SPECIFY] | Monthly and before each use |
| Portable rescue kit (rope, pulleys, carabiners) | 1 per work area | [SPECIFY] | Monthly and before each use |
| Aerial lift (for rescue access) | Per site-specific plan | [SPECIFY] | Per manufacturer schedule |
| Extension ladder(s) | Per site-specific plan | [SPECIFY] | Before each use |
| First aid kit (with trauma supplies) | 1 per work area | [SPECIFY] | Monthly |
| Communication devices (radios) | 1 per worker and rescue team | [SPECIFY] | Daily function check |

# 7. Rescue Team Training and Drills

## 7.1 Training Requirements

Designated rescue team members shall receive training in:

- Suspension trauma — recognition, time urgency, contributing factors

- Rescue equipment setup, operation, and inspection

- Rescue techniques (self-rescue, assisted, mechanical)

- Communication procedures during rescue

- First aid / CPR (current certifications required)

- Post-rescue medical considerations

- Site-specific rescue plan review

## 7.2 Drill Requirements

- Rescue drills shall be conducted at least quarterly

- Drills shall simulate realistic rescue scenarios at actual or similar work locations

- Each drill shall be timed — target rescue time shall be established for each scenario

- Drill results shall be documented using the Rescue Drill Documentation Form (Appendix B)

- Deficiencies identified during drills shall be corrected and the drill repeated

- All rescue team members shall participate in at least two drills per year

# 8. Site-Specific Rescue Planning

Before any work at heights begins, a site-specific rescue plan shall be developed addressing the following:

## 8.1 Key Planning Questions

- How will the rescuer reach the suspended worker? (Lift, ladder, rope access, adjacent structure)

- What rescue equipment is needed? (Retrieval SRL, rescue pole, aerial lift, lowering system)

- Where is the rescue equipment located? (On-site, staged at work area, in rescue cache)

- What is the estimated rescue time? (Must be achievable in under 15 minutes)

- Who are the designated rescue team members for this shift?

- How will the rescue be communicated? (Radio, verbal, phone)

- Where will emergency medical services stage? (Access route, staging area)

- Are there any environmental hazards that affect rescue? (Electrical, chemical, confined space)

## 8.2 Estimated Rescue Time

The site-specific rescue plan must demonstrate that rescue can be accomplished within a time frame that prevents suspension trauma. The target rescue time shall account for:

- Notification time — time from fall event to rescue team alert

- Response time — time for rescue team to reach the rescue equipment

- Setup time — time to position equipment and access the worker

- Rescue time — time to reach the worker and lower them to a safe surface

Total target rescue time: Not to exceed 15 minutes from fall event to worker on safe surface.

# 9. Communication During Rescue

Clear communication is essential during a rescue operation:

- The person discovering the fall event shall immediately alert the designated rescue team and supervisor

- Call 911 or local emergency services immediately — do not wait to see if rescue is successful

- One person shall serve as the rescue coordinator to direct the operation

- Maintain voice or radio contact with the suspended worker (if conscious) to assess condition and provide instructions

- Keep non-essential personnel clear of the rescue area

- Designate a person to guide emergency medical services to the scene

# 10. Post-Rescue Medical Treatment

Post-rescue medical treatment is critical and must follow specific protocols:

## 10.1 DO NOT Lay the Victim Flat Immediately

CRITICAL WARNING: Do NOT lay a rescued suspension trauma victim flat on their back immediately after rescue.

When a person has been suspended for any length of time, blood has pooled in the lower extremities. Laying them flat causes a sudden rush of pooled, deoxygenated, toxin-laden blood back to the heart, which can cause cardiac arrest ("rescue death" or "reflow syndrome").

- Place the rescued worker in a seated or semi-reclined position (W-position or legs bent at the knees)

- Gradually transition to a flat position over 30–40 minutes as medical personnel direct

- Monitor for signs of shock, cardiac distress, and renal failure

- Provide supplemental oxygen if available and trained

## 10.2 Medical Monitoring

- All rescued workers shall receive medical evaluation regardless of apparent condition

- Monitor vital signs continuously until EMS arrives

- Watch for delayed onset symptoms — renal failure can occur hours after rescue

- Inform EMS of the duration of suspension and any symptoms observed

- Do not allow the worker to return to work without medical clearance

# 11. Rescue from Specific Situations

## 11.1 Cranes

- Pre-plan rescue before climbing — identify how to reach the worker on the crane structure

- Consider SRL with rescue/retrieval capability at the crane cab and boom

- Aerial lifts may be positioned to reach workers on crane structures

- Ensure crane operator can be contacted to stop crane operations during rescue

## 11.2 Towers (Communication, Utility)

- Tower rescue requires specialized rope rescue training and equipment

- Pre-rig rescue systems at work locations before climbing begins

- Use tower-specific rescue kits (descender devices, pick-off rescue systems)

- Establish ground-based rescue capability (rigging for lowering systems)

- Consider weather conditions — wind and lightning increase rescue complexity

## 11.3 Scaffolds

- Ensure scaffold access points allow rescue equipment (ladders, lifts) to reach all levels

- Pre-position rescue equipment at scaffold access points

- Plan for rescue from inside the scaffold (through access hatches) or outside (via aerial lift)

- Ensure scaffold design does not create obstructions to rescue

## 11.4 Excavations

- Workers in excavations may become suspended in harness systems at edge transitions

- Pre-position retrieval systems at excavation entry points

- Ensure rescue plan addresses both fall rescue and excavation cave-in protocols

- Coordinate with confined space rescue procedures if the excavation is classified as a permit-required confined space

# 12. Program Review

- Review this plan at least annually

- Update the site-specific plan whenever conditions change (new work areas, equipment changes, personnel changes)

- Review after every actual rescue event

- Review after every drill that identifies deficiencies

- Incorporate lessons learned from industry incidents

# 13. Recordkeeping

- Site-specific rescue plans for each project

- Rescue team training records and certifications

- Rescue drill documentation (date, participants, scenario, time, results)

- Rescue equipment inspection records

- Actual rescue event reports

- Post-rescue medical evaluation records

# Appendix A: Site-Specific Rescue Plan Form

SITE-SPECIFIC FALL RESCUE PLAN

Project Name / Job Site: _______________________________

Date Prepared: _______________________________

Prepared By: _______________________________

Work Description: _______________________________

Height of Work (feet): _______________________________

Fall Arrest System Used: _______________________________

Rescue Method(s) — Check all that apply:

- ☐  Self-Rescue (retractable descent device, climb back)

- ☐  Assisted Rescue — Rescue pole/hook system

- ☐  Assisted Rescue — Retrieval winch (pre-rigged)

- ☐  Mechanical Rescue — Aerial lift (boom/scissor)

- ☐  Mechanical Rescue — SRL with rescue capability

- ☐  Mechanical Rescue — Davit/tripod with rescue winch

- ☐  Other: _________________________________________________

How will the rescuer reach the suspended worker?

__________________________________________________________________________

__________________________________________________________________________

What rescue equipment is needed and where is it located?

__________________________________________________________________________

__________________________________________________________________________

Estimated Rescue Time:

Notification time: _______________________________

Response time: _______________________________

Setup time: _______________________________

Rescue/lowering time: _______________________________

TOTAL estimated rescue time: _______________________________

Designated Rescue Team Members (this shift):

| Name | Role | Training Current? | Contact Method |
| --- | --- | --- | --- |

Emergency Services Phone Number: _______________________________

EMS Access Route / Staging Area: _______________________________

Environmental Hazards Affecting Rescue:

__________________________________________________________________________

Prepared By (Signature): ________________________________________

Date: ________________________________________

Reviewed By (Supervisor): ________________________________________

Date: ________________________________________

# Appendix B: Rescue Drill Documentation Form

FALL RESCUE DRILL RECORD

Date of Drill: _______________________________

Job Site / Location: _______________________________

Drill Coordinator: _______________________________

Scenario Description: _______________________________

Drill Participants:

| Name | Role in Drill | Training Date | Certification Current? |
| --- | --- | --- | --- |

Drill Timeline:

Fall event simulated (start time): _______________________________

Rescue team alerted (time): _______________________________

Rescue team arrived at rescue equipment (time): _______________________________

Equipment deployed / rescue initiated (time): _______________________________

Worker reached by rescuer (time): _______________________________

Worker lowered to safe surface (time): _______________________________

TOTAL RESCUE TIME: _______________________________

Was the target rescue time (<15 minutes) achieved?   ☐ Yes   ☐ No

Rescue Equipment Used:

__________________________________________________________________________

Problems Encountered:

__________________________________________________________________________

__________________________________________________________________________

Corrective Actions Required:

__________________________________________________________________________

__________________________________________________________________________

Follow-Up Drill Required?   ☐ Yes   ☐ No     If yes, date: _______________

Drill Coordinator Signature: ________________________________________

Date: ________________________________________

Safety Manager Review: ________________________________________

Date: ________________________________________