FORBES SAFETY SOLUTIONS

Fire Prevention Plan

OSHA Compliance Program

Prepared for:

[COMPANY NAME]

Regulatory References:

29 CFR 1910.39 | 29 CFR 1926.150-155 | NFPA 10 | NFPA 51B

Date: [DATE]     |     Revision: [REV NUMBER]

Prepared by: [SAFETY MANAGER NAME / TITLE]

# Table of Contents

# 1. Purpose and Scope

This Fire Prevention Plan (FPP) has been developed to comply with OSHA requirements under 29 CFR 1910.39 (General Industry) and 29 CFR 1926.150-155 (Construction). The purpose of this plan is to prevent fires from occurring at [COMPANY NAME] work sites and facilities, and to provide procedures for fire safety in the event of a fire emergency.

This plan applies to all employees, subcontractors, and visitors at [COMPANY NAME] facilities, offices, work sites, and storage areas. It is intended to be used in conjunction with the company's Emergency Action Plan.

## 1.1 Regulatory References

- 29 CFR 1910.39 – Fire Prevention Plans (General Industry)

- 29 CFR 1910.157 – Portable Fire Extinguishers

- 29 CFR 1926.150 – Fire Protection (Construction)

- 29 CFR 1926.151 – Fire Prevention

- 29 CFR 1926.152 – Flammable Liquids

- 29 CFR 1926.153 – Liquefied Petroleum Gas (LPG)

- 29 CFR 1926.154 – Temporary Heating Devices

- 29 CFR 1926.155 – Definitions Applicable to Fire Protection

- NFPA 10 – Standard for Portable Fire Extinguishers

- NFPA 51B – Standard for Fire Prevention During Welding, Cutting, and Other Hot Work

# 2. Fire Hazard Identification

[COMPANY NAME] shall conduct a fire hazard assessment for each work site, facility, and storage area. The assessment shall identify potential fire hazards and the controls necessary to mitigate them.

## 2.1 Common Fire Hazards

| Hazard Category | Examples | Control Measures |
| --- | --- | --- |
| Flammable/Combustible Liquids | Gasoline, diesel, solvents, paints, adhesives | Approved storage cabinets, proper labeling, grounding/bonding, limited quantities |
| Compressed Gases | Acetylene, propane, oxygen | Upright storage, separation of fuel/oxidizer, valve protection caps, ventilated storage |
| Electrical Hazards | Overloaded circuits, damaged cords, temporary wiring | GFCI protection, regular inspections, proper amperage, no daisy-chaining |
| Hot Work Activities | Welding, cutting, brazing, grinding | Hot work permits, fire watch, 35-ft clearance, fire extinguisher at point of operation |
| Combustible Materials | Wood, paper, cardboard, rags, insulation | Proper housekeeping, designated storage, separation from ignition sources |
| Temporary Heating | Salamanders, propane heaters, portable heaters | Clearance from combustibles, ventilation, fuel storage separation, tip-over protection |
| Smoking | Cigarettes, lighters, matches | Designated smoking areas only, minimum 25 ft from flammables/combustibles |

## 2.2 Fire Hazard Assessment Frequency

- Initial assessment before work begins at each new site or facility

- Updated whenever conditions change (new materials, new tasks, weather changes)

- Annual review of all standing facilities

- Documented using the Fire Hazard Assessment Form (Appendix A)

# 3. Roles and Responsibilities

## 3.1 Program Administrator

[SAFETY MANAGER NAME / TITLE] is responsible for:

- Developing, implementing, and maintaining this Fire Prevention Plan

- Conducting or overseeing fire hazard assessments

- Ensuring fire extinguishers are properly placed, inspected, and maintained

- Coordinating employee fire safety training

- Investigating fire incidents and near-misses

- Issuing and managing hot work permits

## 3.2 Supervisors

- Enforce fire prevention procedures at the work site

- Ensure housekeeping standards are maintained to prevent fire hazards

- Verify fire extinguishers are accessible and current inspections are documented

- Ensure employees are trained on emergency procedures and extinguisher use

- Assign fire watch personnel during and after hot work operations

## 3.3 Employees

- Follow all fire prevention procedures and housekeeping requirements

- Report fire hazards and unsafe conditions immediately

- Know the location of fire extinguishers and how to use them (if trained)

- Know the emergency evacuation routes and assembly areas

- Perform fire watch duties when assigned

# 4. Housekeeping for Fire Prevention

Proper housekeeping is one of the most effective methods of fire prevention. [COMPANY NAME] requires the following housekeeping practices:

- Maintain work areas free of unnecessary accumulations of combustible materials (scrap wood, paper, oily rags, cardboard, packaging).

- Dispose of oily or solvent-soaked rags in approved, self-closing metal containers. Empty containers daily or at the end of each shift.

- Keep exits, stairways, aisles, and access to fire extinguishers clear and unobstructed at all times.

- Store combustible materials at least 10 feet away from any building under construction (per 29 CFR 1926.151).

- Remove combustible waste from enclosed areas at regular intervals during construction operations.

- Clean up spills of flammable or combustible liquids immediately.

- Ensure trash and debris dumpsters are located a safe distance from structures.

# 5. Hot Work Controls

Hot work (welding, cutting, brazing, soldering, grinding, and other spark-producing operations) is a leading cause of workplace fires. [COMPANY NAME] implements the following controls:

## 5.1 Hot Work Permit System

A Hot Work Permit is required before any hot work is performed outside of designated welding/cutting areas. The permit system ensures:

- The area has been inspected and combustibles removed or protected within a 35-foot radius

- A fire extinguisher (minimum 20-lb ABC) is immediately available at the point of operation

- A trained fire watch person is assigned

- Fire detection/suppression systems in the area are operational

- Flammable vapors, dusts, or gases have been tested for and eliminated

## 5.2 Fire Watch Requirements

A fire watch is required during all hot work operations and for a minimum of 30 minutes (60 minutes recommended) after hot work is completed. Fire watch personnel shall:

- Have fire-extinguishing equipment readily available and be trained in its use

- Be familiar with the facility/site and the procedures for sounding a fire alarm

- Watch for fires in all exposed areas and attempt to extinguish only fires clearly within the capability of the available equipment

- Have no other duties that would conflict with fire watch responsibilities

- Maintain a Fire Watch Log (Appendix B) for each hot work operation

NOTE: Where hot work is performed on walls, ceilings, or floors, a fire watch shall be maintained on the opposite side if combustible materials are present.

# 6. Flammable and Combustible Liquid Storage

## 6.1 General Requirements

- Store flammable and combustible liquids in approved, properly labeled containers only.

- Do not store more than 25 gallons of flammable liquids outside of an approved storage cabinet or storage room.

- Approved flammable storage cabinets shall hold no more than 60 gallons of Class I or II liquids, or 120 gallons of Class III liquids.

- Storage areas shall be well-ventilated and located away from ignition sources, exits, and stairways.

- Grounding and bonding shall be used when transferring flammable liquids between containers to prevent static discharge.

## 6.2 Quantity Limits in Work Areas

| Container Type | Maximum Capacity | Maximum Quantity in Work Area |
| --- | --- | --- |
| Safety cans (approved) | 5 gallons | 25 gallons total of flammable liquids |
| Original manufacturer containers | As received | One day's supply or 25 gallons (whichever is less) |
| Flammable storage cabinet | 60 gallons Class I/II | Per approved cabinet rating |

# 7. Fire Suppression Equipment

## 7.1 Portable Fire Extinguishers

[COMPANY NAME] shall provide and maintain portable fire extinguishers in accordance with 29 CFR 1910.157, 29 CFR 1926.150, and NFPA 10.

### 7.1.1 Extinguisher Placement – Travel Distance Requirements

| Hazard Class | Extinguisher Type | Maximum Travel Distance |
| --- | --- | --- |
| Class A (ordinary combustibles) | 2-A rated minimum | 75 feet (23 m) |
| Class B (flammable liquids) | 10-B rated minimum | 50 feet (15 m) |
| Class C (electrical equipment) | Rated for electrical use | Based on Class A or B hazard present |
| Class D (combustible metals) | Specific to metal type | 75 feet (23 m) |
| Class K (cooking oils/fats) | Wet chemical type | 30 feet (9 m) |
| Multi-purpose (ABC) | Per rating | 50-75 feet depending on hazard |

### 7.1.2 Construction Site Requirements

Per 29 CFR 1926.150, the following fire protection requirements apply to construction operations:

- A fire extinguisher rated not less than 2A shall be provided for each 3,000 square feet of building area.

- A fire extinguisher rated not less than 10B shall be provided within 50 feet of wherever more than 5 gallons of flammable liquids or 5 pounds of flammable gas are being used.

- Fire extinguishers shall be conspicuously located and readily accessible at all times.

- Fire extinguishers shall be inspected monthly and maintained annually per NFPA 10.

## 7.2 Inspection and Maintenance

| Inspection Type | Frequency | Performed By | Requirements |
| --- | --- | --- | --- |
| Visual Inspection | Monthly | Designated employee | Verify location, access, pressure gauge, physical condition, pin/tamper seal |
| Annual Maintenance | Annually | Certified fire extinguisher technician | Full inspection per NFPA 10, tag and record |
| Hydrostatic Testing | Per NFPA 10 schedule (5-12 years) | Certified technician | Pressure test per extinguisher type |
| 6-Year Internal Exam | Every 6 years (stored pressure) | Certified technician | Internal examination of stored-pressure extinguishers |

# 8. Electrical Fire Prevention

Electrical hazards are a significant source of workplace fires. [COMPANY NAME] implements the following controls:

- All temporary wiring shall comply with OSHA requirements and the National Electrical Code (NEC).

- Extension cords shall be used only as temporary wiring and shall be of adequate gauge for the load.

- Do not daisy-chain extension cords, power strips, or surge protectors.

- Damaged or frayed electrical cords, plugs, and equipment shall be removed from service immediately.

- Ground Fault Circuit Interrupters (GFCIs) shall be used on all temporary power circuits.

- Electrical panels and disconnect switches shall have minimum 36-inch clearance at all times.

- All electrical work shall be performed by qualified personnel only.

- Space heaters, if permitted, shall be UL-listed, have tip-over protection, and maintain 3-foot clearance from combustibles.

# 9. Smoking Policy

[COMPANY NAME] restricts smoking (including e-cigarettes and vaping) to designated smoking areas only.

- Designated smoking areas shall be clearly marked and located a minimum of 25 feet from buildings, flammable/combustible storage, trash containers, and vegetation.

- Appropriate receptacles for disposal of smoking materials shall be provided at all designated areas.

- Smoking is strictly prohibited near flammable liquid storage areas, fueling operations, compressed gas cylinders, and areas posted with "No Smoking" signs.

- Smoking is prohibited inside all company buildings, vehicles, and enclosed structures.

# 10. Emergency Procedures

## 10.1 In the Event of a Fire

- Activate the fire alarm or alert all personnel in the area by voice if no alarm system is available.

- Call 911 (or the local emergency number) immediately.

- If the fire is small and you are trained, attempt to extinguish it using the nearest appropriate fire extinguisher. Use the P.A.S.S. technique: Pull the pin, Aim at the base of the fire, Squeeze the handle, Sweep side to side.

- If the fire cannot be controlled quickly, evacuate the area immediately using the nearest safe exit.

- Close doors behind you as you evacuate to slow the spread of fire and smoke.

- Proceed to the designated assembly area and report to the supervisor or site safety representative.

- Do not re-enter the building or area until the fire department has declared it safe.

## 10.2 Assembly Areas

[COMPANY NAME] shall designate assembly areas at each work site and facility. Assembly areas shall be:

- Located a safe distance from the building/work area (minimum 100 feet recommended)

- Upwind of the fire location when possible

- Clear of emergency vehicle access routes

- Communicated to all employees during orientation and toolbox talks

## 10.3 Accountability

Supervisors shall account for all employees at the assembly area. A headcount shall be conducted and reported to the site safety representative and/or the fire department incident commander.

# 11. Training

[COMPANY NAME] shall provide fire prevention and fire safety training to all employees as follows:

## 11.1 Initial Training

All new employees shall receive fire prevention training during orientation, covering:

- This Fire Prevention Plan and how to access it

- Fire hazards associated with their specific job duties

- Proper handling and storage of flammable/combustible materials

- Location and proper use of fire extinguishers

- Emergency procedures and evacuation routes

- Reporting fire hazards and incidents

## 11.2 Annual Refresher Training

All employees shall receive annual refresher training on fire prevention topics. Additional training shall be provided when:

- An employee's job duties change and introduce new fire hazards

- New materials, equipment, or processes are introduced

- This plan is revised or updated

## 11.3 Specialized Training

- Hot Work Permit Holders: Training on hot work procedures, fire watch duties, and permit requirements

- Fire Watch Personnel: Training on fire watch responsibilities, extinguisher use, and alarm activation

- Fire Extinguisher Use: Hands-on training for employees expected to use portable fire extinguishers (per 29 CFR 1910.157(g))

# 12. Recordkeeping

[COMPANY NAME] shall maintain the following fire prevention records:

- Fire Hazard Assessment Forms (Appendix A)

- Hot Work Permits

- Fire Watch Logs (Appendix B)

- Fire extinguisher monthly inspection tags and annual maintenance records

- Employee fire prevention training records (date, topics, attendees, trainer)

- Fire incident investigation reports

Records shall be maintained for a minimum of three (3) years, or longer if required by applicable regulations.

# Appendix A: Fire Hazard Assessment Form

[COMPANY NAME]

Fire Hazard Assessment Form

Site / Facility Name: __________________________________________________

Address / Location: __________________________________________________

Assessment Date: __________________________________________________

Assessed By: __________________________________________________

Job / Project Number: __________________________________________________

## Fire Hazard Identification

Identify all fire hazards present at this location:

☐  Flammable liquids (gasoline, diesel, solvents, paints)

☐  Combustible materials (wood, paper, cardboard, insulation, vegetation)

☐  Compressed gases (acetylene, propane, oxygen)

☐  Electrical hazards (temporary wiring, overloaded circuits, damaged cords)

☐  Hot work operations (welding, cutting, brazing, grinding)

☐  Temporary heating devices

☐  Cooking equipment

☐  Smoking areas near combustibles

☐  Other: ________________________________________________

## Fire Prevention Controls in Place

☐  Fire extinguishers provided and accessible (type/location): _______________

☐  Flammable storage cabinet(s) in use

☐  Housekeeping procedures established

☐  Hot work permit system in effect

☐  No Smoking signs posted

☐  Designated smoking area established (location): ________________________

☐  Emergency evacuation routes identified

☐  Assembly area designated (location): ________________________________

☐  Employees trained on fire prevention and emergency procedures

☐  Fire alarm / notification system available

## Additional Controls Needed

________________________________________________________________________

________________________________________________________________________

________________________________________________________________________

## Fire Extinguisher Inventory

| Location | Type / Rating | Size | Last Inspection Date | Condition |
| --- | --- | --- | --- | --- |

________________________________________    ________________

Assessor Signature                                                            Date

________________________________________    ________________

Site Supervisor Signature                                                            Date

# Appendix B: Fire Watch Log

[COMPANY NAME]

Fire Watch Log

Date: __________________________________________________

Project / Site Name: __________________________________________________

Location of Hot Work: __________________________________________________

Hot Work Permit Number: __________________________________________________

Hot Work Operator Name: __________________________________________________

Fire Watch Person Name: __________________________________________________

## Pre-Hot Work Checklist

☐  Combustible materials moved or protected within 35-foot radius

☐  Floor / ground swept clean of combustible debris

☐  Flammable vapor / dust test conducted (if applicable) – Result: ___________

☐  Fire extinguisher (minimum 20-lb ABC) at point of operation

☐  Fire alarm / notification method established

☐  Wall / floor / ceiling openings covered or protected

☐  Opposite side of wall / ceiling checked for combustibles (if applicable)

☐  Sprinkler / fire suppression system operational (if applicable)

## Fire Watch Time Log

| Time | Activity / Observation | Initials |
| --- | --- | --- |
|  | Hot work started |  |
|  | Hot work completed |  |
|  | Post-hot work monitoring started |  |
|  | Fire watch completed (minimum 30 min after hot work) |  |

## Post-Hot Work Inspection

☐  Area inspected – No evidence of fire, hot spots, or smoldering materials

☐  Area re-inspected 30 minutes after hot work completion

☐  Area re-inspected 60 minutes after hot work completion (recommended)

Comments / Observations:

________________________________________________________________________

________________________________________________________________________

________________________________________    ________________

Fire Watch Person Signature                                                            Date

________________________________________    ________________

Hot Work Operator Signature                                                            Date

________________________________________    ________________

Supervisor Signature                                                            Date