[  COMPANY LOGO  ]

JOB HAZARD ANALYSIS

Concrete Placement & Finishing Operations

Applicable Standards: 29 CFR 1926 Subpart Q (1926.700–1926.706) — Concrete & Masonry; 29 CFR 1926.1153 — Respirable Crystalline Silica; ACI 347 — Formwork

| Project Name: ____________________________ | Date: ____________________________ |
| --- | --- |
| Location: ____________________________ | Supervisor: ____________________________ |
| Crew Members: _______________________________________________________________________________ |  |

| Step # | Job Steps | Potential Hazards | Hazard Controls | Responsible Person |
| --- | --- | --- | --- | --- |
| 1 | Inspect formwork, shoring, and bracing for structural adequacy | Formwork collapse under concrete weight; shoring failure; workers struck by falling forms | Competent person inspects all formwork per ACI 347 and 1926.703(a). Verify shoring designed by qualified engineer for anticipated loads. Check bracing connections, wedges, and tie spacing. No worker under forms being raised/moved. |  |
| 2 | Place and tie reinforcing steel (rebar) and embed items | Impalement on exposed rebar ends; cuts/lacerations from wire ties; manual handling strain; falls from elevated rebar mats | Install rebar caps (mushroom type) on all exposed vertical rebar per 1926.701(b). Cut-resistant gloves for tie wire work. Two-person carry for rebar >20 ft. Planking over rebar mats for worker access. Rebar impalement protection required at all heights. |  |
| 3 | Set up and position concrete pump truck — outriggers, boom reach, and exclusion zone | Pump truck tip-over; boom contact with overhead power lines; struck-by from boom swing; outrigger bearing failure | Level truck on firm ground. Fully extend outriggers with timber cribbing pads. Maintain min 20-ft clearance from power lines per 1926.1408. Establish swing radius exclusion zone. Certified pump operator only. Pre-pour safety briefing with all crew. |  |
| 4 | Receive and place concrete from ready-mix trucks | Struck-by from chute swing; contact with wet concrete (alkaline burns pH 12+); silica dust from dry areas; slip hazards | Spotter guides trucks — wheel chocks on grade. Long sleeves, waterproof gloves, rubber boots, and splash-proof goggles for all workers contacting concrete. Eye wash station on-site. Wash skin immediately upon contact. No kneeling in wet concrete. |  |
| 5 | Consolidate concrete using internal vibrators | Hand-arm vibration syndrome (HAVS); electrical shock from vibrator in wet conditions; noise exposure >85 dBA | GFCI protection on all electrical vibrators. Limit continuous vibrator use to 15-min intervals. Anti-vibration gloves. Hearing protection required (ear plugs + muffs near pump). Rotate vibrator operators to limit exposure. |  |
| 6 | Screed, bull float, and finish concrete surface | Ergonomic strain (bending/kneeling for extended periods); silica exposure during power trowel operations; slips on wet surface | Knee boards for finishing. Rotate finishers every 30 min. Wet methods during cutting/grinding per 1926.1153 Table 1. Non-slip rubber boots. Power trowel operators trained — kill switch functional. Dust mask or respirator for dry finishing. |  |
| 7 | Apply curing compound or wet cure methods | Chemical inhalation from curing compounds (VOC exposure); skin irritation; slip hazard on treated surfaces | Review SDS for curing compound. Apply with low-pressure sprayer to minimize aerosolization. Half-face respirator with OV cartridge in enclosed areas. Chemical-resistant gloves. Barricade cured areas to prevent foot traffic until set. |  |
| 8 | Strip forms and remove shoring per engineered sequence | Premature form removal (structural collapse); falling forms/shores; pinch points between forms and structure; nail punctures | Strip only after concrete reaches specified strength (cylinder break tests or maturity method). Follow engineer's reshoring sequence. Two workers minimum for panel removal. Hard hats, safety glasses, gloves mandatory. Stack forms on level ground. |  |
| 9 | Saw-cut control joints in cured slab | Respirable crystalline silica (RCS) exposure; blade contact injuries; noise exposure; electrical hazards | Wet cutting required per OSHA Table 1 — continuous water supply to blade. Hearing protection mandatory. Blade guard in place. GFCI on all electric saws. Operator trained in saw-cut timing (typically 4-12 hrs after placement). Barricade cut zone. |  |
| 10 | Site cleanup and equipment washout | Alkaline washout water contaminating stormwater; slips on hardened concrete; disposal violations | Concrete washout in designated lined pit — no discharge to storm drains per EPA NPDES/SWPPP requirements. Scrape and dispose of hardened concrete properly. Wash all tools promptly. PPE until all cleanup is complete. |  |

⚠ This JHA must be reviewed and signed by all crew members before work begins. Conditions on the jobsite may change — reassess hazards throughout the shift and update this JHA as necessary.

REVIEW & APPROVAL

| ________________________________________ Reviewed By (Print Name) | ________________________________________ Signature | ________________________________________ Date |
| --- | --- | --- |